3GPP TSG SA WG 1 Meeting #101 	S1-230456

Athens, GR, 20 – 24 February 2023	(revision of S1-230058)

Source:	Nokia, Novamint, , Sateliot, Gatehouse

pCR Title:	22.865 pCR: Updates in use case 5.2 to align service flows and potential new requirements

Draft Spec:	3GPP TR 22.865-020

Agenda item:	7.7 (FS_5GSAT_Ph3: Study on satellite access - Phase 3)

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.2 in FS_5GSAT_Ph3 TR.

1. Introduction

This contribution proposes to update current use cases on store and forward - MT.

2. Reason for Change

Current service flows and PRs are not aligned.

In addition, editorial changes are also needed.

How security would apply is not the responsibility of SA1 so the Editorial Note can be removed (S1-230137)

Introduced new requirements and terminologies such as S&F data retention period and S&F data storage quota.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865-020.

First Change (Description)

### 5.2.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Terminated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc offers a service of remote monitoring of fields and deploys and tracks many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed include a 5G communication with satellite access. Some of the UEs are deployed in a remote area where there is no mobile coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc uses the service of IoTSAT for the 5G IoT connectivity by satellite, and IoTSAT uses a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs regularly send information related to the area they are monitoring to the application server of TrackingInc and sometimes receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

Second Change (Service Flows)

### 5.2.3	Service Flows

The TrackingInc application server needs to send new parameters to the IoT remote monitoring UE through a dedicated message.

The TrackingInc application server message is sent by conventional means (e.g. IP routing, tunnels) to the network entry-point (e.g. a SCEF, PDN-GW, SMSC).

At this point:

- Limitations on the amount of data to be transferred to the IoT remote monitoring UE could be enforced.

- Delivery priority to the UE could be established.

- Acknowledgement of the received data by the UE could be issued.

- End-to-end acknowledgement policy can be established.

The network stores the message until it can be delivered/relayed to a satellite expected to fly over and provide coverage to the destination IoT remote monitoring UE.

When the satellite is connected via the feeder link to the ground network, the message is uploaded into the satellite.

When flying over the area that the IoT remote monitoring UE is located, the satellite with the stored message triggers paging over the service link for the UE to connect to the network.

The stored message is delivered/downloaded from the satellite to the IoT remote monitoring UE. Acknowledgment may be requested/issued. Mechanisms to ensure integrity of the delivered information may be in place.

Third Change (PRs)

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] The 5G system with satellite access shall be able to inform an application server whether store and forward operation is applied for communication with a UE.

[PR 5.1.6-002] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party on a per UE basis to specify the S&F data retention period after which undelivered data stored is being discarded.

[PR 5.1.6-003] The 5G system with satellite access supporting store and forward operation shall be able to enforce a S&F data retention period for the data to be stored.

[PR 5.2.6-004] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party on a per UE basis to set a S&F data storage quota.

[PR 5.2.6-005] The 5G system with satellite access supporting store and forward operation shall be able to enforce on a per UE basis a S&F data storage quota for the data to be stored on the satellite or the ground station.

[PR 5.2.6-006] The 5G system with satellite access supporting store and forward operation shall be able to inform the application server when the S&F data storage quota for the UE is reached and reject storing further messages.

[PR 5.2.6-007] The 5G system with satellite access supporting store and forward operation shall support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. delivery priority, acknowledgment policy).

Editor’s Note: this requirement is FFS.

[PR 5.2.6-008] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied.

End of Changes