3GPP TSG-SA WG1 Meeting #101 	S1-230457

Athens, Greece, 20 - 24 February 2023	(revision of S1-230450, S1-230129)

Title:	Use case on service continuity for UE-to-UE communication across multiple satellites

Agenda Item:	7.7

Source:	ETRI, Novamint, Thales, Eutelsat, Lockheed Martin, KT Corporation, GateHouse

Contact:	JaeSheung Shin (sjs@etri.re.kr)

Abstract: This document proposes a use case on service continuity for UE-to-UE communication across multiple satellites and relevant requirements for TR22.865 v0.2.0 (FS_5GSAT_ph3).

* * * First Change * * * *

# 2	References

[x1]	Xingqin Lin, Stefano Cioni, Gilles Charbit, Nicolas Chuberre, Sven Hellsten, and Jean-Francois Boutillon, “On the Path to 6G: Embracing the Next Wave of Low Earth Orbit Satellite Access,” IEEE Communications Magazine 59(12), Dec. 2021, pp.36-42

[x2]	https://www.maine.gov/governor/mills/news/old-town-governor-mills-unveils-states-new-helicopter-fight-forest-fires-assist-search-rescue

[x3]	 “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020”, https://www.itu.int/hub/publication/r-rep-m-2514-2022/

* * * End of First Change * * * *

* * * Second Change * * * *

## 5.x.	Use case on service continuity for UE-to-UE communication across multiple satellites

### 5.x.1	Description

The provision of Internet services using mega-constellations of LEO (Low Earth Orbit) satellites is a promising solution on the path to the future mobile communication systems. LEO is the Earth-centered orbit with an altitude in the range of 350km and 2000km above sea level. The LEO satellites at 600km altitude travel at a speed of about 7.8km/sec[x1]. Due to the fast movement of LEO satellite, the service duration of a satellite for the coverage with 1000km diameter is less than 3 minutes. Therefore, guaranteeing robust service continuity and satisfactory user experience is the most critical issue in LEO satellite system.

In some countries, the state government operates Aviation Branches for Forest Protection Service to fight forest fires and assist in search and rescue missions[x2]. The helicopters are part of the Aviation Branch and used for fire detection and firefighting, dropping water, and moving firefighters and equipment to rural and remote locations. In these locations, there may be no terrestrial network, so the helicopters and firefighters can collaborate with each other by communicating in the help of satellite. Moreover, since it takes several hours or days to complete their missions, it should be considered how to ensure the continuity of communication service using satellite access across multiple NGSO satellites.

Furthermore, the draft report of ITU-R for IMT2020-satellite requirement for mobility interruption time in satellite radio interfaces is 50ms [x3].

Figure 5.x.1-1: Service continuity for UE-to-UE communication across multiple satellites

without going through the ground network

### 5.x.2	Pre-conditions

Satellite operator Sat-OP has deployed NGSO satellites and has an agreement with Terrestrial Operator Ter-OP to provide communication services for UEs under satellite coverage.

Firefighter A and B have signed contract with Sat-OP for communication services using satellite access. Thus, their devices can communicate with each other directly via satellite without going through the ground network.

Firefighter A and B move to the rural or remote area in which there is no terrestrial network, but the satellites operated by Sat-OP can provide communicate services.

### 5.x.3	Service Flows

- Firefighter A and B make a phone call or exchange some data (e.g. pictures, video streams) during their work. Then, their data traffic is routed through satellite Sat-1.

- During the communication service, if Firefighter B is located in the coverage of satellite Sat-2, Firefighter B has a connection to satellite Sat-2 and the communication between Firefighter A and B is provided by satellite Sat-1 and Sat-2 through inter satellite link.

- After sometime, if satellite Sat-2 serves the area in which Firefighter A and B are located, the satellite Sat-2 takes over the data sessions for Firefighter A and B from satellite Sat-1, and then the data traffic is routed through satellite Sat-2.

### 5.x.4	Post-conditions

Firefighter A and B can finish the phone call or data exchange without any discontinuation of communication service with the support of multiple NGSO satellites.

### 5.x.5	Existing features partly or fully covering the use case functionality

Regarding TS 22.261 [2], satellite access and satellite connectivity are supported in Rel-18, as

The 5G system shall be able to provide services using satellite access.

The 5G core network shall support collection of charging information based on the access type (e.g. 3GPP, non-3GPP, satellite access).

For a 5G system with satellite access, the following requirements apply:

-	A 5G system with satellite access shall enable roaming of UE supporting both satellite access and terrestrial access between 5G satellite networks and 5G terrestrial networks.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] Subject to regulatory requirements and operator’s policy, the 5G system with satellite access shall support  service continuity, in case that the communication between UEs using the satellite access without going through the ground network should be served across multiple successive satellites.

[PR 5.x.6-002] Subject to regulatory requirements and operator’s policies, the 5G system with satellite access shall be able to support takeover of data sessions from one serving satellite to another without going through the ground network, while preserving integrity protection, confidentiality and security of the communication.

NOTE: 	The IMT-2020 requirement for mobility interruption time in satellite radio interfaces is 50ms [x3]

[PR 5.x.6-003] The 5G system with satellite access shall support service continuity of a communication between UEs via one or several satellites (through inter satellite links) without going through the ground network.

* * * End of Second Change * * * *

* * * Third Change * * * *

## Definitions of terms, symbols and abbreviations

## 3.1      Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

satellite access: direct connectivity between the UE and the satellite.

serving satellite: a satellite providing the satellite access to an UE. In the case of NGSO (Non-Geostationary Satellite Orbit), the serving satellite is always changing due to the nature of the constellation.

* * * End of Third Change * * * *