3GPP TSG SA WG 1 Meeting #101 	S1-230467

Athens, GR, 20 – 24 February 2023	(revision of S1-230455, S1-230057)

Source:	Nokia, Novamint, Sateliot, Gatehouse

pCR Title:	22.865 pCR: Updates in use case 5.1 to align service flows and potential new requirements

Draft Spec:	3GPP TR 22.865-020

Agenda item:	7.7 (FS_5GSAT_Ph3: Study on satellite access - Phase 3)

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.1 in FS_5GSAT_Ph3 TR.

1. Introduction

This contribution proposes to update current use cases on store and forward - MO.

2. Reason for Change

Current service flows, post-conditions and PRs are not aligned.

Definition on S&F data retention period as well as new requirements on S&F data retention period and S&F data storage quota are introduced.

In addition, editorial changes are also needed.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865-020.

First Change (Definitions)

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

satellite access: direct connectivity between the UE and the satellite.

NOTE: 	This definition was taken from TS 22.261 [2].

Store & Forward Satellite operation: in the context of this study, it is an operation mode of a 5G system offering satellite-access where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

S&F data retention period: it is the data storage validity period for the 5G system with satellite access supporting store and forward operation (e.g. after which undelivered data stored is being discarded).

Second Change (Description)

### 5.1.1	Description

This use case illustrates the realization of a S&F service between a UE with satellite access and an Application Server for a delay-tolerant/non-real-time IoT NTN service in the case of a Mobile Originated message.

A description of store and forward operation is provided in Annex A.

Company TrackingInc offers a service of remote monitoring of fields and deploys and tracks many battery-powered IoT type UEs across the globe. All the IoT remote monitoring UEs deployed include a 5G communication with satellite access. Some of the UEs are deployed in a remote area where there is no mobile coverage by MNO and only satellite is possible.

For the satellite access, TrackingInc uses the service of IoTSAT for the 5G IoT connectivity by satellite, and IoTSAT uses a LEO constellation which supports S&F operation mode.

All IoT remote monitoring UEs regularly send information related to the area they are monitoring to the application server of TrackingInc and sometimes receive new parameters from the application server. In most of the cases, the messages exchanged are delay-tolerant/non-real-time IoT.

Third Change (Service Flows)

### 5.1.3	Service Flows

The IoT remote monitoring UE needs to send a message to the TrackingInc application server. The UE waits for satellite network coverage and sends its message when the satellite passes by.

The IoT remote monitoring UE and the satellite providing coverage interact over the service link, allowing the UE to transfer the message to the satellite, which has no connectivity to the ground segment. And consequently, the satellite has to store locally the received message.

At this point:

- Limitations to the size/amount of data that can be sent from the UE could be enforced.

- Forwarding priority for the stored data to the ground station and data retention period for the exchanged data could be established.

- Acknowledgement of the received data by the satellite could be issued.

At a later time, the satellite with the stored message establishes connectivity with the ground network via a feeder link and relays/forwards/downloads the message to the ground network.

The ground network, based on established connectivity configuration and routing, delivers message to the TrackingInc application server.

Fourth Change (Post-conditions)

### 5.1.4	Post-conditions

The message generated by the IoT remote monitoring UE has been either delivered successfully to the TrackingInc application server without relying on a continuous end-to-end network connectivity path between them or, in case the data retention period has been exceeded, the message has been discarded.

Fifth Change (PRs)

### 5.1.6	Potential New Requirements needed to support the use case

[PR 5.1.6-001] The 5G system with satellite access shall be able to support store and forward operation.

[PR 5.1.6-002] The 5G system with satellite access shall be able to inform a UE that store and forward operation is applied.

[PR 5.1.6-003] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data retention period.

[PR 5.1.6-004] Subject to operator policy, the 5G system with satellite access supporting store and forward operation shall be able to allow the operator or a trusted 3rd party to set and enforce, on a per UE basis, a S&F data storage quota.

[PR 5.1.6-005] The 5G system with satellite access supporting store and forward operation shall be able to support a mechanism to configure and provision specific required QoS and policies for S&F operation (e.g. forwarding priority, acknowledgment policy).

[PR 5.1.6-006] The 5G system with satellite access shall be able to provide integrity protection and confidentiality for communications between an authorized UE and the network when store and forward operation is applied.

End of Changes