3GPP TSG SA WG 1 Meeting #101 	S1-230485

Athens, GR, 20 – 24 February 2023	(revision of S1-230426)

Source:	Nokia

pCR Title:	Revision of use-case 5.7 Immersive AR experience

Draft Spec:	3GPP TR 22.856-030

Agenda item:	7.3

Document for:	Approval

Contact:	Orlett PEARSON, orlett.pearson@nokia.com

Laurent-Walter GOIX, laurent-walter.goix@nokia.com

Thibaud BIATEK, thibaud.biatek@nokia.com,

Abstract: This document proposes a revision of use-case 5.7

1. Introduction

The TR 22.856-030 introduces one entertainment use-case, namely “5.7: AR Enabled Immersive Experience”. This pCR provides changes to better take into account ecosystem aspects related to such metaverse experience.

230426 changes :

- Deleted PR#2

- Clarified the notions of extended period of time (eg 2h) and mobility (in the train)

- Clarified the multiparty communications

- Added references to KPI values

230485 changes :

- merged with change to 5.7 from 230402

2. Reason for Change

Use case 5.7 deals with traditional live or on-demand video streaming service in an AR environment, providing virtual screen incrustation in the user’s field of view (FOV), thereby indicating that the experience might be shared with friends at home or in mobility.

It is understood that this Metaverse experience is composed of one or several elements. First, a static screen overlayed in the user’s Field of View (FOV) on which the video is displayed. Second, one or several friends can be overlayed in the user’s FOV, for instance, sitting next to them on the train or the bus.

- Issue 1: The collaborative aspect of the experience is not reflected in the current description. In its current form, the use case describes a traditional video streaming service that superimposes reality, thanks to AR glasses. This multi-viewer aspect involves specific stream synchronization aspects which are not covered in the current requirements.

It is understood that the use case typically addresses on-demand streaming, e.g., movie viewing. The scene overlayed in the user’s FOV is composed of a movie screen, a few avatars, a chatbox etc. In the current description, it needs to be clarified where the viewports are rendered. We foresee the following possible cases:

- Case 1: The viewports are rendered in the edge and streamed down to the AR glasses. The viewports are either decoded and displayed directly on the AR glasses through NG-RAN or into a tethered UE through a direct device connection. For this case, the currently envisaged datarate (up to 2Gbit/s) is not realistic nor reasonable (also from a network energy sobriety perspective).

- Case 2: The viewports are rendered in a UE tethered to the AR glasses using a direct device connection. Here the glasses act as a display; thus, a low-delay, high-data-rate codec is used between the UE and the AR glasses.

- Case 3: The UE is in the AR glasses, and the viewports are rendered or represented visually. Advanced SoCs embed low-power hardware for both viewport rendering and video decoding; thus, the rendering can be achieved on the device with minimal energy impact on both the network and the device. A study on energy consumption of hardware video decoders [1] shows that a typical HEVC hardware decoder embedded on an android device spends 40-50mA per hour of video playback (decoding only). Hence, usage of the hardware decoder seems reasonable given the 1000mAh battery capacity assumption made in the current description.

- Issue 2: The current description is not clear enough regarding the different possible delivery cases, which should be more explicit as they trigger different requirements and KPIs.

3. Conclusions

Proposed changes have been identified and should be incorporated in 22.856. The major changes are the following :

- Add requirements on users synchronization in the context of a collaborative viewing experience

- Refine KPIs to consider the different dataflows (video DL, avatars, communications, speech).

The proposed KPIs are either based on the current use case or aligned with the ones already mentioned in 22.261, in particular in sections 7.6 and 7.11.

4. Proposal

It is proposed to agree to the following changes to 3GPP TR 22.856.

References

[1] R. Kazantsev and D. Vatolin, "Power Consumption of Video-Decoders on Various Android Devices," 2021 Picture Coding Symposium (PCS), Bristol, United Kingdom, 2021, pp. 1-5, doi: 10.1109/PCS50896.2021.9477481.

* * * Start of Change #1 : add reference * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem (IMS)".

[3]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services".

[4]	3GPP TS 22.101: "Service principles".

[5]	3GPP TS 22.261: "Service requirements for the 5G system".

[6] 	Talaba D, Horvath I, Lee K H, "Special issue of Computer-Aided Design on virtual and augmented reality technologies in product design," Computer-Aided Design 42:361 - 363.

[7] 	Wang X, Tsai J J-H, "Collaborative Design in Virtual Environments," In: ISCA 48:1-2.

[8] 	Benford S, Greenhalgh C, Rodden T, "Collaborative virtual environments." Communications of the ACM 44: 79–85.

[9]	M. Eid, J. Cha, and A. El Saddik, "Admux: An adaptive multiplexer for haptic-audio-visual data communication", IEEE Tran. Instrument. and Measurement, vol. 60, pp. 21–31, Jan 2011.

[10]	K. Iwata, Y. Ishibashi, N. Fukushima, and S. Sugawara, "QoE assessment in haptic media, sound, and video transmission: Effect of playout buffering control", Comput. Entertain., vol. 8, pp. 12:1–12:14, Dec 2010.

[11]	N. Suzuki and S. Katsura, "Evaluation of QoS in haptic communication based on bilateral control", in IEEE Int. Conf. on Mechatronics (ICM), Feb 2013, pp. 886–891.

[12]	E. Isomura, S. Tasaka, and T. Nunome, "A multidimensional QoE monitoring system for audiovisual and haptic interactive IP communications", in IEEE Consumer Communications and Networking Conference (CCNC), Jan 2013, pp. 196–202.

[13]	A. Hamam and A. El Saddik, "Toward a mathematical model for quality of experience evaluation of haptic applications", IEEE Tran. Instrument. and Measurement, vol. 62, pp. 3315–3322, Dec 2013.

[14]	O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

[15]	Lee, Donghwan et. al., “Large-scale Localization Datasets in Crowded Indoor Spaces,” NAVER LABS, NAVER LABS Europe, CVPR 2021.

[16]	3GPP TR 22.837, " Study on Integrated Sensing and Communication."

[17]	Alriksson, F., Kang, D.H., Phillips, C., Pradas, J.L. and Zaidi, A., 2021. XR and 5G: Extended reality at scale with time-critical communication. Ericsson Technology Review.

[18]	3GPP TR 26.928, "Extended Reality (XR) in 5G."

[19] 	Ge, Y., Wen, F., Kim, H., Zhu, M., Jiang, F., Kim, S., Svensson, L. and Wymeersch, H., 2020. 5G SLAM using the clustering and assignment approach with diffuse multipath. Sensors, 20(16), p.4656.

[20] 	Kim, H., Granström, K., Gao, L., Battistelli, G., Kim, S. and Wymeersch, H., 2020. 5G mmWave cooperative positioning and mapping using multi-model PHD filter and map fusion. IEEE Transactions on Wireless Communications, 19(6), pp.3782-3795.

[21] 	Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[22] 	Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[23]	Suhr, C. "S.L.A.M. and Optical Tracking for XR", https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f <accessed 1.9.22>

[24]	Kim, H., Granstrom, K., Svensson, L., Kim, S. and Wymeersch, H., 2022. PMBM-based SLAM Filters in 5G mmWave Vehicular Networks. IEEE Transactions on Vehicular Technology.

[25]	5GAA " C-V2X Use Cases Volume II: Examples and Service Level Requirements", 5G Automobile Assocaition White Paper,  https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf <accessed 02.09.22>

[26]	A. Ebrahimzadeh, M. Maier and R. H. Glitho, "Trace-Driven Haptic Traffic Characterization for Tactile Internet Performance Evaluation," 2021 International Conference on Engineering and Emerging Technologies (ICEET), 2021, pp. 1-6.

[27]	Lee, L.-H., Braud, T., Zhou, P., Wang, L., Xu, D., Lin,  Z., Kumar, A., Bermejo, C., and Hui, P. All One Needs to Know about Metaverse: A Complete Survey on Technological Singularity, Virtual Ecosystem, and Research Agenda, 2021.

[28]	Halbhuber, David & Henze, Niels & Schwind, Valentin. (2021). Increasing Player Performance and Game Experience in High Latency Systems. Proceedings of the ACM on Human-Computer Interaction. 5. 1-20. 10.1145/3474710.

[29] 	Holland, O., Steinbach, E., Prasad, R.V., Liu, Q., Dawy, Z., Aijaz, A., Pappas, N., Chandra, K., Rao, V.S., Oteafy, S. and Eid, M., 2019. The IEEE 1918.1 “tactile internet” standards working group and its standards. Proceedings of the IEEE, 107(2), pp.256-279.

[30] 	Skalidis, I., Muller, O. and Fournier, S., 2022. CardioVerse: The Cardiovascular Medicine in the Era of Metaverse. Trends in Cardiovascular Medicine.

[31] 	Koo, H., 2021. Training in lung cancer surgery through the metaverse, including extended reality, in the smart operating room of Seoul National University Bundang Hospital, Korea. Journal of educational evaluation for health professions, 18.

[32] 	Mozumder, M.A.I., Sheeraz, M.M., Athar, A., Aich, S. and Kim, H.C., 2022, February. Overview: technology roadmap of the future trend of metaverse based on IoT, blockchain, AI technique, and medical domain metaverse activity. In 2022 24th International Conference on Advanced Communication Technology (ICACT) (pp. 256-261). IEEE.

[33] 	Ning, H., Wang, H., Lin, Y., Wang, W., Dhelim, S., Farha, F., Ding, J. and Daneshmand, M., 2021. A Survey on Metaverse: the State-of-the-art, Technologies, Applications, and Challenges. arXiv preprint arXiv:2111.09673.

[34] 	https://www.xrtoday.com/virtual-reality/ukrainian-doctors-perform-first-vr-surgery/

[35] 	https://www.cryptotimes.io/surgeon-performed-surgery-remotely-through-metaverse/

[36]	 Senk, S., Ulbricht, M., Tsokalo, I., Rischke, J., Li, S.C., Speidel, S., Nguyen, G.T., Seeling, P. and Fitzek, F.H., 2022. Healing Hands: The Tactile Internet in Future Tele-Healthcare. Sensors, 22(4), p.1404.

[37] 	Chen, D. and Zhang, R., 2022. Exploring Research Trends of Emerging Technologies in Health Metaverse: A Bibliometric Analysis. Available at SSRN 3998068.

[38] 	https://www.yankodesign.com/2022/05/15/the-metaverse-has-the-power-to-improve-healthcare-and-it-has-already-begun/

[39] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[40] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[41] 	3GPP TR 22.826 release-17 - https://www.3gpp.org/ftp/Specs/archive/22_series/22.826/22826-h20.zip

[42] 	Tataria, H., Shafi, M., Molisch, A.F., Dohler, M., Sjöland, H. and Tufvesson, F., 2021. 6G wireless systems: Vision, requirements, challenges, insights, and opportunities. Proceedings of the IEEE, 109(7), pp.1166-1199.

[43]	3GPP TS 23.682, "Architecture enhancements to facilitate communications with packet data networks and applications".

[44]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2".

[45]	"MPEG-4 Face and Body Animation", https://visagetechnologies.com/mpeg-4-face-and-body-animation/ (accessed 2.11.22).

[46]	3GPP TS 22.105, "Services and service capabilities".[47]	Virtual humans: https://en.wikipedia.org/wiki/Virtual_humans

[48]	Kwang Soon Kim, et al., "Ultrareliable and Low-Latency Communication Techniques for Tactile Internet Services", PROCEEDINGS OF THE IEEE, Vol. 107, No. 2, February 2019

[49]	I.-J. Hirsh and C. E. J. Sherrick, “Perceived order in different sense modalities,” Journal of Experimental Psychology, vol. 62, no. 5, pp. 423–432, 1961. [50]                       VESA Compression Codecs - vesa.org/vesa-display-compression-codecs

[51]	3GPP TR 23.700-80: "Study on 5G System Support for AI/ML-based Services".

[52]	EU data protection rules | European Commission (europa.eu) (https://ec.europa.eu/info/law/law-topic/data-protection/eu-data-protection-rules_en)

[53] 	California Consumer Privacy Act (CCPA) | State of California - Department of Justice - Office of the Attorney General (https://oag.ca.gov/privacy/ccpa)

[X] 	R. Kazantsev and D. Vatolin, "Power Consumption of Video-Decoders on Various Android Devices," 2021 Picture Coding Symposium (PCS), Bristol, United Kingdom, 2021, pp. 1-5, doi: 10.1109/PCS50896.2021.9477481.

[Y]	glTF 2.0 specification, https://registry.khronos.org/glTF/specs/2.0/glTF-2.0.html, accessed 02/02/2023

* * * End of Change #1 * * *

* * * Start of Change #2 : revised use-case* * *

## 5.7	AR Enabled Immersive Experience

### 5.7.1	Description

In addition to watching movies at the cinema, people will also choose to watch a movie on their mobile phones, laptops, or TVs when they don’t have time to go to the cinema, e.g., when travelling or at home. However, by watching through these terminals,  users will feel more or less uncomfortable in the neck or cervical spine because the users always keep their necks down. Moreover, the screen is relatively small; if users want to see more realistic screen details or watch an immersive 3D movie, using these terminals is not feasible.

Figure 5.7.1-1: AR Enabled Immersive Experience

Editor’s note: Confusing illustration, also missing source/licensing information. It would be relevant to show a “see-through” vision of the screen in the AR glasses (and not floating in the air), ideally with other people sharing the experience around.

In this use case, users can get an immersive location agnostic service experience of watching movies in certain circumstances, such as at home or on the train. They can even invite some of their friends to watch a movie at different places simultaneously by wearing a wearable device, such as AR glasses. A large screen like a movie theatre will be presented in the field of vision (FOV) of the wearable device, which not only provides an immersive watching experience like private cinema but also has very low demands on the environment and space of the user's location, 3D cinematic effects can also be easily rendered in the device.

To achieve an immersive location agnostic service experience through AR glasses, a 5G system is required to provide a reliable transmission of uplink and downlink data and a way for users to synchronize their experience and interact together. In mobility scenarios, e.g., when a user is travelling on the train, the continuity of data transmission also needs to be guaranteed. Moreover, when AR glasses are wireless, the power supply relies on the battery integrated inside the AR glasses. This use case investigates how the 5G system (through direct device connection or NG-RAN) can be used to support UE to establish a data connection with the mobile metaverse server. The 5G system can provides services to AR glasses so as to minimize energy consumption in the overall system.

The service dataflows and requirements may differ depending on whether the AR glasses are accessing the service through a direct device connection or NG-RAN.

### 5.7.2	Pre-conditions

Bob wants to watch an AR movie with friends to relax while travelling in the train. So he wears wearable equipment such as AR glasses that can access into the 5G network (through direct device connection or NG-RAN) to access metaverse services.

Bob has subscribed to an immersive movie service as a mobile metaverse service that he can access via AR glasses. The service gives Bob access to a large movie catalog (2D/3D). The battery capacity of the AR glasses is enough to watch a two-hour movie. Before starting the movie, Bob can invite some friends to join him. If people join Bob, their avatars are also placed into his FOV and Bob can interact with them (speech or text).

Considering the wearing comfort of users, mainstream AR glasses should not be too heavy (normally less than 150 grams). The maximum capacity of the battery (50 grams) is about 1000mAh. Usually, 25% of the battery capacity of AR glasses is allocated to the mobile termination of the UE.

When a user watches a 4K movie, some video compression techniques are usually used to reduce the amount of data transmitted while maintaining the image quality.

In general, a large video compression ratio will cause a delay increase. Considering the overall factors of delay and energy consumption, using AR glasses with a direct device connection would require a low compression ratio, for instance, 3:1 [50]. However , some advanced AR glasses SoC embeds hardware video decoders (e.g., AVC, HEVC, and VVC) and can render viewport efficiently.

A study on energy consumption of hardware video decoders [X] shows that a typical HEVC hardware decoder embedded on an Android device is spending 40-50mA per hour of video playback (decoding only). The usage of hardware decoders seems reasonable, given the 1000mAh battery capacity assumption made in the current description. For the NG-RAN case, it is reasonable to think that AR glasses can decode and render efficiently with low energy consumption.

### 5.7.3	Service Flows

1. Data connections are set up between AR glasses and the Metaverse server, which can provide immersive location agnostic service experience of a movie service. The 5G module can be connected to the 5G network either via direct device connection or NG-RAN.

2. The mobile metaverse server provides movie access to the client AR glasses through the downlink data stream.

3. The mobile metaverse server manages communications between clients (friends), e.g., including video, avatar, speech, and text.

4. The mobile metaverse server manages synchronization between the clients (e.g., the various AR glasses) of the friends.

### 5.7.4	Post-conditions

Bob is able to watch an immersive movie with friends, while travelling, obtaining a good user experience. The battery capacity of the AR glasses is enough to watch a two-hour movie.

Bob is able to communicate with them in a synchronized manner. Bob’s friends (or avatars) can be visible in his FOV.

The 5G system is able to support communication for immersive location agnostic AR services, providing a reliable transmission, a continuity of service and a synchronized experience across users sharing a viewing experience.

### 5.7.5	Existing features partly or fully covering the use case functionality

The performance requirements for high data rate AR services have been captured in TS 22.261 clause 7.6. The performance requirements for UE to network relaying in 5G systems have been captured in TS 22.261 clause 7.7. The functional and performance requirements for tactile and multi-modal communication services have been captured in TS 22.261 clauses 6.43 and 7.11, respectively.

However, existing requirements still need to consider the power consumption of the 5G UE onboard AR terminals.

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-1] Subject to operator policy, the 5G system shall support a means to provide high data rate to a UE during an extended period of time, including when in high-speed mobility.

NOTE 1: Metaverse service experience over an extended period of time (e.g. 2h)  requires significant power consumption by the UE. In some cases, a device with no external power supply cannot sustain downloading and rendering of media over a long interval, e.g., for the duration of an entire feature film or athletic event.

[PR 5.7.6-2] Subject to operator policy, the 5G system shall support a mechanism that enables flexible adjustment of communication services based on the type of devices (e.g., wearables), such that the services can be operated with reduced energy utilization.

[PR 5.7.6-3] Subject to operator policy, the 5G system shall support a means to enable interactive immersive multiparty communications in the metaverse service.

NOTE 2: The multiparty immersive experience (e.g. amongst multiple friends) could be location related or location agnostic.

Table 5.7.6-1 – Potential key performance requirements for Immersive AR interactive experience: tethered link

|  | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |
| Viewports streaming from rendering device to AR glasses through direct device connection (tethered/relaying case) (note 1) | 10 ms (i.e., UL+DL between AR Glasses display and the rendering UE) (note 2) | [200-2000] Mbit/s | 99,9 % (note 2) | 1-2 | Stationary or pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Pose information from AR glasses to rendering device through direct device connection (tethered/relaying case) (note 1) | 5 ms (note 2) | [100-400] Kbit/s (note 2) | 99,99 % (note 2) | 1-2 | Stationary or pedestrian | 20 m x 10 m; in one vehicle (up to 120 km/h) and in one train (up to 500 km/h) |
| Note 1: These KPIs are only valid for cases where the viewport rendering is done in the tethered device and streamed down to the AR glasses. In the case of rendering capable AR glasses, these KPIs are not valid. Note 2: These values are aligned with the tactile and multi-modal communication KPI table in TS 22.261 [5], cl 7.11 |  |  |  |  |  |  |

Table 5.7.6-2 – Potential key performance requirements for Immersive AR interactive experience: NG-RAN multimodal communication link

|  | Characteristic parameter (KPI) |  |  | Influence quantity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | # of UEs | UE Speed | Service Area |
| Movie streaming from metaverse server to the rendering device (note 2) | Only relevant for live streaming. [1-5] s in case of live streaming | [0,1-50] Mbit/s (i.e., covering a complete OTT ladder from low resolution to 3D-8K) (note 1) | 99,9 % | 1 to [10] | 0-500km/h | - |
| Avatar information streaming between remote UEs (end to end) (note 3) | 20ms (i.e., UL between UE and the interface to metaverse server + DL back to the other UE) | [0,1-30] Mbit/s | 99,9 % | 1 to [10] | 0-500km/h | - |
| Interactive data exchange: voice and text between remote UEs (end to end) (note 4) | 20ms (i.e., UL between UE and the interface to metaverse server + DL back to the other UE) | [0,1-0,5] Mbit/sec | 99,9 % | 1 to [10] | 0-500km/h | - |
| Note 1: aligned with “high-speed train” DL KPI from TS 22.261 cl 7.1 Note 2: To leverage existing streaming assets and delivery ecosystem, it is assumed that the legacy streaming data are delivered to the rendering device, which incrusts this in the virtual screen prior to rendering. For a live streaming event, the user-experience end-to-end latency is expected to be competitive with traditional live TV services, typically [1-5] seconds. Note 3: For example, the glTF format [Y] can be used to deliver avatar representation and animation metadata in a standardized manner. Based on this format, the required bitrate for transmitting such data is highly dependent on avatar’s complexity (e.g., basic model versus photorealistic). Note 4: These values are aligned with “immersive multi-modal VR” KPIs in TS 22.261 [5], cl 7.11. End-to-end latency in this table is calculated as twice the value of the DL “immersive multi-modal VR” latency in TS 22.261 [5], cl 7.11. |  |  |  |  |  |  |

* * * End of Change #2 * * *