3GPP TSG SA WG 1 Meeting #101 	S1-230526

Athens, Greece, 20 – 24 February 2023	(revision of S1-230122)

Source:	Qualcomm

pCR Title:	Update to Use case 5.7

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This contribution discusses updates to Clause 5.7.

1. Introduction

Introduce updates to Clause 5.7

2. Reason for Change

- Updated KPI values for communication range  and device densityto NA

- Removed Editor’s Notes from Clauses 5.7.5 and 5.7.6 .

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840, V1.0.0

* * * First Change * * * *

### 5.7.5	Existing features partly or fully covering the use case functionality

None

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] The 5G system shall be able to support means to discover and locate Ambient-IoT devices in a certain geographical area, e.g. at cell level.

[PR 5.7.6-002] The 5G system shall be able to provide communication with Ambient-IoT devices with the following KPIs :

Table 5.7.6-1:  KPI Requirements for Airport Terminal/Shipping Port Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airport Terminal/ Shipping Port | >1sec | 99% | NA | NA | 256 bits (UL) (Note 1) | >100 devices/1km2 | 50m | 1-10km2 (Note 2) | NA | NA | NA | NA | NA |
| Note 1: 128 bits for the Electronic Product Code (EPC) of the tracked object and additional 128 bits assumed for control / other data (e.g., location-related). Note 2: As an example, Newark Airport size ~8km2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of First Change * * * *