3GPP TSG SA WG 1 Meeting #101 	S1-230539

Athens, Greece, 20 - 24 February 2023	(revision of S1-230116)

Source:	NTT DOCOMO, NTT

pCR Title:	Update use case on sensing for flooding in smart cities

Draft Spec:	3GPP TR 22.837 version 0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Atsushi Minokuchi, minokuchi@nttdocomo.com

Yutaka Musaka, yutaka.musaka.nw@nttdocomo.com

Yoshifumi Morihiro, morihiro@nttdocomo.com

Shinya Otsuki, shinya.otsuki.ma@hco.ntt.co.jp

Tomoki Murakami, tomoki.murakami.nm@hco.ntt.co.jp

Abstract: KPI table and NOTE are added to the use case on sensing for flooding in smart cities.

1. Introduction

<Introduction part >

2. Reason for Change

KPI table is now missing; it is to be added. Information used to derive KPI values is added as NOTE.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 version 0.3.0.

* * * First Change * * * *

5.5	Use case on sensing for flooding in smart cities

5.5.1	Description

Due to the climate change in recent years, a larger amount of rain sometimes falls within a short duration of time inside a small area. This result, in particular in urban areas, in inundation and flooding even in areas where these did not happen in the past. When flooding is about to happen on roads, people might enter areas getting in danger without knowing it. Once flooding really happens there, this might result in loss of human life. At places where flooding is expected to occur, monitoring of flooding is performed using cameras and other sensors. However, due to the recent climate change, it may be difficult to recognize places where flooding is expected to occur. Using radio waves, it is possible to recognize places where flooding occurs in an efficient way.

NOTE: 	There has been a related trend, although a mobile communication is not directly involved so far and it's monitoring of the river, not of the road as this use case deals with. In Japan, MLIT (Ministry of Land, Infrastructure, Transport and Tourism) takes care of the river administration and supervises water level observation of rivers to prevent and predict flooding. In the past, water-level gauges were only sparsely deployed along the river. Water levels at places where those gauges were not placed were estimated based on water levels observed some distance away where such gauges were placed. Detailed degree of possibility of flooding at each place was not directly understood. To improve this situation, MLIT has encouraged to develop a low-cost water-level gauge and has started placing such gauges e.g., at places that are relatively prone to flooding or show a specific water behavior due to the form of the river, or that are close to hospitals or important facilities. Disaster Information for River is now available at https://www.river.go.jp/e/ for public.

5.5.2	Pre-conditions

Good partnership and cooperation are established between Mobile Operator #A and administrators of roads such as a local government in City #B. Mobile Operator #A constantly senses the surface of the road and informs results of sensing to the administrator of the road.

5.5.3	Service Flows

Figure 5.5.3-1: Sensing for flooding in smart cities

- Base stations owned by Mobile Operator #A are deployed around the road. Mobile Operator #A carries out sensing of the surface of the road in City #B. This sensing is performed using radio wave. Results of sensing information, incl. whether flooding occurs on the road, are informed to the administrator of the road in City #B.

- The administrator of the road usually monitors the state of flooding on the road using information from sensors including information from Mobile Operator #A. In addition, in the case of heavy rain, the administrator may request Mobile Operator #A to increase frequency of monitoring of situation of roads and Mobile Operator #A monitors the situation more frequently responding to this request.

- If there is information received that flooding occurs, the administrator advises people in the areas concerned to evacuate the areas. The administrator advises via mobile networks.

- People who received the advice evacuate the areas or do not enter such areas.

- Now City #B trusts Mobile Operator #A and allows it to advise people about evacuation without City #B's intervention in case of flooding. Next time a similar flooding occurs, Mobile Operator #A sends advice for evacuation directly to people.

5.5.4	Post-conditions

Damage of the flooding has been kept at minimum.

5.5.5	Existing features partly or fully covering the use case functionality

None.

5.5.6	Potential New Requirements needed to support the use case

[PR. 5.5.6 - 001] Subject to operator policy, the 5G system shall be able to provide sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area to authorized third parties in a timely manner.

[PR. 5.5.6 - 002] Subject to regional or national regulatory requirements and operator policy, the 5G system shall be able to provide its public warning system with a warning notification based on sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area in a timely manner.

[PR. 5.5.6 - 003] The 5G system shall be able to support the following KPIs:

Table 5.5.6-1	Performance requirements of sensing for flooding in smart cities

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| sensing for flooding in smart cities | Outdoor | 95 | ≤10 | [≤0.2] NOTE 1 | N/A | N/A | N/A | N/A | ≤ 1min NOTE 2 | < 1min NOTE 2 | < 0.1 | < 3 |
| NOTE: The terms in Table 5.5.6-1 are found in Section 3.1. NOTE 1: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. NOTE 2: Description related to NOTE in clause 5.5.1 suggests 2 minute-interval monitoring when the water level of the river rises quickly. |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *