3GPP TSG SA WG 1 Meeting #101 	S1-230610

Athens, Greece, 20 – 24 February 2023	(revision of S1-230183)

Source:	Huawei

pCR Title:	Pseudo-CR on updates to clause 5.3

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	Alice Li (alice.li1@huawei.com)

Abstract: This document provides a TP to TR 22.840 to update clause 5.3 on the use case of Ambient IoT devices in substations in smart grids.

Revision is to address the comment received.

1. Introduction

The use case of “Ambient IoT devices in substations in smart grids” has been agreed and captured in TR 22.840, which includes the use case description and some potential requirements. The further clarification is expected to provide more details on the power consumption in such use case.

2. Reason for Change

Additional information on the power consumption is expected in the use case description in order to provide more clarity.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16] 	Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[17] 	Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[18] 	Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[18] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[20] 	Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[21] 	Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[22]	Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[23] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[24] 	H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[25] 	Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

[26] 	https://www.usda.gov/foodwaste/faqs

[27] 	https://www.fda.gov/food/new-era-smarter-food-safety#:~:text=Welcome%20to%20the%20New%20Era,reducing%20the%20number%20of%20illnesses

[28] 	Alam AU, Rathi P, Beshai H, Sarabha GK, Deen MJ. Fruit Quality Monitoring with Smart Packaging. Sensors (Basel). 2021 Feb 22;21(4):1509. doi: 10.3390/s21041509. PMID: 33671571; PMCID: PMC7926787.

[29] 	Taoukis, Petros & Nychas, George-John. (2000). Use of time-temperature integrators and predictive modeling for shelf life control of chilled fish under dynamic storage conditions. International journal of food microbiology. 53. 21-31. 10.1016/S0168-1605(99)00142-7.

[30] 	Mishu, M.K., Rokonuzzaman, M., Pasupuleti, J., Shakeri, M., Rahman, K.S., Hamid, F.A., Tiong, S.K. and Amin, N., 2020. Prospective efficient ambient energy harvesting sources for iot-equipped sensor applications. Electronics, 9(9), p.1345.

[31] 	Bagchi, S., Abdelzaher, T.F., Govindan, R., Shenoy, P., Atrey, A., Ghosh, P. and Xu, R., 2020. New frontiers in IoT: Networking, systems, reliability, and security challenges. IEEE Internet of Things Journal, 7(12), pp.11330-11346.

[32] 	O’Neill, V. and Soh, B., 2022. Improving Fault Tolerance and Reliability of Heterogeneous Multi-Agent IoT Systems Using Intelligence Transfer. Electronics, 11(17), p.2724.

[33] 	Katanbaf, M., Jain, V. and Smith, J.R., 2020. Relacks: Reliable backscatter communication in indoor environments. Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies, 4(2), pp.1-24.

[34] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[35] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[36] 	https://www.mokosmart.com/use-iot-fire-detector-sensor/

[37]  	https://www.5gamericas.org/wpcontent/uploads/2019/07/5G_Americas_URLLLC_White_Paper_ Final__updateJW.pdf

[38] 	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261/22261-j00.zip

[39] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[40] 		     		Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[41]		     		Sensors: Technologies and Global Markets (

https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[42]		     		Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[43]	Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021

[44]                	Journal of Dairy Science - Comparison of 2 systems of pasture allocation on milking intervals and total daily milk yield of dairy cows in a pasture-based automatic milking system, American Dairy Science Association, 2013

[45]	Grazing and automation, Proceedings 4th Meeting European Grassland Federation WG “Grazing”, 2015

[46] 	Grazing Methods, UK Agriculture Food and Environment, https://grazer.ca.uky.edu/content/grazing-methods-which-one-you#:~:text=Strip%20Grazing%20%E2%80%93%20This%20technique%20involves,a%20new%20allocation%20of%20forage.

[47]                	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-    advice/putting-a-size-on-paddocks-30447793.html

[48]	A. van den Pol-van Dasselaar, P. W. Blokland, et al, Beweidbare oppervlakte en weidegang op melkveebedrijven in Nederland (Pasture area and grazing on dairy farms in the Netherlands), Livestock Research Wageningen, 2015

[49]	Laura Paine, A Summary of Dairy Grazing Practices in Wisconsin

[50]	R. J. Rendell, Designing Paddocks on an Irrigated Dairy Farm, Victoria, Australia

[51]	The EU pig meat sector, European Parliament

[52]	http://www.bestgenetics.com.cn/EN/PigFarm1

[53]		    			Lisette. E. van der Zande, et al, Individual detection and tracking of group housed pigs

in their home pen using computer vision, Front. Anim. Sci. 2:669312. doi:

10.3389/fanim.2021.669312, April 2021

[54]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[55]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

[56] 	https://www.concreteconstruction.net/projects/infrastructure/so-many-manholes-so-little-time_o#:~:text=U.S.%20EPA%20estimates%20the%20number,from%20100%20to%20500%20feet.

[57]	http://news.cjn.cn/24hour/wh24/201101/t1276749.htm

[58] 	https://www.dibblecorp.com/insights/manhole-infrastructure-assessments/

[59] 	https://www.francebleu.fr/infos/faits-divers-justice/un-homme-retrouve-mort-dans-une-bouche-d-egout-a-saint-nazaire-1666456765

[60]	https://guancha.gmw.cn/2022-05/16/content_35739227.htm

[61]	Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People's Republic of China

[62]	MEMS gauge pressure sensor NSPGD1 Novosense, liquid level measurement, https://www.novosns.com/Public/Uploads/uploadfile/files/20220726/-897.pdf

[63]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[64]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[65] 	Sung-Pil CHANG, Jinkyo CHOO, Values of Bridge in the Formation of Cities

[66]	https://www.thepaper.cn/newsDetail_forward_4643273

[67] 	https://news.sina.cn/gn/2021-12-19/detail-ikyamrmy9954352.d.html

[68] 	https://www.nytimes.com/2020/12/22/world/europe/genoa-bridge-collapse.html

[69]	https://www.theguardian.com/world/2022/oct/31/india-bridge-collapse-death-toll-rises-rescue-efforts-continue

[70]	Role of the Local Government in Monitoring and Maintenance Bridges, IAPA Proceedings Conference, 2019

[71]	https://elastisense.com/structural-health-monitoring-bridges

[72]	GB/T 39559.2-2020 National Specification for operational monitoring of urban rail transit facilities – Part 2: Bridge

[73]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[74]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[75]	Big data analysis for bridges health monitoring, Tongji University, 2017

[76]	Beijing Dongsha bridge safe operation monitoring, http://www.ccea.zju.edu.cn/_upload/article/files/64/e6/a824c7d04932af200750fbd9e690/fb0a5e9c-a1e7-47b8-bdb4-82b584d1c52f.pdf

[77]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[78]	F. Palacio, et al. (2006). "Active RFID tag with sensing capabilities and low power consumption".

[79]	T. O. John, H. C. Ukwuoma, S. Danjuma and M. Ibrahim, "Energy consumption in wireless sensor network", Energy, vol. 7, no. 8, 2016.

[80]	Tuna, G., and V. C. Gungor. "Energy harvesting and battery technologies for powering wireless sensor networks." Industrial Wireless Sensor Networks. Woodhead Publishing, 2016. 25-38.

[81]	M. Zeinali and J. Thompson, "Impact of Compression and Small Cell Deployment on NB-IoT Devices Coverage and Energy Consumption with a Realistic Simulation Model", Sensors. no. 19, 2021.

* * * Second Change * * * *

## 5.3	Use Case on Ambient IoT devices in substations in smart grids

### 5.3.1	Description

With around 80 million kilometres of transmission and distribution lines worldwide, electricity networks are the backbone of secure and reliable power systems. As stated in the World Energy Outlook 2020 [2], significant investment takes place in new network capacity between 2019 and 2030 as a result of growing demand for electricity, the addition of new renewable generation capacity and the need to develop smart grids. The expansion of electricity networks to 2030 is about 80% more than over the past decade. Around 30% of the increase in transmission lines and 20% of the increase in distribution network lines are attributable to the increase of renewables. Over the next ten years, around 16 million km of existing distribution lines and 1.5 million km of transmission lines need to be replaced or digitalised, together with switching equipment, transformers, meters and other crucial components. In regions with older power systems, such as the United States and the European Union, roughly one-fifth of current networks need to be replaced or digitalised; this corresponds to 2.7 million km in the United States and 3.7 million km in the European Union. More than 60% of global line replacements and new lines are in emerging market and developing economies, with China alone accounting for a third of what is needed (over 7 million km).

Smart grids with wide use of IoT devices have a vital role to play in supporting the penetration of variable renewables electricity sources. IoT offers a wide number of applications in the energy sector, i.e., in energy supply, transmission and distribution, and demand [3]. In particular substations are a significant part of the electrical power grid. Through these stations, the voltage level is converted from high voltage to low voltage using (transformer). The substation transfers power to distribution stations by the transmission lines (see (a) in Figure 5.3.1-1). Monitoring electrical substations are necessary to detect faults and treat them, because if left unattended, it may lead to electrical problems and cause long-term consequences. These problems not only cause energy losses but also lead to electrical outages and losses in expensive equipment, in addition to injuries and accidents such as fire. Therefore, monitoring of substations and their equipment is important to ensure safety, protection, and stability in the electric power networks. Different types of sensors (e.g. temperature and humidity sensors) can be used in the outdoor ultra-high voltage substation (see (b) in Figure 5.3.1-1) to detect the anomaly and trigger predictive maintenance. In addition, various sensors can be used in other use cases in the power transmission and distribution networks (see (c) to (e) in figure F.3.1-1) for remote monitoring and protection purposes.

- Overview

- Outdoor ultra-high voltage substation 
(Possible issues such as poor contact can cause faulty service.)

- Indoor/outdoor shielding cabinet 
(Problems such as poor wire connection can cause short circuit or even fire.)

- Underground transmission and distribution lines
(Possible issues include that the underground cables can be cut or bitten by rats, and can be damaged due to the faulty drainage system.)

- Aerial transmission and distribution lines 
(Possible issues include that the aerial cables can be vibrated or displaced, and difficult to provide power sources for sensors to monitor these parameters.)

Figure 5.3.1-1: Transmission and distribution networks in smart grids

For these use cases, the data acquisition process is typically not latency-critical, but a large number of sensors have to be efficiently connected, especially considering many of these sensors have limited power source and relatively frequent data transmission (every 5-15 minutes) is expected in some cases (e.g., sensor data is collected once per several seconds for critical equipment monitoring and protection). Moreover, lifespans of the field IoT devices are expected to be one decade or longer, which is one of the main differences compared with consumer products. Many production systems are subject to regulatory approvals (e.g., safety certification), changes to a running production system have often to be avoided. Often sensors are deployed in locations that are inaccessible, where physical replacement would be unduly expensive. Research continues to develop efficient communication techniques to meet the requirements, among which Ambient IoT [4] is very promising to enable wireless communication with minimum energy consumption. The Ambient IoT devices typically are battery-less or with limited energy storage capability, and obtain energy from the environment. The communication power consumption of such Ambient IoT devices are expected to be a few hundred μW [78] [79] [80] [81]. Moreover communication service availability with sufficient 5G coverage is important especially for remote monitoring of the critical equipment.

### 5.3.2	Pre-conditions

In this use case GreenGrid has service level agreement with GreenMobile to deploy 5G network to enable the communication of these Ambient IoT devices with the network.  As part of the service level agreement, GreenMobile provides energy efficient communication and management services to GreenGrid including:

- interfacing with GreenGrid’s grid monitoring and management platform;

- ensuring the lifespan of the Ambient IoT devices of 10-15 years;

- providing energy efficient device management for the Ambient IoT devices based on the instructions from the grid monitoring and management platform;

- providing energy efficient operation (e.g. inventory, read, write) the Ambient IoT devices based on the instructions from the grid monitoring and management platform;

- providing sufficient positioning information of the Ambient IoT devices;

- providing energy efficient security mechanisms for the communication between Ambient IoT devices and the network.

GreenGrid has installed wireless sensors, one form of Ambient IoT devices, in the outdoor ultra-high voltage substations of their power transmission and distribution networks to monitor the corresponding parameters to detect malfunctioning and broken elements in the surrounding environment. This environment is typically monitored using various types of sensors such as temperature sensors, humidity sensors, pressure sensors and vibration sensors. Remote monitoring along with a classification of the anomaly can help with predictive maintenance. For example, the sensor data (e.g. temperature, humidity) can be used to detect high-temperature problems (which is a typical indication of poor connection), excessive humidity (which could be due to floods), etc.

### 5.3.3	Service Flows

- The 5G core network receives the request from the application function (in this case GreenGrid’s grid monitoring and management platform) to operate on the Ambient IoT devices in a certain area. The 5G core network starts to operate on these devices accordingly. Once detecting the signals from the 5G network these Ambient IoT devices can respond to the command.

- These Ambient IoT devices send the identification information to the 5G core network and complete the authentication procedure.

- The Ambient IoT devices (wireless sensors) measure the environmental parameters, such as temperature, humidity, pressure and vibration.

For temperature, humidity and pressure measurement, typical sampling rate is 10 Hz with sample size of 32 bits, thus the data generation per Ambient IoT device is about 320 bit/s. For vibration measurement, typical sampling rate is 10 Hz with sample size of 96 bits, thus the data generation per Ambient IoT device is about 960 bit/s.

- The 5G core network, based on the requests issued by the application function, performs operations such as "inventory", "read" and "write" on the Ambient IoT devices correspondingly. "Inventory" operation is to read the Ambient IoT device identifier. "Read" operation is to read sensor data. "Write" operation can be used to configure the Ambient IoT device.

- The 5G core network then sends the results of the operations to the application function. The application function includes diagnostic functions that analyze the sensor data and detect the anomaly and trigger predictive maintenance when necessary. Typically, the sensor data is collected once per several seconds.

- The 5G system is also expected to provide positioning service for these Ambient IoT devices, which can be for device management purpose (e.g., to automatically identify the locations of the Ambient IoT devices) or for security reason (e.g., to make sure the devices have not been removed). The positioning for these purposes is not expected to be performed frequently.

### 5.3.4	Post-conditions

The 5G system enables efficient communication, with enhanced security and tens of meter-level positioning accuracy, for the Ambient IoT devices installed in the power transmission and distribution networks for remote monitoring and protection purposes.

### 5.3.5	Existing features partly or fully covering the use case functionality

Service requirements for MTC (Machine-Type Communications) have been captured in TS 22.368 [6] since release 10, which specifies the service requirements for network improvements. In addition to the common service requirements, specific service requirements have also been defined corresponding to the following MTC Features:

-	Low Mobility;

-	Time Controlled;

-	Small Data Transmissions;

-	Infrequent Mobile Terminated;

-	MTC Monitoring;

-	Secure Connection;

-	Group Based MTC Features:

-     Group Based Policing;

-    Group Based Addressing.

Resource efficiency is one of the key service requirements for IoT, for which there are a few requirements specified in 3GPP TS 22.278 [7] and 3GPP TS 22.261 [8]. The target IoT scenarios have been described as

As sensors and monitoring UEs are deployed more extensively, the need to support UEs that send data packages ranging in size from a small status update in a few bits to streaming video increases. A similar need exists for smart phones with widely varying amounts of data. Specifically, to support short data bursts, the network should be able to operate in a mode where there is no need for a lengthy and high overhead signalling procedure before and after small amounts of data are sent. The system will, as a result, avoid both a negative impact to battery life for the UE and wasting signalling resources.

The related service requirements are kept at a relatively general level, e.g.

The 5G system shall minimize control and user plane resource usage for data transfer from send only UEs.

The 5G system shall minimize control and user plane resource usage for stationary UEs (e.g. lower signalling to user data resource usage ratio).

The 5G system shall minimize control and user plane resource usage for transfer of infrequent small data units.

The 5G system shall optimize the resource use of the control plane and/or user plane for transfer of small data units.

Additional consideration needs to be given in support of Ambient IoT devices that are battery-less or with limited energy storage capability, which present new challenges to the 5G system.

### 5.3.6	Potential New Requirements needed to support the use case

[PR.5.3.6-001] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the overall and peak device communication power consumption) for Ambient IoT devices.

[PR 5.3.6-002] The 5G system shall be able to support energy efficient security mechanisms for Ambient IoT devices, including authentication, encryption and data integrity.

[PR 5.3.6-003] The 5G system shall support a mechanism to interface a 3rd party application to manage and operate on the Ambient IoT devices.

[PR.5.3.6-004] The 5G system shall be able to collect charging information for using Ambient IoT services on per Ambient IoT device basis (e.g., total number of communications per charging period).

[PR 5.3.6-005] The 5G system shall be able to collect charging information per application for using Ambient IoT services (e.g., total number of Ambient IoT devices per charging period).

[PR 5.3.6-006] The 5G system shall provide the network connection to address the KPIs for the use of Ambient IoT devices in substations in smart grids, see table 5.3.6-1.

Table 5.3.6-1: Potential key performance requirements for the use of Ambient IoT devices in substations in smart grids

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote monitoring of transmission and distribution networks in smart grids | 1 s (note 4) | 99% | NA | < 1kbit/s (note 5) | Typically < 100 bytes (note 1) | < 10,000 /km2 (note 3) | Outdoor: typically 50-200 meters | [several km2 up to 100 000 km2] (note 2) | Stationary | 5-15 min | NA | NA | several 10 m |
| NOTE 1: Electronic Product Code standard [5], this size is the payload size. NOTE 2: The service are refers to the overall size of transmission and distribution networks. Typically, the size of the individual substations varies from 100m x 200m to 500m x 600m. NOTE 3: The device density is calculated based on an individual substation, where typically several hundreds of Ambient IoT devices are required to monitor the environmental parameters. NOTE 4: This is calculated based on assumption that the sensor data are collected once per several seconds. NOTE 5: For temperature, humidity and pressure measurement, typical sampling rate is 10 Hz with sample size of 32 bits, thus the data generation per Ambient IoT device is about 320 bit/s. For vibration measurement, typical sampling rate is 10 Hz with sample size of 96 bits, thus the data generation per Ambient IoT device is about 960 bit/s. |  |  |  |  |  |  |  |  |  |  |  |  |  |