3GPP TSG SA WG 1 Meeting #101 	S1-230620

Athens, Greece, 20 - 24 February 2023	(revision of S1-230309)

Source:	KPN

pCR Title:	Pseudo-CR to clarify terminology in clause 5.28

Draft Spec:	3GPP TR 22.840v1.0.0.

Agenda item:	7.2

Document for:	Approval

Contact:	toon .norp @ tno. nl

Abstract: <provide a short description of the content>

1. Introduction

This contribution addresses an editor’s note in clause 5.28.

2. Reason for Change

The terminology of transaction was not so clear. In ICT a transaction generally implies a sequence of interactions (e.g. at least a request and then a response) to achieve something. With Ambient IoT, there may not even be enough power to receive or send a response and hence it is better to talk about interactions. Such interactions can be control or user data transmissions. To clarify the terminology of interactions in this specific context a note is added.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840v1.0.0.

* * * First Change * * * *

## 5.28	Use case on pressure powered switch

### 5.28.1	Description

A pressure powered switch can harvest energy from the kinetic energy of the pushing on the switch. Existing non-3GPP products can e.g. connect wirelessly to a controller over approximately 25 m indoor and 150 m outdoor. These products do not adhere to 3GPP protocols for signalling and user communication.

Figure 5.28.1-1: A pressure powered switch

The registration procedure for 5G according to TS 23.502 [77] shows 25 different steps which 8 involve the UE. Several of these steps involve multiple interactions. This registration procedure does not even include the actual data communication. Compared to the minimal information transfer requirement for a button switch (only its ID), it is clear that the registration involves far more information than the actual data transport. Furthermore, a registration procedure will likely take longer than the time the switch has power available. This runs the risk that the switch can start a registration (or other) signaling procedure, but cannot successfully complete it.

The purpose of the use case is to propose a requirement to optimize Ambient IoT signaling to reduce the amount of signaling interactions and thus save power and time.

Note:	The pressure powered switch is an example for other Ambient IoT devices that only have a very limited amount of energy for a very short amount of time (e.g. door/window sensors, vibration sensors).

### 5.28.2	Pre-conditions

None

### 5.28.3	Service Flows

1. The switch is being pushed.

2. The switch harvests the energy from the push, wakes up and communicates to the network.

3. Through optimized protocols only a minimal number of signalling interactions is needed. This allows the switch to complete the signalling procedure. As part of the signalling, the switch also transmits its identity.

4. The network transfers the identity information from the switch to an application server, which e.g. determines that a light needs to be turned on.

### 5.28.4	Post-conditions

The light is successfully switched on.

### 5.28.5	Existing features partly or fully covering the use case functionality

Existing signaling procedures and data transfer procedures for 5G are specified in TS 23.502 [77]. It is clear that these procedures are not optimized for a minimal number of signaling interactions. This runs the risk that an Ambient IoT device cannot complete these procedures before running out of power.

### 5.28.6	Potential New Requirements needed to support the use case

[P.R.5.1.6-001] The 5G system shall support more efficient procedures for Ambient IoT control and user data transmission; to use a reduced number of interactions between the network and the Ambient IoT device compared to earlier 3GPP technologies.

NOTE:	In this context each interaction is a single instance of control or user data transmission to or from the Ambient IoT device