3GPP TSG-SA WG1 Meeting #101 	S1-230163

Athens, Greece, 20 – 24 Jan., 2023

Source:	OPPO, vivo

pCR Title:	Consolidation on Functional Requirement of Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	xuyang@oppo.com, xuweijie@oppo.com

Abstract: it is proposed to put the functional requirements into 6 subclauses. Then make CPR mapping table for consolidation under each subclause.

Discussion

Based on all functional requirements (see Annex below) captured in the latest version of TR 22.840v0.3.0, it is proposed to put them into 6 subclauses as below for Consolidated Potential Requirements.

- Subclause 1: Communication aspects of Ambient IoT devices

- Subclause 2: Positioning/location of Ambient IoT devices

- Subclause 3: Management of Ambient IoT devices

- Subclause 4: Collected information and network capability exposure

- Subclause 5: Charging

- Subclause 6: Security and privacy

A PR/CPR mapping table is proposed under each subclause. The mapping table is described in the clause 7.1 as change proposal except the following three Potential Requirements which have critical Editor’s Note to be resolved.

[PR 5.2.6-003] The 5G system shall be able to provide a mechanism to collect the information stored in an Ambient-IoT device.

Editor’s Note: This requirement need to be revisited to clarify how the device can store information without application and what kind of data that can be stored.

[PR 5.5.6-002] The 5G system shall support collection of charging information based on different charging policies for Ambient-IoT type of communication, i.e., total number of communication (e.g. data payload) per charging period, or total number of Ambient IoT devices per charging period.

Editor’s Note: 	Additional requirements for this use case are FFS.

[PR 5.11.6-002] The 5G system shall be able to provide a mechanism for a trusted 3rd party to modify the information stored on an Ambient IoT device.

Editor’s Note: This requirement need to be revisited to clarify how the device can store and modify information without application and what kind of data that can be stored.

[PR.5.15.6-001] The 5G system shall support communication for Ambient_IoT devices.

Editor’s Note:  	It is FFS enhancement of specific 5GS efficiency required to support this use case.

[PR 5.19.6-001] The 5G system shall support Fault-Tolerant Reliable Ambient IoT communication.

Editor’s Note: Requirement PR5.2.6-001 is for FFS.

[PR.5.27-002] The 5G system shall support functionality to trigger Ambient IoT devices to communicate.

Editor’s Note: terminology of triggering to be checked with existing MTC triggering terminology. Definitions still to be provided in the definition section

Subclause A: Communication aspects of Ambient IoT devices

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [PR 5.2.6-001] The 5G system shall be able to communicate with an Ambient-IoT device [PR.5.6.6-001] The 5G system shall be able to support communication services for ambient power-enabled IoT devices. [PR 5.11.6-001] The 5G system shall be able to communicate with an Ambient-IoT device. [PR.5.12.6-002] The 5G system shall be able to collect information from a specific Ambient IoT device. [PR.5.13.6-001] The 5G system shall be able to support communication with Ambient IoT devices. [PR.5.1.6-001] The 5G system shall be able to support communication with Ambient IoT device which is battery-less or with limited energy storage (e.g., capacitor). [PR.5.4.6-001] 5G system shall support network access for Ambient IoT devices while considering the constraint power consumption. [PR.5.20.6-001] The 5G system shall support communication with Ambient IoT device with 3rd party application server. | In these PRs, it clearly mentioned the communication with Ambient IoT devices. Meanwhile, some PRs mentioned the additional features for the communication. - the communication with Ambient IoT device is to collect information stored or from IoT devices. And - the Ambient IoT device is with battery-less or limited energy storage - The information stored in Ambient IoT device could be device ID Proposal: incorporate the PRs into CPR 7.1.1-1 | [CPR 7.1.1-1] The 5G system shall support energy-efficient communication mechanisms for an Ambient IoT device, for the information (e.g. device ID, sensor data) stored in the Ambient IoT device to be reported to or collected by a trusted 3rd party. |
| [PR.5.13.6-004] The 5G system shall support energy efficient communication mechanisms for Ambient IoT devices (i.e., minimizing the device communication power consumption). [PR.5.3.6-001] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the overall and peak device communication power consumption) for Ambient IoT devices [PR 5.22.6-1, PR5.23.6-1, PR.5.24.6-1] The 5G system shall support energy efficient communication mechanisms (i.e. minimizing the device communication power consumption) for Ambient IoT devices, while meeting the communication performance requirements. [PR.5.26.6-001] The 5G system shall be able to support mechanisms to communicate efficiently with Ambient IoT devices. [PR.5.2.6-002] The 5G system shall be able to provide group communication for a group of Ambient-IoT devices | The PRs mentioned the energy efficient communication and further described by means of minimizing the power consumption of Ambient IoT. It can be merged as a CPR. Proposal: incoperate these PRs into CPR 7.1.1-2 | [CPR 7.1.1-2] The 5G system shall minimize the overall and peak device communication power consumption for Ambient IoT devices. |
| [PR.5.12.6-002] The 5G system shall be able to collect information from a specific Ambient IoT device. [PR 5.8.6-002] Based on operator policy, the 5G system shall be able to support authorization of UEs communicating with an Ambient IoT device. | Two PRs mentioned the communication can be per specific or a group of Ambient IoT device(s). This is related to whether and how the 5GS need to indicate which Ambinet IoT will responds to the 5GS. So it can be a separate CPR. Proposal: incorporate these PRs into CPR 7.1.1-3, CPR 7.1.1-4 | [CPR 7.1.1-3] Based on authorized 3rd party’s request, the 5G system shall support to communication with a specific Ambient IoT device [CPR 7.1.1-4] Based on authorized 3rd party’s request, the 5G system shall support to communication with a group of Ambient IoT devices. Comments: one more req for the group. |
| [PR.5.12.6-001] The 5G system shall support to authorize a UE to obtain device information of an Ambient IoT device. [PR.5.14.6-001] The 5G system shall support be able to authorize a UE to perform Ambient IoT communication services with specific Ambient IoT devices [PR.5.21.6-003] The 5G system shall support to authorize a UE to obtain device identity information of an Ambient IoT device [PR.5.21.6-001] The 5G system shall support to authorize a UE to perform Ambient IoT communication services with specific Ambient IoT devices. | These PRs mentions authorization for a UE for communication with specific Ambient IoT device. Proposal: Incorperate these PRs into CPR 7.1.1-4 | [CPR 7.1.1-5] The 5G network shall support a mechanism for the network operator to authorize a UE to communicate with specific Ambient IoT devices. |
| [PR.5.8.6-003] The 5G system shall be able to support means to support RAN entities and authorized UEs to communicate with Ambient IoT devices and transfer related information to other 5G system entities (e.g., core network) / servers. [PR.5.27-004] The 5G system shall support relaying communication from an Ambient IoT device to the network. | This PR mentions the RAN entities and authorized UE communicate with Ambient IoT devices and transfer the information to other 5G system entities. The transfer of AIoT related data to 5GS entities is interpreted as a kind of relay as mentioned in PR 5.27.-004. Proposal: These PRsare adopted to CPR 7.1.1-5. Since the relay is commonly used in ProSe where the relay is simply relay data from remote UE to network while here the data transferred by UE may be further processed. The term “relay” is not used in the CPR SZ: [PR.5.27-004] refers to normal forwarding of info from UE to e.g. a server via user plane connectivity. This is OTT. | [CPR 7.1.1-6] The 5G network shall be able to support RAN entities to transfer related information (i.e. collected from Ambient IoT device) to other 5G network entities (e.g. core network). [CPR 7.1.1-6] The 5G network shall provide a suitable means for 5G network to authorize a UE to transfer related information (i.e. collected from Ambient IoT device) via RAN node to other 5G network entities (e.g. core network). |
| [PR.5.8.6-001] The 5G system shall be able to assist an Ambient IoT device with discovery and communication with 5GS entities that can provide location related information. | This PR mentions the discovery for communication with Ambient IoT devices. Proposal: PR 5.8.6-001 is adopted as CPR-7.1.1-7 | [CPR 7.1.1-7] The 5G network shall be able to assist an Ambient IoT device with discovery and communication with 5GS entities that can provide location related information. |
| [PR 5.9.6-001] 5G system shall optimize mobility management support for mobile Ambient-enabled IoT devices that are unable to constantly stay active. [PR 5.16.6-002] The 5G system shall optimize mobility management support for non-stationary Ambient IoT devices that are unable to initiate communication towards the network. | These two PRs mentions the mobility management for the Ambinet IoT devices which are non-stationary and unable to initiate communication to network. Proposal: Incoperate the PRs into CPR 7.1.1-8 | [CPR 7.1.1-8] The 5G system shall support mobility management for Ambient IoT devices which are not stationary and unable to initiate communication towards the network. |
| [P.R.5.1.6-001] The 5G system shall support procedures for Ambient IoT control and user data transmission; to use a minimum number of transactions with the Ambient IoT device. Editor’s Note: terminology of ‘transactions’ is FFS |  | [CPR 7.1.1-8] The 5G system shall support procedures for Ambient IoT control and user data transmission; to use a minimum number of interactions with the Ambient IoT device. NOTE: In this context an interaction implies a single instance of control or user data transmission to or from the Ambient IoT device |

Subclause B: Positioning/location of Ambient IoT devices

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [PR.5.7.6-001] The 5G system shall be able to support means to discover and locate Ambient-IoT devices in a certain geographical area, e.g. at cell level [PR 5.9.6-002] 5G system shall be able to determine the location of Ambient IoT device, when it becomes active as triggered by the 5G network. [PR 5.12.6-004] The 5G system shall be able to support indoor and outdoor positioning for Ambient IoT devices. [PR 5.2.6-005] The 5G system shall be able to support positioning for an Ambient-IoT device. | The two PRs mentions the 5G system to discover and locate the Ambient IoT devices. These two PRs further clarifies the type of positioning i.e. ranging, indoor/outdoor positioning. Proposal: incorporate these PRs into CPR 7.1.2-1 with some rewording | [CPR 7.1.2-1] The 5G system shall be able to provide positioning services for Ambient IoT devices in a certain geographical area. |
| [PR 5.10.6-001] 5G system shall be able to support the ranging operation between UE and Ambient IoT device. [PR.5.21.6-002] The 5G system shall be able to support to authorize a UE to perform Ambient IoT ranging services with specific Ambient IoT devices |  | Ranging related CPR is TBD NOTE: to be defined later? |
| [PR.5.14.6.1-002] The 5G system shall be able to support authorizing a UE to perform Ambient IoT positioning services with specific Ambient IoT devices. | The PRs are about authorzing a UE for positiong service to an Ambnient IoT device. Proposal: incorperate these PRs into CPR 7.1.2-3 Shuang: [PR.5.14.6.1-002], it is the relative positioning using UE, the positioning service and performance is not controlled or guaranteed by the 5G network. Because of this, this PR is not relevant as long as authorization by operator is concerned. | [CPR 7.1.2-3] The 5G system shall support authorizing a UE to perform Ambient IoT positioning services with specific Ambient IoT devices NOTE: how to distinguish the intention of this requirement from LCS based positioning? |

Subclause C Management of Ambient IoT devices

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [P.R.5.1.6-005] The 5G system shall be able to manage (e.g. provide service parameters, activate, deactivate) multiple Ambient IoT devices in bulk. [PR 5.16.6-003] The 5G System shall allow an operator to manage (e.g. provision, authenticate, authorise, etc.) Ambient_IoT devices that have limited or no power source. | The PRs mention how to manage the Ambient IoT devices. Proposal: Incorperate the PRs into CPR 7.1.3-1 NOTE: the “Authenticate” and “authorize” belongs to subclause 7.1.1 and 7.1.6 respectively. The last part of PR 5.16.6-003 has been covered in Subclause 7.1.1. | [CPR 7.1.3-1] The 5G network shall support a 3GPP-supported mechanism for a mobile network operator to manage one or multiple Ambient IoT devices, e.g. provision, activate, deactivate Ambient IoT devices. |
| [PR 5.22.6-2, PR 5.23.6-2, PR 5.24.6-2] The 5G system shall provide a mechanism for a 3rd party application to write user data to and to read user data from an Ambient IoT device. | The PR proposes to let 5GS to write the information/data to the Ambient IoT device Note: reading data from an Ambient IoT device in PR 5.22.6-2 has been captured in CPR-7.1.1-1 in subclause A Proposal: adopt the “write” requirement into CPR-7.1.3-2 | [CPR 7.1.3-2] The 5G system shall provide a mechanism for a 3rd party application to write user data to an Ambient IoT device. (TBD) NOTE: what does “write” exactly mean? |
| [PR.5.26.6-002] When setting up communication to an Ambient IoT device the 5G system shall be able to handle the unavailability of Ambient IoT devices either due to lack of power or due to power saving mechanisms of the Ambient IoT device. | Handling of unavailable Ambient IoT devices should be addressed under management of AIoT clause Proposal: adopt the requirement into CPR 7.1.3-3 | [CPR 7.1.3-3] When setting up communication to an Ambient IoT device the 5G system shall be able tocommunicate with Ambient IoT devices either in the condition of lack of power or due to power saving mechanisms of the Ambient IoT device. NOTE: What does “handle” exactly mean? |
| [PR.5.27-001] The 5G system shall be able to support an Ambient IoT device to function in different countries in accordance with local regulations | This should be a kind of management of AIoT device Proposal: adopt the requirement into CPR 7.1.3-4 | (TBD, proposed to be deleted?) |

Subclause D: “Collected information” and network capability exposure

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [PR.5.1.6-002] The 5G system shall support to provide collected information from Ambient IoT devices to the trusted 3rd party. [PR.5.12.6-003] The 5G system shall be able to provide information of a specific Ambient IoT device to the trusted 3rd party. NOTE: The request from 3rd party can include the requested Ambient IoT device identity, the requested service area to find the Ambient IoT device, the requested information of an Ambient IoT device includes position information. [PR 5.13.6-005] The 5G system shall support transferring data collected from Ambient IoT devices to a trusted 3rd party. [PR.5.14.6-003] Subject to user consent and operator’s policy, the 5G system shall be able to expose the identities and positions of Ambient IoT devices to a 3rd party. [PR 5.16.6-004] The 5G System shall be able to provide suitable and secure means to report to an authorized third-party the location of Ambient_IoT devices. [PR 5.18.6-1] Based on operator policy, the 5G system shall provide means for an authorised third party to poll a group of multiple ambient IoT devices. [PR.5.21.6-004] The 5G system shall be able to expose the identities and ranging results of Ambient IoT devices to a trusted third party. [PR 5.2.6-004] The 5G system shall be able to provide a mechanism to expose the information collected from an Ambient-IoT device to a trusted 3rd party. | These PRs are about exposing the information of Ambient IoT devices to a 3rd party. Furthermore, the PR.5.14.6.1-003 and a NOTE in P.R.5.12.6-003 mentions the specific informations which hcould be provided to 3rd party. The following features are abstracted from the PRs in lest column: 3rd party isTrsuted, authorized Requested condition includes device ID, service area, Requested information includes: device id and position result, ranging result Taget: single AIoT device or a group of devices Proposal: Put these PRs into CPR 7.1.4-1a, b, c capturing the features above. NOTE: The Ranging result will be discussed later when the relevant requirement in is fixedsublause B | [CPR 7.1.4-1a] Based on operator policy, the 5G network shall provide suitable APIs to allow a trusted and authorized 3rd party to request information of Ambient IoT devices. The requested information may include Ambient IoT device identity, and /or requested service area. NOTE: Further separation of the CPR needs to be discussed (e.g. exposre and poll, Subject to user consent, trusted 3rd party, device id and location). NOTE: how to describe “exposure” with SA1 language (e.g. provide API…) NOTE: Check if all things in PRs are covered |
| [PR 5.3.6-003] The 5G system shall support a mechanism to interface a 3rd party application to manage and operate on the Ambient IoT devices. [PR 5.19.6-001] The 5G system shall support on-demand access to/from Ambient IoT device. | This PR is about exposing some capability (manage and operate on Ambient IoT device) to the 3rd party. PR 5.19.6-001 has a further elaboration about how the manage and operate could be. Proposal: adopt the the PRs as CPR 7.1.4-2 | [CPR 7.1.4-2] The 5G system shall support a mechanism to interface a 3rd party application to manage and operate on the Ambient IoT devices (e.g. on-demand access to/from Ambient IoT device). |
| [PR 5.17.6-1] Based on operator policy, the 5G system shall provide means for an authorised user or authorised third parties to request enable and disable an Ambient IoT device capability to transmit RF signals. | This PR is about exposing the capability of disabling/enabling an Ambient IoT device. Proposal: Adopt the PR as CPR 7.1.4-3 | [CPR 7.1.4-3] Based on operator policy, the 5G system shall support an authorized user or authorized 3rd party to request to enable and disable an Ambient IoT device, e.g. the capability to transmit RF signals. |
| [PR.5.27-003] The 5G network shall be able to trigger Ambient IoT devices. The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area, which group of Ambient IoT devices needs to be triggered and which action the Ambient IoT devices need to perform when triggered (e.g. send ID, receive further information, send measurement value). | The PR raises some assistant information from 3rd party which is useful for realizing “on-demand” exposure. Proposal: Adopt the PR as CPR 7.1.4-4 with minor resording | [CPR 7.1.4-4] The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area, which group of Ambient IoT devices needs to be triggered for communication and which action the Ambient IoT devices need to perform communication when triggered (e.g. send ID, receive further information, send measurement value). |

Subclause E: Charging

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [PR.5.3.6-004] The 5G system shall be able to collect charging information for using Ambient IoT services on per Ambient IoT device basis (e.g., total number of communications per charging period). [PR 5.3.6-005] The 5G system shall be able to collect charging information per application for using Ambient IoT services (e.g., total number of Ambient IoT devices per charging period). [PR 5.22.6-3, PR 5.23.6-3, PR 5.24.6-3] The 5G system shall be able to collect charging information for a large group of closely located Ambient IoT devices in an efficient way NOTE: for example, the efficiency could be reduced total number of charging data related to a group of Ambient IoT devices, the reduction is compared with already specified 3GPP technologies. | These PRs mention the similar requirement about charging, considering the following aspects: - total number of communications - total number of Ambient IoT devices; - Payload - per carging period - a group of Ambient IoT devices for a specific location Proposal: incorporate these PRs into CPR 7.1.5-1 | [CPR 7.1.5-1] The 5G system shall support to collect charging information for a large group of closely located Ambient IoT devices in an efficient way, the charging policies being based on e.g. number of Ambient IoT devices, number of communications, data payload and per charging period. NOTE: for example, the efficiency could be reduced total number of charging data related to a group of Ambient IoT devices, the reduction is compared with already specified 3GPP technologies. |

Subclause F: Security and privacy

| Potential Requirement | Clean-up Steps Taken | Consolidated Potential Requirement |
| --- | --- | --- |
| [P.R.5.1.6-003] The 5G system shall support suitable security mechanisms for Ambient_IoT devices, including encryption and data integrity. [PR 5.3.6-002] The 5G system shall be able to support energy efficient security mechanisms for Ambient IoT devices, including authentication, encryption and data integrity. [PR.5.6-002] The 5G system shall support suitable security mechanisms for Ambient IoT devices, including authentication, encryption and data integrity. [P.R.5.13.6-002] The 5G system shall be able to support suitable security mechanisms for Ambient IoT devices, including encryption and data integrity. [PR.5.21.6-005] The 5G system shall be able to support suitable security mechanisms for Ambient IoT devices, including encryption and data integrity. | All the PRs are about encryption and data integrity for communication with Ambient IoT device Proposal: Incorperate the PRs into CPR 7.1.6-1 | [CPR 7.1.6-1] The 5G system shall support energy-efficient security mechanisms for communication with Ambient_IoT devices, e.g. encryption and data integrity. |
| [PR. 5.14.6-004] The 5G system shall be able to support an Ambient IoT device to authenticate a UE triggering Ambient IoT services. | This PR is about authenticating a UE for performing Ambient IoT services. Proposal: Adopt these PRs to CPR 7.1.6-2 | [CPR 7.1.6-2] The 5G system shall support to authenticate a UE triggering Ambient IoT services; |
| [P.R.5.1.6-004] The 5G system shall be able to support the authentication and authorization mechanisms of Ambient_IoT devices. [PR 5.8.6-005] The 5G system shall be able to support a UE to authenticate an Ambient IoT device [PR. 5.12.6-006] The 5G system shall support to validate an Ambient IoT device [P.R.5.13.6-003] The 5G system shall be able to support suitable mechanisms to authenticate and authorize Ambient IoT devices. [PR. 5.14.6-005] The 5G system shall be able to support a UE to verify an Ambient IoT device’s identity. [PR 5.16.6-001] The 5G system shall be able to support a mechanism to authenticate and authorize Ambient_IoT devices. [PR.5.20.6-002] The 5G system shall be able to authenticate an Ambient IoT device. | The PRs are mainly about how a UE can authenticate, validate or verify a Ambient IoT. It seems people may have some different understanding for these three words, it just keeps the three word in the CPRs for further comments. Proposal: Incorporate these PRs into CPR 7.1.6-3 NOTE 1: Further comments may be needed to decide whether and which “authenticate”, “validate” and “verify” should be used for the CPR. NOTE 2: The “authorize” in some PRs have been addressed by subclause 7.1.1 | [CPR 7.1.6-3] The 5G system shall support a mechanism to authenticate an Ambient IoT device; |
| [PR. 5.12.6-005] The 5G system shall be able to support an Ambient IoT device to validate a UE which communicates with the device. | This PR is to let Ambient IoT device to validate the UE which communicate with the device. Proposal: Adopt the PR 5.12.6.1-005 to CPR 7.1.6-4 with a bit rewording. | [CPR 7.1.6-4] The 5G system shall support an Ambient IoT device to validate UE/RAN node which communicates with an Ambient IoT device; |
| [PR.5.8.6-004] The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device. NOTE 1: This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs. | This PR is addressing the privacy for communication with Ambient IoT. Proposal: adopt the PR to CPR 7.1.6-5 | [CPR 7.1.6-5] The 5G system shall support to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device. NOTE 1: This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs. |

****************Start of Change *************

7	Consolidated potential requirements and KPIs

7.1	Consolidated potential requirements

Mapping table is used per each subclause for consolidated potential requirements.

7.1.1	Communication aspects of Ambient IoT devices

Table 7.1.1 – [Communication aspects of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.1-1 | The 5G system shall support communication with an Ambient IoT device to collect the information (e.g. device ID, sensor data) stored in the Ambient IoT device. | PR 5.2.6-001, PR 5.2.6-003, PR 5.6-001, PR 5.4.6-001, PR 5.11.6-001, PR.5.13.6-001, PR 5.15.6-001, PR 5.2.6-003, PR 5.5.6-001, PR 5.4.6-001, PR.5.20.6-001, PR.5.21.6-003 | In these PRs, it clearly mentioned the communication with Ambient IoT devices. Meanwhile, some PRs mentioned the additional features for the communication. - the communication with Ambient IoT device is to collect information stored or from IoT devices (PR 5.2.6-3, P.R.5.12.6.1-002). And - the Ambient IoT device is with battery-less or limited energy storage (PR.5.15.6.2-1, P.R.5.1.6-001, PR.5.4.6-001) - The information stored in Ambient IoT device could be device ID |
| CPR 7.1.1-2 | The 5G system shall minimize the device communication power consumption for Ambient IoT devices. | PR 5.3.6-001, PR 5.13.6-004, PR 5.22.6-001, PR.5.26.6-001 | The PRs mentioned the energy efficient communication and further described by means of minimizing the power consumption of Ambient IoT. |
| CPR 7.1.1-3 | Based on authorized 3rd party’s request, the 5G system shall support communication with a specific Ambient IoT device or a group of Ambient IoT devices. Comments: one more req for the group. | PR 5.2.6-002, PR 5.12.6-002 | The PRs mentioned the communication can be per specific or a group of Ambient IoT device(s). This is related to whether and how the 5GS need to indicate which Ambinet IoT will responds to the 5GS. |
| CPR 7.1.1-4 | Based on authorized 3rd party’s request, the 5G system shall support communication with a group of Ambient IoT devices. |  |  |
| CPR 7.1.1-5 | The 5G network shall be able to authorize an UE to communicate with specific Ambient IoT devices. | PR 5.8.6-002, PR 5.12.6-001, PR 5.14.6-001, PR.5.21.6-001 | These PRs are about authorization for a UE for communication with specific Ambient IoT device. |
| CPR 7.1.1-6 | The 5G network shall be able to \authorize a UE to transfer the Ambient IoT related information to other 5G system entities (e.g. core network) . | PR 5.8.6-003 PR 5.27-004 | This PR mentions the RAN entities and authorized UE communicate with Ambient IoT devices and transfer the information to other 5G system entities. The transfer of AIoT related data to 5GS entities is interpreted as a kind of relay as mentioned in PR 5.27.-004. |
| CPR 7.1.1-7 | The 5G network shall be able to assist an Ambient IoT device with discovery and communication with UEs/RAN entities. | PR 5.8.6-001 | This PR mentions the discovery for communication with Ambient IoT devices. |
| CPR 7.1.4-8 | The 5G system shall optimize mobility management for Ambient IoT devices which are not stationary and unable to initiate communication towards the network | PR 5.9.6-001 PR 5.16.6-002 | These PRs mentions the mobility management for the Ambinet IoT devices which are non-stationary and unable to initiate communication to network |

7.1.2	Positioning/location of Ambient IoT devices

Table 7.1.2 – [Positioning/location of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.2-1 | The 5G system shall support means to discover and position an Ambient IoT device in a certain geographical area | PR 5.7.6-001, PR 5.9.6-002 | The PRs mentions the 5G system to discover and locate the Ambient IoT devices |
| CPR 7.1.2-2 | The 5G system shall support positioning for an Ambient IoT device. The positioning includes indoor and outdoor positioning | PR 5.10.6-001, PR 5.12.6-004 PR 5.2.6-005 | These PRs further clarifies the type of positioning i.e. ranging, indoor/outdoor positioning |
| CPR 7.1.2-3 | The 5G system shall support authorizing a UE to perform Ambient IoT positioning services with specific Ambient IoT devices | PR 5.14.6-002, PR.5.21.6-002 | The PRs are about authorizing a UE for positiong service to an Ambnient IoT device |

7.1.3	Management of Ambient IoT devices

Table 7.1.3  [Management of Ambient IoT devices] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.4-1 | The 5G system shall support to manage Ambient IoT devices, including provisioning parameters, activate, deactivate an Ambient IoT devices. | PR 5.1.6-005 and PR 5.16.6-003 | The PRs mentions how to manage the Ambient IoT devices |
| CPR 7.1.4-2 | The 5G system shall provide a mechanism for a 3rd party application to write user data to an Ambient IoT device. | PR 5.22.6-002, PR 5.23.6-002, PR 5.24.6-002 | The PR proposes to let 5GS to write the information/data to the Ambient IoT device (Note: reading data from an Ambient IoT device in PR 5.22.6-2 has been captured in CPR-A1 in subclause 7.1.1) |
| CPR 7.1.4-3 | When setting up communication to an Ambient IoT device the 5G system shall be able to handle the unavailability of Ambient IoT devices either due to lack of power or due to power saving mechanisms of the Ambient IoT device. | PR.5.26.6-002 | Handling of unavailable Ambient IoT devices should be addressed under management of AIoT clause |
| CPR 7.1.4-4 | The 5G system shall be able to support an Ambient IoT device to function in different countries in accordance with local regulations | PR.5.27-001 | This should be a kind of management of Ambient IoT device |

7.1.4	“Collected information” and network capability exposure

Table 7.1.4  [“Collected information” and network capability exposure] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.3-1 | The 5G system shall support to provide a suitable and secure means to expose the collected information of one or a group of Ambient IoT devices to a 3rd party. The collected information includes the requested Ambient IoT device identity, positioning result. | PR 5.1.6-002, PR 5.12.6-003, PR 5.13.6-005, PR 5.14.6-003, PR 5.16.6-004 PR 5.18.6-001 PR 5.2.6-004 | These PRs are about exposing the information of Ambient IoT devices to a 3rd party. Furthermore, the PR.5.14.6.1-003 and a NOTE in P.R.5.12.6.1-003 mentions the specific informations which hcould be provided to 3rd party. Meanwhile, PR 5.18.6-1 mentioned the poll “a group of” Ambient IoT devices, it assumes “poll” is technically the same meaning of data collection. |
| CPR 7.1.3-2 | The 5G system shall support a mechanism to interface a 3rd party application to manage and operate on the Ambient IoT devices (e.g. on-demand access to/from Ambient IoT device) | PR 5.3.6-003, PR 5.19.6-001 | This PR 5.3.6.2-3 is about exposing some capability (manage and operate on Ambient IoT device) to the 3rd party. PR 5.19.6-001 has a further elaboration about how the manage and operate could be |
| CPR 7.1.3-3 | The 5G system shall support an authorized user or authorized 3rd party to enable and disable an Ambient IoT device capability to transmit RF signals | PR 5.17.6-001 | This PR is about exposing the capability of disabling/enabling an Ambient IoT device |
| CPR 7.1.3-4 | The 5G system shall enable an authorized 3rd party to instruct the 5G network in which area, which group of Ambient IoT devices needs to be triggered for communication and which action the Ambient IoT devices need to perform communication when triggered (e.g. send ID, receive further information, send measurement value) | PR.5.27-003 | The PR raises some assistant information from 3rd party which is useful for realizing “on-demand” exposur |

7.1.5	Charging

Table 7.1.5  [Charging] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.5-1 | The 5G system shall support to collect charging information for using Ambient IoT services, based on different charging policies for Ambient-IoT type of communication, e.g. per charging period the number of Ambient IoT devices, number of communications or data payload, a group of Ambient IoT devices for a specific location. | PR 5.3.6-004, PR 5.3.6-005 PR 5.5.6-002, PR 5.22.6-003 | These PRs mention the similar requirement about charging, considering the following aspects: - total number of communications - total number of Ambient IoT devices; - Payload - per carging period - a group of Ambient IoT devices for a specific location |

7.1.6	Security and privacy

Table 7.1.6  [Security and privacy] Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 7.1.6-1 | The 5G system shall support suitable security mechanisms for communication with Ambient_IoT devices with data encryption and integrity | PR 5.1.6-003, PR 5.3.6-002, PR 5.6.-002, PR 5.13.6-002, PR. 5.21.6-005 | All the PRs are about encryption and data integrity for communication with Ambient IoT device |
| CPR 7.1.6-2 | The 5G system shall support to authenticate a UE triggering Ambient IoT services | PR 5.14.6-004 | This PR is about authenticating a UE for performing Ambient IoT services |
| CPR 7.1.6-3 | The 5G system shall support a mechanism to authenticate, validate, or verify an Ambient IoT device | PR 5.1.6-004, PR 5.8.6-005, PR 5.12.6-006, PR 5.13.6-003, PR 5.14.6-005, PR 5.16.6-001, PR.5.20.6-002 | The PRs are mainly about how a UE can authenticate, validate or verify a Ambient IoT. NOTE: It seems people may have some different understanding for these three words, hence the CPR keeps all the three word for further comments |
| CPR 7.1.6-4 | The 5G system shall support an Ambient IoT device to validate UE/RAN node which communicates with an Ambient IoT device | PR 5.12.6-005 | This PR is to let Ambient IoT device to validate the UE which communicate with the device |
| CPR 7.1.6-5 | The 5G system shall support to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device. NOTE: This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs | PR 5.8.6-004 | This PR is addressing the privacy for communication with Ambient IoT |

*******************End of Change*****************