3GPP TSG SA WG 1 Meeting #101 	S1-230623

(Revision of S1-230165)

Athens, Greece, 20 - 24 February 2023

Source:	OPPO, Futurewei

pCR Title:	Consolidation on KPI for Ambient IoT

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	xuweijie@oppo.com,xuyang@oppo.com, amanda.xiang@futurewei.com

Abstract: This document provides a Text Proposal for the consolidation on KPI for Ambient IoT.

Discussion

28 use cases and 2 traffic scenarios have been agreed and captured in the latest version of TR 22.840v0.3.0, based on the functionality that ambient IoT can provided, the use cases and traffic scenarios can be put into the following 4 categorizations for KPI consolidation.

- Use case categorization 1: Inventory

- Use case categorization 2: Sensors

- Use case categorization 3: Positioning

- Use case categorization 4: Actuator (e.g., switch)

Motivation of use cases categorizations:

- There are different requirements (payload size, device density, transfer intveral, need of positioning etc.) for different categorizations;

- There are similar requirements for each categorization;

- Facilitate the specification implementation for the downlink group by take the specific characteristcis of each categorization.

In addition, the specific categorizations of the use cases are summaried in Table 1. Please note that some use cases (as highlighted) may belong to more than one categorization.

Table 1: Ambient IoT use case classification

| Use cases Categories | Use cases agreed in TR 22.840 v0.3.0 |
| --- | --- |
| Inventory | 1; 2; 4; 5; 7; 11;16; 27; traffic scenario 1 |
| Sensors | 3; 6; 13; 15; 17; 18; 19; 20; 22; 23; 24; 25; traffic scenario 2 |
| Positioning | 1; 2; 5; 8; 9; 10; 12; 14; 16; 21; |
| Actuator | 26; 28; |

****************Start of Change *************

7.2	Consolidated potential KPIs

Editor’s Note: 	This section summarizes the available KPI values from the various approved use cases based on TR22840v0.3.0. There is no KPI for the use cases in categorization 4: Actuator (e.g., switch).

The CPR number will be added later.  Companies can firstly check the categorization.

Table 7.2.1 KPI for inventory using ambient IoT

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Automated warehousing | 1s | NA | NA | <100/128bits/s | 96/128 bits | NA | 30m indoors | NA | 5~10km/h | NA | NA | NA | 2~3 m | UC#5.1 |
|  | 2 | Automatic Intralogistics in automobile manufacturing | >100 ms | NA | NA | <1 kbit/s | 96 bits | <1,5 Million/km2 | <30 meters Indoors | 600 000 m2 | NA | NA | NA | NA | [3] m | UC#5.5 |
|  | 3 | Ambient IoT devices for automated supply distribution | >10 s | NA | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km2 | NA | 600 000 m2 | NA | NA | NA | NA | [3] m (Indoor, 90% confidence level and in horizontal) | UC#5.16 |
|  | 4 | Container logistics in a flower auction | [<10 s] | [99,99%] | NA | [5] kbit/s | [96] bits | [< 1,3] Million/km2 | [35] m Indoors | [1 700 000] m2 | NA | NA | NA | NA | NA | TS#6.1 |
| Inodoor/ Outdoor | 5 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps (note 1) | 176bits | ≥1000/km2 | 50m Indoor 200m Outdoor | NA | Static or walking speed <6km/h | hundreds ms level | NA | NA | NA | UC5.2 |
|  | 6 | Medical instrument inventory management and positioning | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 50m Indoor 200m Outdoor | NA | Static or walking speed <6km/h | hundreds ms level | NA | NA | NA | UC#5.11 |
| Outdoor | 7 | Airport Terminal/ Shipping Port | >1sec | 99% | NA | NA | 256 bits (UL) | FFS | FFS | 1-10km2 | NA | NA | NA | NA | NA | UC#5.7 |

Table 7.2.2 KPI for sensors using ambient IoT

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Ambient IoT devices in smart home | 20 s | 99.9 % | NA | NA | 8~96bits | NA | [10-30]m Indoors | NA | Stationary | NA | NA | NA | NA | UC#5.6 |
|  | 2 | BSMR environmental supervision | 1s | NA | 99.9% | <1kbit/s | NA | NA | 30m indoors | NA | NA | NA | NA | NA | NA | UC#5.13 |
|  | 3 | Ambient IoT devices for smart laundry | >10 s | NA | NA | <100bit/s | Typically, < 100 bytes | 20 / 100m2 | NA | several m2 up to 1000 m2 | NA | NA | NA | NA | NA | UC#5.15 |
|  | 4 | Ambient IoT devices for food supply chain | >1 minute | NA | NA | <0.12 bit/s (Note 1) | Typically, < 100 bits | 1.5 Million devices/ km2 | NA | 30,000m2 | NA | NA | NA | NA | NA | UC#5.18 |
|  | 5 | Smart Agriculture | >1 s | 99.9% | NA | <1 kbit/s | <1000 bits | 1 per m2 | 30-100m | 500-70000 m2 per greenhouse | static | 1 hour | NA | NA | NA | UC#5.20 |
|  | 6 | Smart livestock farming (pig barns) | >10 s | NA | NA | <500 bit/s | Typically, [< 100 bytes] | 850 000 devices / km2 | 250 m Indoor | 6000 m2 | NA | 15 minutes to half an hour | NA | NA | NA | UC#5.23 |
|  | 7 | Cows in dairy stable | 1 s | [99,9%] | NA | [< 0.5] kbit/s | [500] bits | [< 1] /km2 | [35] m Indoors | [1 800] m2 | NA | NA | NA | NA | NA | TS#6.2 |
| Outdoor | 8 | Remote monitoring of transmission and distribution networks in smart grids | 1 s | NA | NA | [< 1kbit/s] | Typically, [< 100 bytes] | [< 10,000 /km2] | Outdoor: typically [50-200] meters | [several km2 up to 100 000 km2] | NA | NA | NA | NA | NA | UC#5.3 |
|  | 9 | Forest Fire Monitor | > 10sec | NA | 99.9% with fault tolerance | NA | NA | FFS | [150-FFS] meters | FFS | NA | FFS | NA | NA | NA | UC#5.19 |
|  | 10 | Smart dairy farm | >1 s | NA | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km2 | FFS | 430000 m2 | NA | 15 min | NA | NA | NA | UC#5.22 |
|  | 11 | Smart manhole cover remote monitoring | 10 s - 30 s | NA | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | FFS | City wide including rural areas | NA | 15 min | NA | NA | NA | UC#5.24 |
|  | 12 | Smart bridge health monitoring | 10 s | NA | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | FFS | Along the bridge | NA | 15 min | NA | NA | NA | UC#5.25 |

Table 7.2.3 KPI for positioning using ambient IoT

| Deployment |  | Scenarios |  | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor | 1 | Automated warehousing |  | 1s | NA | NA | <100/128bits/s | 96/128 bits | NA | 30m indoors | NA | 5~10km/h | NA | NA | NA | 2~3 m | UC#5.1 |
|  | 2 | Automatic Intralogistics in automobile manufacturing |  | >100 ms | NA | NA | <1 kbit/s | 96 bits | <1,5 Million/km2 | <30 meters Indoors | 600 000 m2 | NA | NA | NA | NA | [3] m | UC#5.5 |
|  | 3 | Remote lost item finding (Indoor) |  | >5s | 99% | NA | NA | 256 bits | <750 devices/100m2 | 10m | NA | NA | NA | NA | NA | NA | UC#5.8 |
|  | 4 | Finding Items in a home |  | NA | NA | NA | NA | NA | 20 Ambient IoT devices/(100m2) | 10m IC/PC/OOC | NA | Static/ Moving(<1m/s) | 500ms | NA | 95 % | 100 cm, 10 degree | UC#5.10 |
|  | 5 | Personal belongings finding (indoor) |  | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <5 per 100 m2 | 10 m | <200 m2 | Static | 1 per hour | NA | NA | [1-3] m, 90% availability | UC#5.12 |
|  | 6 | Parking area (e.g. in shopping centre) |  | 0.5 s | 99.9% | NA | <1 kbit/s | 96 bits | 2500/ 10000m2 | 10 m | NA | NA | NA | 0.5 s | 90% | 3 m | UC#5.14 |
|  | 7 | Shopping area (e.g. in shopping centre) |  | 0.5 s | 99.9% | NA | <1 kbit/s | 96 bits | 2500/ 10000m2 | 10m | NA | NA | NA | 0.5 s | 90% | 3 m | UC#5.14 |
|  | 8 | Ambient IoT devices for automated supply distribution |  | >10 s | NA | NA | <100 bit/s | Typically, <100 bytes | <1,5 Million/km2 | NA | 600 000 m2 | NA | NA | NA | NA | [3] m (Indoor, 90% confidence level and in horizontal) | UC#5.16 |
|  | 9 | Museum guide (indoor) |  | [2] s | 99.9% | NA | [< 1 kbit/s] UL | [96] bits | [<10,000 /km²] | [30] m | [20,000] m² | [3] km/h | NA | NA | 90% | [3] m | UC#5.21 |
| Indoor/ Outdoor | 10 | Medical instrument inventory management and positioning |  | hundreds ms level | 99% | NA | <2Kbps | 176bits | ≥1000/km2 | 50m Indoor 200m Outdoor | NA | Static or walking speed <6km/h | hundreds ms level | NA | NA | NA | UC#5.2 |
|  | 11 | Absolute positioning |  | NA | NA | NA | NA | NA | NA | NA | NA | Indoor - up to [5 km/h] Outdoor - up to [20 km/h] | NA | [10 s] | [95 %] | [10 m] Horizontal Accuracy [3 m] Vertical Accuracy | UC#5.9 |
| Outdoor | 12 | Personal belongings finding(outdoor) |  | 1 s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m2 | 100 m | Up to the whole PLMN | Static | 1 per hour | NA | NA | FFS | UC#5.12 |

*******************End of Change*****************