3GPP TSG-SA WG1 Meeting #88 	S1-230631

Athens, Greece, 20 - 24 February 2023	(revision of S1-230343)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.104 | CR | 0093 | rev | 1- | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional clarification on security , privacy for mobile robots using edge cloud |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange, Verizon, Ericsson, Huawei NTT DOCOMO, China Unicom, Vodafone, Nokia, Deutsche Telekom, InterDigital, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGINDUS |  |  |  |  |  | Date: |  |  | 2023-01-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add clarifications related to security and privacy for mobile robots enabled by edge cloud. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A further security and privacy requirement will be missing for Mobile Robots use cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Relevant aspects will not be updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******       Start of  changes  ***********

### A.2.2.3	Mobile robots

Mobile robots and mobile platforms, such as automated guided vehicles, have numerous applications in industrial and intra-logistics environments and will play an increasingly important role in the Factory of the Future. A mobile robot essentially is a programmable machine able to execute multiple operations, following programmed paths to fulfil a large variety of tasks. This means, a mobile robot can perform activities like assistance in work steps, collaboration with other robots, e.g., for car assembly, and transport of goods, materials and other objects. Mobile robot systems are characterised by a maximum flexibility in mobility relative to the environment, with a certain level of autonomy and perception ability, i.e., they can sense and react with their environment.

Autonomous guided vehicles (AGVs) are a sub-group of mobile robots. AGVs are driverless and used for moving materials efficiently within a facility. A detailed overview of the state of the art of autonomous-guided-vehicle systems is provided elsewhere in the literature [16]. All mobile robots incorporate all functionalities needed for an AGV.

Today, the AGV’s control intelligence is hosted inside the AGV. In the future, centralised fleet control will be hosted in the edge cloud, which will require reliable wireless communication between the control entity and all AGVs. Also, the current paradigm of pre-describing a route for the AGV will be replaced with target-based navigation. This paradigm change will make AGV routes more flexible.

Mobile robots are monitored and controlled from a guidance control system. Radio-controlled guidance control is necessary to get up-to-date process information, to avoid collisions between mobile robots, to assign driving jobs to the mobile robots, and to manage the traffic of mobile robots. The mobile robots are track-guided by the infrastructure with markers or wires in the floor or guided by own surround sensors, like cameras and laser scanners.

Mobile robot systems can be divided in operation in indoor, outdoor and both indoor and outdoor areas. These environmental conditions have an impact on the requirements of the communication system, e.g., the handover process, to guarantee the required cycle times.

Where this document does not explicitly refer to AGVs, the term mobile robots applies to AGVs as well as to mobile robots.

Table A.2.2.3-1: Service performance requirements for mobile robots

| Use case # | Characteristic parameter |  |  |  | Influence quantity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Communication service availability: target value [%] | Communication service reliability: mean time between failures | End-to-end latency: maximum | Service bitrate: user experienced data rate | Message size [byte] | Transfer interval: lower bound | Transfer interval: target value (note) | Transfer interval: upper bound | Survival time | UE speed | # of UEs | Service area |
| 1 | > 99.999 9 | ~ 10 years | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 1 ms to 50 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 2 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 15 k to 250 k | – < 25 % of target transfer interval value | 10 ms to 100 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 3 | > 99.999 9 | ~ 1 year | < target transfer interval value | – | 40 to 250 | – < 25 % of target transfer interval value | 40 ms to 500 ms | + < 25 % of target transfer interval value | target transfer interval value | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| 4 | > 99.999 9 | ~ 1 week | 10 ms | > 10 Mbit/s | – | – |  | – | – | ≤ 50 km/h | ≤ 2,000 | ≤ 1 km2 |
| NOTE: The transfer interval is not so strictly periodic in these use cases. The transfer interval deviates around its target value within bounds. The mean of the transfer interval is close to the target value. |  |  |  |  |  |  |  |  |  |  |  |  |

Use case one

Periodic communication for the support of precise cooperative robotic motion control (transfer interval: 1 ms), machine control (transfer interval: 1 ms to 10 ms), co-operative driving (10 ms to 50 ms).

Use case two

Periodic communication for video-operated remote control.

Use case three

Periodic communication for standard mobile robot operation and traffic management.

Use case four

Real-time streaming data transmission (video data) from a mobile robot to the guidance control system.

Additional information

AGVs have the following needs.

–	The direct-device control is time-critical since the communication involves safety-relevant functions such as emergency stop and the avoidance of obstacles.

–	For the implementation of swarm intelligence, position and availability information are needed. A possible route change due to a blocked route affects the routes of all other AGVs that will follow. The communication is less time-critical than for safety-relevant functions.

–	Camera-based navigation requires high data rates. Examples for camera-based navigation are the positioning of boxes/containers, detection of persons and obstacles, as well as creation and administration of a map for flexible navigation. Note that sensor-based navigation requires lower data rates than camera-based navigation.

Mobile robots have additional needs.

–	The mobile robot provides an additional service during transport (for instance quality inspection, scanning of surroundings, asset identification, carrying of work pieces). In order to reduce the uplink data rate, pre-compression of data is possible directly on the device. The communication is not or at least less time-critical than the motion control of the mobile robot.

–	The mobile robot needs to interact with the periphery (for instance intelligent storage racks, stationary robots, and moving machines). This communication is time-critical. Interaction with the periphery can be relevant at the start point, end point, and also at several intermediate stations (for instance the collection of parts from intelligent storage racks). The exact position and orientation can be determined by a centering station and the AGV sensors. Additional scanning by the robot with a video camera may be necessary.

–	For some mobile robots, their control intelligence might be centralized and hosted in an edge cloud. They require secure communication towards the edge cloud. If the path layout such a mobile robot follows (e.g., including indoor and outdoor) causes it to switch the communication between a private network and a public network, consideration should be given to provide the required level of security for communication.

–	Mobile robots whose control intelligence is centralized and hosted in the edge cloud needs privacy protection of data stored in the edge cloud

.

******       End of  changes  ***********