3GPP TSG SA WG 1 Meeting #101 	S1-230636

Athens, Greece, 20 - 24 February 2023	(revision of S1-230509)

Source:	Qualcomm, CATT

pCR Title:	Use Case on Gesture Recognition for Application Navigation and Immersive Interaction

Draft Spec:	3GPP TR 22.837 v0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri  oawoniyi@qti.qualcomm.com；

Qing Wan wanqing1@cictmobile.com

Abstract: This document proposes a use case of NR based sensing for gesture recognition

1. Introduction

Introduce a gesture recognition use case into the sensing study

2. Reason for Change

Adding a use case with potential functional requirements and KPIs  for gesture recognition to the study

3. Conclusions

none

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v0.3.0.

* * * First Change * * * *

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	W. Favoreel, "Pedestrian sensing for increased traffic safety and efficiency at signalized intersections," 2011 8th IEEE International Conference on Advanced Video and Signal Based Surveillance (AVSS), 2011, pp. 539-542, doi: 10.1109/AVSS.2011.6027406.

[3]	Advances in Wildlife Crossing Technologies: https://highways.dot.gov/public-roads/septoct-2009/advances-wildlife-crossing-technologies.

[4]	Protection Detection: Making Roads Safe for Drivers and Wildlife: https://onlinepubs.trb.org/onlinepubs/webinars/201118.pdf.

[5]	F. Liu et al., "Integrated Sensing and Communications: Towards Dual-functional Wireless Networks for 6G and Beyond," in IEEE Journal on Selected Areas in Communications, doi: 10.1109/JSAC.2022.3156632.

[6]	T. S. Rappaport, G. R. MacCartney, M. K. Samimi and S. Sun, "Wideband Millimeter-Wave Propagation Measurements and Channel Models for Future Wireless Communication System Design," in IEEE Transactions on Communications, vol. 63, no. 9, pp. 3029-3056, Sept. 2015, doi: 10.1109/TCOMM.2015.2434384.

[7]	C. Han, Y. Bi, S. Duan and G. Lu, "Rain Rate Retrieval Test From 25-GHz, 28-GHz, and 38-GHz Millimeter-Wave Link Measurement in Beijing," in IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, vol. 12, no. 8, pp. 2835-2847, Aug. 2019, doi: 10.1109/JSTARS.2019.2918507.

[8]	IEEE 802.11-18/0611r16: “Wireless LANs, WiFi Sensing Uses Cases”

[9]	TEM STANDARDS TEM STANDARDS AND RECOMMENDED PRACTICE: https://unece.org/fileadmin/DAM/trans/main/tem/temdocs/TEM-Std-Ed3.pdf

[11]	3GPP TR 22.856, "Localized Mobile Metaverse Services".

[13]	“Velodyne™ LiDAR VPL-16 User Manual,” 63-9243 Rev. E, Velodyne™ LiDAR, https://velodynelidar.com/wp-content/uploads/2019/12/63-9243-Rev-E-VLP-16-User-Manual.pdf

[14] 		Liu, A., Huang, Z., Li, M., Wan, Y., Li, W., Han, T.X., Liu, C., Du, R., Tan, D.K.P., Lu, J. and Shen, Y., 2022. A survey on fundamental limits of integrated sensing and communication. IEEE Communications Surveys & Tutorials, 24(2), pp.994-1034.

[15] 		https://medium.com/desn325-emergentdesign/s-l-a-m-and-optical-tracking-for-xr-cfabb7dd536f

[16] 		Dwivedi, S., Shreevastav, R., Munier, F., Nygren, J., Siomina, I., Lyazidi, Y., Shrestha, D., Lindmark, G., Ernström, P., Stare, E. and Razavi, S.M., 2021. Positioning in 5G networks. IEEE Communications Magazine, 59(11), pp.38-44.

[17]	T. Murakami et al, “Wildlife Detection System Using Wireless LAN Signal,” in NTT Technical Review vol.17, No.6, pp. 45-48, June 20019, https://www.ntt-review.jp/archive/ntttechnical.php?contents=ntr201906fa13.pdf&mode=show_pdf

[18]	Eradication of elephant mortality and injury due to railway accidents through automatic tracking and alert system in IEEE Conference Publication, IEEE Xplore

[19]	Impact of wild animals (deer and bears) on train operations 210616_KO_Animal2.pdf (jrhokkaido.co.jp) (in Japanese) [z]	Rail Industry Safety Induction Handbook: https://railsafe.org.au/__data/assets/pdf_file/0009/32022/Rail-Industry-Safety-Induction-RISI-Handbook-V5.1.pdf

[21]	Society of Automotive Engineers (SAE), “Taxonomy and definition for terms related to Driving automation systems for on-Road Motor Vehicles”, https://www.sae.org/standards/content/j3016_202104/

[22]	Census of Fatal Occupational Injuries Summary, 2020, https://www.bls.gov/news.release/cfoi.nr0.htm

[23]	Javed MA, Muram FU, Hansson H, Punnekkat S, Thane H. Towards dynamic safety assurance for Industry 4.0. Journal of Systems Architecture. 2021 Mar 1; 114:101914.

[24]	American National Standards Institute/Industrial Truck Safety Development Foundation, Safety standard for driverless, automatic guided industrial vehicles and automated functions of manned industrial vehicles, December 2019, 2019, [Online] http://www.itsdf.org.

[25]	Moore, Erik George, "Radar Detection, Tracking and Identification for UAV Sense and Avoid Applications" (2019). Electronic Theses and Dissertations. 1544.

[26] 	I. Sobron, I. Landa, I. Eizmendi and M. Velez, "Adaptive TOA Estimation with Imperfect LOS and NLOS Knowledge in UWB Positioning Systems," 2020 IEEE SENSORS, 2020, pp. 1-4. available at: https://sci-hub.wf/10.1109/sensors47125.2020.9278589

[27] 	L. Wang, Z. Zhang, X. Di and J. Tian, "A Roadside Camera-Radar Sensing Fusion System for Intelligent Transportation," 2020 17th European Radar Conference (EuRAD), 2021, pp. 282-285.

[28]	5GAA_White_Paper_C-V2X Use Cases Volume II: Examples and Service Level Requirements

[29] 	https://www.ieee802.org/11/Reports/tgbf_update.htm

[30]	https://mentor.ieee.org/802.11/dcn/20/11-20-1712-02-00bf-wifi-sensing-use-cases.xlsx

[31]	X. Liu, J. Cao, S. Tang and J. Wen, "Wi-Sleep: Contactless Sleep Monitoring via WiFi Signals," 2014 IEEE Real-Time Systems Symposium, 2014, pp. 346-355, doi: 10.1109/RTSS.2014.30.

[32]	Chen V C. The micro-Doppler effect in radar. Artech house, 2019.

[33]	3GPP TS 22.261: “Service requirements for the 5G system”.

[34]	A. Chebrolu, "FallWatch: A Novel Approach for Through-Wall Fall Detection in Real-Time for the Elderly Using Artificial Intelligence", 2021 Third International Conference on Transdisciplinary AI (TransAI), 2021, pp. 57-63, doi: 10.1109/TransAI51903.2021.00018, https://ieeexplore.ieee.org/document/9565618

[35]	B. A. Alsaify et al., “A CSI-Based Multi-Environment Human Activity Recognition Framework” Applied Sciences 12, no. 2: 930, 2022. https://doi.org/10.3390/app12020930

[36]	U. Saeed U et al., "Discrete Human Activity Recognition and Fall Detection by Combining FMCW RADAR Data of Heterogeneous Environments for Independent Assistive Living", Electronics 10(18):2237, 2021. https://doi.org/10.3390/electronics10182237

[37]	C. Dou, H. Huan, "Full Respiration Rate Monitoring Exploiting Doppler Information with Commodity Wi-Fi Devices". Sensors 21, 3505, 2021. https://doi.org/10.3390/s21103505

[38]	J. Pu, H. Zhang, "RF-Heartbeat: Robust and Contactless Heartbeat Monitoring Based on FMCW Radar", 2021. TechRxiv Preprint. https://doi.org/10.36227/techrxiv.15021645.v2

[39] 	H. V. Habi and H. Messer, “Recurrent Neural Network for Rain Estimation Using Commercial Microwave Links,” in IEEE Transactions on Geoscience and Remote Sensing, vol. 59, no. 5, pp. 3672-3681, May 2021, doi: 10.1109/TGRS.2020.3010305.[40]                    Roberto Opromolla, etc., “Perspectives and Sensing Concepts for Small UAS Sense and Avoid”, 2018 IEEE/AIAA 37th Digital Avionics Systems Conference (DASC)

[41]	https://www.rwjbh.org/trinitas-regional-medical-center/treatment-care/sleep-disorders/sleep-apnea/

[42]	https://my.clevelandclinic.org/health/articles/10881-vital-signs

[43]	3GPP TR 22.855, "Study on Ranging-based Services".

[44]	Guoxuan Chi, et. al., "Wi-Drone: Wi-Fi-based 6-DoF Tracking for Indoor Drone Flight Control", MobiSys 22, Association for Computing Machinery, 2022.

[45]	Report on Automated Valet Parking: technology assessment and use case implementation description – 5G Automotive Association (5gaa.org). https://5gaa.org/news/report-on-automated-valet-parking-technology-assessment-and-use-case-implementation-description/

[46]	Nie Y B , Zhang L . Main amendments to Working Safety Regulation of State Grid Company(Dynamical Part for Hydrodynamic Power Plant)[J]. East China Electric Power, 2008.

[47] 	Giuseppe Fragapane, René de Koster, Fabio Sgarbossa, Jan Ola Strandhagen, Planning and control of autonomous mobile robots for intralogistics: Literature review and research agenda, European Journal of Operational Research, Volume 294, Issue 2,2021, Pages 405-426

[48]	Li S, Li X, Lv Q, et al. WiFit: Ubiquitous bodyweight exercise monitoring with commodity wi-fi devices, 2018 IEEE SmartWorld, Ubiquitous Intelligence & Computing, Advanced & Trusted Computing, Scalable Computing & Communications, Cloud & Big Data Computing, Internet of People and Smart City Innovation, IEEE, 2018: 530-537.

[49]	https://www.synopsys.com/automotive/autonomous-driving-levels.html

[x]                      National Academies of Sciences, Engineering, and Medicine. 1995. Virtual Reality: Scientific and Technological Challenges. Washington, DC: The National Academies Press. https://doi.org/10.17226/4761

[Y]   		C. Liu, Y. Li, D. Ao and H. Tian, “Spectrum-Based Hand Gesture Recognition Using Millimeter-Wave Radar Parameter Measurements,” in IEEE Access, vol. 7, pp. 79147-79158, 2019, doi: 10.1109/ACCESS.2019.2923122.

* * * Next Change * * * *

## 5.x. Use case on Gesture Recognition for Application Navigation and Immersive Interaction

### 5.x.1	Description

As a new way of human-device interface, gesture recognition enables a more intuitive interaction between humans and machines, compared to the conventional text or GUI-based interfaces. Common applications of gesture recognition include touchless control of mobile devices, such as smartphones, laptops, and smart watches. Compared to other use cases, such as sports monitoring or sleep monitoring, gesture recognition requires higher resolution, higher update rate, and lower latency, which makes it more challenging in terms of resource utilization and processing complexity.

Gesture recognition identifies motions and postures of human body parts, such as head, hands, and fingers. As shown in Figure Fig 5.X.1-1, Gesture recognition can be applied to various applications such as human motion recognition, keystroke detection, sign language recognition and touchless control.

|  | Gesture/motion/posture recognition - Human motion recognition - Keystroke detection - Touchless control - Sign language recognition |
| --- | --- |

Fig 5.X.1-1 Gesture Recognition

In this use case, we focus on application of gesture recognition for touchless control and immersive (i.e. XR) application.

For touchless control, the identified gestures are then interpreted to specific behaviours or operations of the device, including locking/unlocking a screen, increasing/decreasing volume, and navigating forward/backward web pages.

For XR application, the position tracking and mapping of the human body is a basic requirement that permeates XR application to provide the immersive experience[x]. Hereby a variety of sensors are integrated in the XR devices to measure the movement of the human body (e.g., head, eye, hand, arm, etc) in order to simulate normal human mimicry. Besides, the hand tracking goes beyond the simulation and enables a natural and intuitive way of the interaction between the human and the machine compared with the use of the physical controller. The gesture recognition and hand tracking becomes a vital function in XR applications, especially when the human and the controlled object stay in physical and virtual world separately.

For both touchless control and XR application, NR-based RF sensing is suitable for gesture recognition because certain RF signals can detect small body movements and the RF signals are not susceptible to the ambient illumination condition and occlusions in the environment. In addition, NR-based RF sensing allows for finger and hand tracking in a lower-complexity and economic manner.

### 5.x.2	Pre-conditions

There are two roommates, Jose and Bob, both of whom subscribed to MNO A, which has deployed RAN entity (e.g., an indoor base station) supporting NR-based sensing.

Jose subscribes to the touchless user interface service and his mobile device has NR sensing capability.

Bob subscribes to the immersive interaction service, provided by both MNO A and XR application (e.g. game, sports training) provider AppX on the access rights of the interaction information relevant to Bob’s hands. Bob’s UEs (e.g. smartphone, XR device such as headset) are capable of NR-based sensing and Bob’s XR device also have other sensors (e.g. IMU) embedded on the device.

### 5.x.3	Service Flows

Social Media Navigation service with Gesture Recognition

Step 1: Sitting in this room, Jose is reading through social media posts using his smartphone, to navigate to either the previous or next posts. Jose waves his hand in the air from left to right or from right to left.

Step 2: Jose’s smartphone detects the hand gesture using 5G wireless sensing using the RAN entity, UE or both. The smartphone and RAN entity can send sensing data (with extracted gesture features such as range and Doppler of the detected gesture) to the 5G network.

Step 3: 5G network then aggregates and processes the information collected from the UE and RAN entity to detect Jose’s gesture and provides the sensing results to Jose’s smartphone which is shared with the social media application and used for the navigation of the post.

Immersive interaction service with Gesture Recognition

Step 1: Bob launches the XR application in his room, at which moment one avatar is generated to represent him in the virtual world of the XR application, and the immersive interaction service is activated as shown in Fig 5.X.3-1.

Step 2: When Bob sees a basketball flying toward him, he bends the fingers to catch the ball and throws it back. The characteristics of the gesture (e.g. range, doppler shift) are detected using NR sensing signals of UEs, the RAN entity or both.

Step 3: 5G network (e.g. 5GC) collects the 3GPP sensing data from UEs, RAN entity or both, process the data and exposes the sensing result (e.g. 3D position, velocity) to XR application. In parallel, the non-3GPP sensing data obtained from the XR device is transmitted to the application platform transparently through UE to 5GC.

Step 4: The gesture and hand movement are recognized and the basketball bounces to another direction as a sensing result. The entire course will be presented on Bob’s headset as that the basketball is caught by one hand of Bob’s avatar and thrown back.

Fig 5.X.3-1 Hand Tracking in XR applications

### 5.x.4	Post-conditions

Due to the RF sensing capability in Jose’s mobile device and a nearby RAN entity, Jose’s gestures are detected and used to navigate the social media posts on his phone.

Similarly, due to the RF sensing capability in Bob’s smartphone, XR device and a nearby RAN entity, Bob’s gesture and the motion of his fingers and hands will be recognized and tracked correctly. The avatar in XR application will show the correct gesture and execute the correct action triggered by the gesture.

### 5.x.5	Existing features partly or fully covering the use case functionality

None

### 5.x.6	Potential New Requirements needed to support the use case

Editor’s note: Functional requirements are FFS .

[PR 5.x.6-001] The 5G system shall be able to provide sensing with the following KPIs

Table 5.x.6-1	Performance requirements of sensing results for gesture recognition

| Scenario | Sensing service area | Confidence level [%] | Motion rate accuracy | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Gesture recognition | Indoor | N/A | N/A | 0.01 | 0.01 | 0.1 | 0.1 | [0.04] | [0.3] | FFS | ≤0.1 | FFS | FFS |
| NOTE: KPIs provided in table are derived from [Y] and with the use of the pre-defined gestures to be identified some of the KPIs e.g. positioning accuracy and range resolution could be relaxed. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Changes * * * *