3GPP TSG SA WG 1 Meeting #101 	S1-230652

Athens, Greece, 20 – 24 February 2023	(revision of S1-230279, 545)

Source:	Samsung

pCR Title:	Pseudo-CR on Update to Use Case 5.4

Draft Spec:	3GPP TR 23.837 v.0.3.0

Agenda item:	7.1

Document for:	Approval

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: This pCR removes editor's notes and cleans up the user of terminology in the use case.

1. Introduction

This pCR proposes to remove Editor's Notes.

| 5.4.3 Editor's Note: Additional details on the use of LiDAR, cameras and/or spatial localization need to be added. | Details concerning the use of LiDAR, cameras and spatial localization will not be added. |
| --- | --- |
| 5.4.6 Editor's Note: It is FFS if explicit requirements for LiDAR, cameras and other sensors need to be added. | Explicit requirements for LiDAR, cameras and other sensors will not be added. |
| 5.4.6 Editor's Note: Using existing E-UTRA measurements is FFS. | E-UTRA sensing is not discussed in this use case. |

2. Reason for Change

The end of the study is near and the Editor's Notes need to be removed at SA1 101. The text is aligned with new terminology and clarified.

3. Discussion

545 revision aligns terminology

652 removes a few remaining 'RF' terms and some meaningless text that was in the requirement section 6.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v.0.3.0.

FIRST CHANGE

## 5.4	Use Case on Transparent Sensing Use Case

### 5.4.1	Description

In general, a UE senses using either or combination of the non-3GPP sensors such as camera, Lidar, 3GPP-based sensing. In 3GPP sensing, the Sensing transmitters and Sensing receivers sense for stationary and moving Objects around them – using time-difference-of-arrival (TDoA), angle-of-arrival (AoA), angle-of-departure (AoD) measurements, RSSI etc. as shown in Figure 5.4.1-1 [14]. Transparent sensing is a use case in which sensing data is captured by Sensing transmitter and/or Sensing receiver and communicated so that the 5GS is aware of the 3GPP sensing information, while the sensing result is the result of non-3GPP sensing data and results are transparent to 5GS. From this information, service enablers can be defined. One example of such information is location data, whose corresponding service enabler is Location Based Services.

Figure 5.4.1-1: BS and UE sensing Objects

In this use case, sensing data is made available to the 5GS, and the requirements for this exposure are considered. The data so obtained can be used for diverse purposes. One such purpose is Localization (identifying both a three- dimensional position and orientation.) Transparent Sensing data used for Localization is described in TR 22.856 [11]

Figure 5.4.1-2: Opaque and Transparent Sensing Data

The distinguishing characteristic of this use case is that the sensing data is provided to the 5GS itself.

The application server receiving 'transparent sensing data' as shown in figure 5.4.1-2 can be operated by the MNO. This enables the MNO to provide specific processing to produce 'sensing results as a service,' where the sensing measurement data is supplied by sensors owned and operated by third parties, subscribers, etc.

In this use case it is the 5GS that receives 3GPP and non-3GPP sensing data, not a third party.

### 5.4.2	Pre-conditions

A UE has access to one or more sensors. In this use case. the UE has access to four sensors: NR-based sensing, 3D LiDAR, an RGB Camera and a Smart Phone Camera. The sensors' physical configuration is known (e.g. the cameras are 10 cm apart). The NR-based sensing capabilities of the UE and its connected BS are used to capture information about the nearby environment by the UE.

A mobile network MN supports the acquisition of sensing data. We term this support by the network a 'sensing data consuming service.'

### 5.4.3	Service Flows

The user U activates a mechanism to enable Non-3GPP sensing data acquisitionthat can be collected at U's UE.

The user U provides this non-3GPP sensing data via the 5GS. This process is analogous to activating or enabling a location tracking service.

MN acquires sensing measurement data provided by U's UE, for a period of time.

MN can also acquire 3GPP sensing data. 3GPP-RF sensing measurement data can be processed only in 5GS to derive sensing results. The sensing results and the Non-3GPP sensing measurement data can be combined to produce a Fused sensing result.

The user U deactivates the mechanism to provide sensing measurement to the 5GS.

### 5.4.4	Post-conditions

The sensing data acquired by the 5GS is processed in order to enable other services. The processed information can for example provide 'Spatial Localization' information that can be exposed to authorized third parties, as discussed in 22.856 [11], "Spatial Localization Use Case".

### 5.4.5	Existing feature partly or fully covering use case functionality

Positioning in 5G Networks been proposed in 3GPP release-16, it specifies positioning signals and measurements for the 5G NR. In release-16, 5G Positioning architecture extends 4G positioning architecture by adding Location Management Function (LMF) and Transmission reception points (TRP).  5GS provides new positioning methods based on multi-cell round-trip time measurements, multiple antenna beam measurements, multiple to enable downlink angle of departure (DL-AoD) and uplink angle of arrival (UL-AoA) [15][16]. The Rel-17 5G system supports positioning of the device-based but not device-free – objects that do not radiate EM signals. [14][15][16]

The 5GS already supports transport of sensor data. The table below provides indicative performance requirements for media used for sensor information communication.

| Sensor Type | Uplink KPI | Remarks |
| --- | --- | --- |
| 3D Lidar | 30 Mbps | An example 3D LiDAR: 16 channel, 0.3M data points, dual return mode 2 bytes distance, 1byte [13] |
| Industrial RGB Camera | 16 ~ 800 Mbps | 2,592 x 2,048 x 10bits x 2.5 Hz x 6 EA, compression ratio 2% |
| Smart Phone Camera | 4 ~ 200 Mbps | 2,160 x 2,880 x 8bits x 1 Hz x 4 EA, compression ratio 2% |

Figure 5.4.5-1: Performance Requirements (already possible to fulfill with the 5GS)

### 5.4.6	Potential New Requirements needed to support the use case

[PR.5.4.6 - 001] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to receive uplink 3GPP and non-3GPP sensing data from authorized Sensing transmitter and/or Sensing receiver.

NOTE 1: 	This requirement assumes there is some functionality in the 5GS to discern and interpret the acquired 3GPP and non-3GPP sensing data.

[PR.5.4.6 - 002] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to expose 3GPP sensing results to trusted third parties.

[PR.5.4.6 - 003] Subject to user consent and national or regional regulation, based on operator policy, the 5GS shall support a mechanism to expose sensing non-3GPP sensing measurement results to trusted third parties.

[PR.5.4.6 - 004] Subject to user consent, network operator policy and national or regional regulation, the 5GS shall support a mechanism to enable Sensing transmitters and Sensing receivers to acquire 3GPP sensing measurement data to capture information about the nearby environment and for this to be combined with Non-3GPP sensing data to produce a Fused sensing result.

NOTE 2:	This requirement does not imply or allow 3GPP sensing data to be exposed to third parites. This data is considered confidential.

END OF CHANGES