3GPP TSG-SA WG1 Meeting #101 	S1-230654

(Revision of S1-230515)

Athens, Greece, 20 - 24 February 2023

Source:	OPPO

pCR Title:	pCR on overview of TR22840

Draft Spec:	3GPP TR 22.840

Agenda item:	7.2

Document for:	Approval

Contact:	xuweijie@oppo.com, v-qiyinan@oppo.com

Abstract: This document provides a Text Proposal on the overview part of TR22840.

****************Start of Change *************

4 Overview

In today’s IoT networks, the legacy IoT devices are usually driven by conventional batteries with limited lifespan, which has significantly affected user experience in a negative way. The astronomical growing of IoT network together with the advent of huge amount of IoT devices has pushed the maintaining expenditure, including both labor and battery costs, to a whole new level. Billions of conventional batteries have been disposed every year and only a small part of it can be efficiently recycled, leading to harmful impacts on the earth’s ecosystem. In some extreme environmental conditions, maintaining the operation of IoT networks and replacing the batteries can be quite challenging. In this regard, battery-free IoT communication has been proposed and it will improve the network performance and sustainability, and expand the application scenarios. In addition, battery-free communication can be much more environmentally friendly and much safer for the kids and the elders. By removing the conventional battery, the device size and cost can be significantly reduced, thus paving the way to a variety of new applications.

In the 5G era, various LPWA technologies such as MTC, NB-IoT, RedCap etc. have been developed to fulfil the increasing demand from verticals. These LPWA technologies have achieved low cost, low power and massive connections and can meet requirements of many applications. However, there are still lots of use cases and applications that can not be addressed in the following circumstances. Firstly, a device driven by a conventional battery is not applicable, e.g. under extreme environmental conditions (e.g. high pressure, extremely high/low temperature, humid environment). Secondly, maintenance-free devices are required (e.g. no need to replace a conventional battery for the device). Finally, ultra-low complexity, very small device size/form factor (e.g. thickness of mm), longer life cycle, etc. are required.

Ambient power-enabled IoT is a promising technology to fulfil the unmet requirements as mentioned above. Ambient power-enabled Internet of Things device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other power source that could be seen suitable. This study targets at supporing ambient IoT device with low complexity, small size and lower capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices) with emphasis on network efficiency. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years).

Therefore, a new kind of IoT service for the verticals will be provided by combining ambient power-enabled IoT with celluar network and the 3GPP ecosystem will benefit vastly.

*******************End of Change*****************