3GPP TSG SA WG 1 Meeting #101 	S1-230677

Athens, Greece, 20 - 24 February 2023  	(revision of S1-230661, S1-230464, S1-230028)

Source:	NEC, Qualcomm, NOVAMINT, Lockheed Martin

pCR Title:	Pseudo-CR on Use Case on a set of devices accessing local NPN services in inter NPN – PLMN scenario

Draft Spec:	3GPP TR 22.841 v 0.20

Agenda item:	7.9

Document for:	Approval

Contact:	jasmina.mcmenamy@neclab.eu; g-fujimoto@nec.com

Abstract: This paper proposes an addition of a new use case in TR 22.841 v.0.2.0.

1. Introduction

<Introduction part >

2. Reason for Change

Introduction of a new use case

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.841 v.0.2.0.

* * * First Change * * * *

### 5.x.1	Description

In many cases, devices operating on the factory floor will be grouped based on their function, location or other characteristics. For example, Automated Guided Vehicles (AGVs) may be required to move in groups to transport large or heavy work-pieces cooperatively [TS 22.104]. This next generation of AGVs will have a remote control system, residing in the edge could, guiding them. Such AGVs with a remote controller form a closed-loop system, requiring timely delivery of commands, feedback, and, generally, access to services. The communication link(s) between the control system and AGV must be reliable in both directions. They also need to provide the required QoS in up-link and downlink to ensure AGVs’ safe and efficient operation and enable flexibility of the routes that they may take.

Furthermore, AGVs may be, e.g. moving material from the outside loading areas to the inside of the plant and vice versa. The outside loading area may not be in the immediate vicinity of the plant and, hence, not within the coverage of PNI-NPN. Furthermore, such AGVs may be required to transport the material between multiple sites in a locality where there is limited or no NPN coverage between the sites. Also, the distance between different sites may be too large for an NPN to cover.

Hence, this use case addresses a set of infrastructure-assisted AGVs that stream high-quality video. Each AGV has the same QoS and has to move in a coordinated way with other AGVs. They also require, closed-loop control, whereby a central system controller that may be located in the edge cloud controls the drives and movements of each of the AGVs. The communication between AGVs and the control system is bidirectional. The use case addresses the requirement for smooth mobility and high-quality video streaming to the server residing in the NPN’s network, via an MNO’s access network when AGVs are moving in and out of the coverage footprint of NPN.

Finally, it should be possible that, based on the QoS requirements and the conditions of two networks (e.g. availability and available capacity), both networks are utilised so that the data traffic to and from each AGV can be appropriately split or steered.

Figure 1 This figure illustrates loading areas in a vicinity of the plant, where NPN-OP#1 and MNO-A provide 3GPP access networks, Note that loading areas may be located further away from the plant.

NOTE 1:  As stated in TS 22.104, AGVs are a sub-group of mobile robots. In this use case, the term AGV refers to the future generation of AGVs, which can be controlled remotely, e.g. from the edge cloud, enabling flexible route navigation.

NOTE 2:

This use case does not preclude the use of UE-UE/Prose communication in addition to communication with the radio access network.

* * * Next Change * * * *

### 5.x.2 Pre-conditions

AGVs with on-board UEs are inside the plant, where NPN operator NPN-OP#1, which is a PNI-NPN operator, provides coverage using 3GPP access network(s), and each AGV’s connectivity to the NPN-OP#1’s data network.

Each UE of each AGV supports dual-3GPP access and dual-radio over the two RATs, and each AGV is equipped with cameras and sensors to collect and report real-time information.

Video streaming is one of the supported services by AGVs. All AGVs have the same QoS requirements for video streaming.

A specific traffic policy for each AGV to access NPN-OP#1 data network includes the use of MNO-A’s access network when there is no coverage by NPN-OP#1, or dual access (home network and MNO-A network) based on QoS requirements.

* * * Next Change * * * *

### 5.x.3 	Service Flows

1) A set of AGVs are inside the plant, at an AGV’s stationary point, and within the NR coverage of NPN-OP#1. Each AGV is in idle mode.

2) The controller instructs AGVs to start real-time video streaming to the server in the edge cloud, based on which the controller navigates their movement through the plant and coordinates cooperative carrying. AGVs are using NPN-OP#1 access network to provide real-time video streaming.

3) AGVs have moved outside, by Loading Area 1, where there is an excellent coverage by MNO-A. At the same time, NPN OP#1 is experiencing a surge in capacity demand. To continue providing high-quality video streaming to the remote control system in its data network, the traffic of all UEs is steered to utilize MNO-A’s access network only.

4) The designated material is offloaded correctly and the AGVs are instructed to move to Loading Area 2, while still utilizing MNO-A’s network, as NPN-OP’1s network coverage in this area is poor.

5) In Loading Area 2, AGVs are instructed to collect another object and transport it to the loading area inside the plant.

6) As AGVs move from Loading Area 2 to the inside of the plant, NPN-OP#1 coverage is excellent and the network has ample capacity.  Streaming video of each UE  is switched to NPN-OP#1’s network.

7) AGVs offload the collected object to the loading area inside the plant and are instructed to return to the stationary point and each to stop the streaming service.

Figure 2 AGVs are moving through the factory floor and to the outside of the plant, streaming real-time video to the NPN’s edge cloud by splitting and switching/steering traffic between NPN-OP#1 and MNO-A networks.

* * * Next Change * * * *

### 5.x.4	Post-conditions

All AGVs have had a reliable access and required QoS for video-streaming while moving seamlessly between NPN-OP#1 and MNO-A coverage. AGVs have safely and efficiently moved inside and outside the plant, transported the designated objects to/from the loading areas, moving in a coordinated way.

* * * Next Change * * * *

### 5.x.5 Existing features partly or fully covering the use case functionality

Same as in 5.3.5 of the TR.

Further existing features that partly or fully cover the use case functionality are FFS.

* * * Next Change * * * *

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-001] Based on operators’ policy, the 5G system shall be able to support mechanisms to enable dynamic steering, splitting and switching of UE’s user data for one or more UEs across two 3GPP access networks of two different operators to meet service-specific QoS requirements for these UEs, where each UE’s user data pertains to a single data session.

NOTE 1: It is assumed that each UE is accessing the same PNI-NPN services with the same QoS requirements.

[PR 5.x.6-002] Based on operator’s policy, the 5G system shall be able to support mechanisms to enable dynamic steering, splitting and switching of UE’s user data for one or more UEs such that specific service flow(s) use the same access network for all the UEs.

NOTE 2: The above requirements assume a proper business agreement is in place between the network operators, including negotiation of specific traffic routing policies and rules.