3GPP TSG SA WG 1 Meeting #102 	S1-231054

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Xiaomi, Inspur

Title:	Discussion on Low Power High Accuracy Positioning (LPHAP)

Agenda item:	3

Document for:	Discussion

Contact:	Chunhui Zhu, zhuchunhui@xiaomi.com

Abstract: The LPHAP requirements of the relationship between Low Power and High Accuracy Positioning need to be clarified.

1. Introduction

In Rel-18, the LPHAP(Low Power High Accuracy Positioning) requirements are introduced in TS 22.261 and TS 22.104.

During the Stage 2 work, SA2 sends a LS (S2-2303414) to SA1 to ask for the clarification about LPHAP:

In the study phase of release 18 eLCS_Ph3 work item, SA2 discusses the key issue “support of low power and/or high accuracy positioning” and concludes the solution only targeting the aspect of “low power and high accuracy”. However, in the normative phase (February meeting) discussion, some companies proposed to consider another aspect of LPHAP, i.e. “low power or high accuracy” positioning.

SA2 kindly ask RAN1 and RAN2:

whether the study on “low power or high accuracy” positioning is in the release 18 RAN working scope. If yes, SA2 would like to ask RAN1 and RAN2 to provide the feedback on the architectural impact.

SA2 kindly ask SA1:

whether SA1 specifies the requirement on “low power or high accuracy” positioning in release 18.

2. Discussion

In TS22.104 v18.3.0, Table 5.7.1-1: Positioning performance requirements does not include Service Level 1 (Horizontal accuracy is 10m), since Service Level 1 is not regarded as high accuracy positioning.

5.7.1	General requirements

High accuracy positioning is becoming essential for Factories of the Future. The reason for this is that tracking of mobile devices as well as mobile assets is becoming increasingly important in improving processes and increasing flexibility in industrial environments.

The 5G system shall provide positioning information for a UE that is out of coverage of the network, with accuracy of < [1 m] relative to other UEs that are in proximity and in coverage of the network.

Table 5.7.1-1 below lists typical scenarios and the corresponding high positioning requirements for horizontal and vertical accuracy, availability, heading, latency, and UE speed.

NOTE:	The column on "Corresponding Positioning Service Level in TS 22.261" maps the scenarios listed in Table 5.7.1-1 to the service levels defined in TS 22.261 [2].

Table 5.7.1-1: Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

In TS22.104 v18.3.0, Table A.7.2-1: Low power high accuracy positioning use cases has 9 use cases. The Horizontal accuracy of Use Case 1 and 9 are 10m (Service Level 1), which is not for high accuracy positioning, but can be regarded as Low power positioning.

A.7.2	Low Power High Accuracy Positioning

Low power high accuracy positioning is an integral part of a considerable number of industrial applications. The total energy needed for a specific operation time for such a low power high accuracy positioning optimized IoT-device is a combination of energy for positioning (varies depending on the used positioning method), energy for communication/‌synchronization and a difficult to predict factor to take additional losses through e.g. security, power management, microcontroller, and self-discharge of batteries into account.

Examples of target applications for low power high accuracy positioning are asset tracking in process automation, tracking of vehicles, and tool tracking.

Table A.7.2-1 gives an indication of the required operation time of the 5G enabled IoT device and duty cycle of the updated position information for different use cases.

Table A.7.2-1: Low power high accuracy positioning use cases

| Use Case # |  | Horizontal accuracy |  | Corresponding service level (22.261) |  | Positioning interval/ duty cycle |  | battery life time/ minimum operation time |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 10 m |  | Service Level 1 |  | on request |  | 24 months |  |
| 2 |  | 2 m to 3 m |  | Service Level 2 |  | < 4 seconds |  | > 6 months |  |
| 3 |  | < 1 m |  | Service Level 3 |  | no indication |  | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |  |
| 4 |  | < 1 m |  | Service Level 3 |  | 1 second |  | 6 - 8 years |  |
| 5 |  | < 1 m |  | Service Level 3 |  | 5 seconds - 15 minutes |  | 18 months |  |
| 6 |  | < 1 m |  | Service Level 3 |  | 15 s to 30 s |  | 6 - 12 months |  |
| 7 |  | 30 cm |  | Service Level 5 |  | 250 ms |  | 18 months |  |
| 8 |  | 30 cm |  | Service Level 5 |  | 1 second |  | 6 - 8 years (no strong limitation in battery size) |  |
| 9 |  | 10 m |  | Service Level 1 |  | 20 minutes |  | 12 years (@20mJ/position fix) |  |

Use case one

Process automation: Dolly tracking (outdoor).

Use case two

Process automation: Asset tracking.

Use case three

Flexible modulare assembly area: Tool tracking in flexible, modular assembly areas in smart factories.

Use case four

Process automation: Sequence container (Intralogistics).

Use case five

Process automation: Palette tracking (e.g. in turbine construction).

Use case six

Flexible modulare assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse.

Use case seven

Flexible modulare assembly area: Tool assignment (assign tool to vehicles in a production line, left/right) in flexible, modular assembly area in smart factories.

Use case eight

Flexible modulare assembly area: Positioning of autonomous vehicles for monitoring purposes (vehicles in line, distance 1.5 meter).

Use case nine

(Intra-)logistics: Asset tracking

3. Proposal

It is proposed to clarify that LPHAP includes ‘Low Power and High Accuracy Positioning’ and ‘Low Power Positioning’ in TS 22.104 for Rel-18 and Rel-19.