3GPP TSG-SA WG1 Meeting #102 	S1-231106

Berlin, Germany, 22 - 26 May 2023

Title:	CR on mobility between naval non-public networks

Agenda Item:	7.12

Source:	b<>com, Airbus, Novamint

Contact:	Mathieu Lagrange, mathieu.lagrange@b-com.com

Abstract: This change request provides a use case related to supporting mobility between naval non-public networks.

## x.2	Use case Mobility between interconnected naval communication nodes

### x.2.1	Description

This use case deals with self-organizing, broadband, low latency, connectivity solution between ships in a war fleet. It is an extension of the use case x.1 on interconnection of autonomous naval non-public networks.

This network architecture, unusual in the world of public cellular networks, will however have to support all the mobility functions supported by terrestrial networks. In particular, a terminal will have to be able to move freely from one vessel to another, either in standby or while being actively involved in a communication. This movement should not be perceptible to the user.

The situation described in this use case is depicted on the figure below:

### x.2.2	Pre-conditions

User A terminal is loaded with a USIM that is registered to network A and to network B.

Network A has local coverage on-board ship A and network B has coverage extension on-board ship A.

Service B is offered by an application B instantiated on ship B. Application B has 5G connectivity through network B.

User A has needed credentials to access service B.

Network A and Network B broadcast different PLMN-IDs or NIDs or combination of both.

### x.2.3	Service Flows

- User A on-board ship A and attached to the on-board network A, selects network B, performs an initial registration procedure, authenticates itself to network B and establishes a PDU session to access service B.

- User A takes a speed boat, equipped with necessary communication equipment to deploy locally a small tactical cell around the speed boat and to enable connectivity with any vessels in the fleet through 5G links. Once it is deployed, the on-board tactical cell selects the standalone non-public network offering the best radio link quality (network A), identifies and authenticates itself to the federation of networks as a new node offering coverage extension.

- As the speed boat moves away from ship A, radio measurements made by user A terminal trigger the 5G network to handover the ongoing communication from ship A on-board cell to the tactical cell on-board the speed boat.

- At this point, user A terminal, although under radio coverage of the speed boat, keeps being connected to network B through a communication path made of speed boat + ship A + ship D along its trip from ship A to ship B.

- Somewhere along the path from ship A to ship B, as radio conditions on the 5G link with ship A worsen and as the speedboat approaches ship D, the tactical cell on-board the speed boat selects network D as a new anchoring point to the federation of standalone non-public networks. The 5G system then selects a new communication path that goes from the speedboat to ship B through ship D. User A’s communication is seamlessly handed over to this path which is now more suitable to the quality of service required by service B.

- Later on, as the speedboat approaches ship B, the on-board tactical cell directly connects to network B in the same anchoring point selection and path switching process as previously described to reduce communication latency.

- From the point in time user A gets on board the speedboat and gets to ship B, his communication with service B is maintained. There is no noticeable alteration of the communication along the path from ship A to ship B.

- When user A arrives at his destination on ship B, his communication is seamlessly handed over from the cell on-board the speedboat to one of the cells on ship B. On ship B, he can still access to service B and other local services according to authorizations he’s entrusted with.

### x.2.4	Post-conditions

User A is attached to network B and still in communication with service B.

### x.2.5	Existing features partly or fully covering the use case functionality

Editor’s note : section left for further study

### x.2.6	Potential New Requirements needed to support the use case

[PR.5.x.6-004]: The 5G system shall be able to support discovery, addition and authentication, or detachment of standalone non-public networks in an interconnected topology through 5G links without interrupting ongoing communications.

[PR.5.x.6-002]: The 5G system shall be able to support measurements allowing to seamlessly handover a standalone non-public network from one standalone non-public network anchor point to another one.

[PR.5.x.6-003]: The 5G system shall be able support seamless handover of communications between collocated standalone non-public networks.

[PR.5.x.6-003]: The 5G system shall be able to support hot modification of the list of neighbouring cells signalled to the UEs in the broadcast channel.

Editor’s note : Performance requirements are left for further study