3GPP TSG-SA WG1 Meeting #102 	S1-231168

Berlin, Germany, 22 - 26 May 2023	(revision of S1-230xxx)

Title:	Consolidated KPI table

Agenda Item:	7.1

Source:	vivo

Contact:	Amy Zhang, amy.zhang@vivo.com

Abstract: this text provides consolidated KPI table for section 7.

There are 32 use cases. Among which, there are 21 use cases having KPI tables, 8 use cases having no KPI table, 3 non-3GPP related use cases.

| Use cases | KPI? | Main affecting factor (sensing target) | Applied scenario | Application |
| --- | --- | --- | --- | --- |
| 5.1 intruder detection at home | Yes | People | Smart home | Detection |
| 5.2 intruder detection at highway | Yes | People/animal | Smart city | Detection |
| 5.3 rainfall monitoring | Yes | Rainfall | Smart city | Monitoring |
| 5.4 transparent sensing | No |  |  |  |
| 5.5 flood monitoring | Yes | Water | Public safety | Monitoring |
| 5.6 intruder detection at yard | Yes | People/animal | Smart home | Detection |
| 5.7 intruder detection at railway | Yes | People/animal | Static city | Detection |
| 5.8 Automotive Maneuvering | No | Car | Static city | Collision avoidance |
| 5.9 AGV tracking | No | AGV | Smart factory | Positioning/tracking |
| 5.10 UAV trajectory tracing | Yes | UAV | Smart city | Positioning/tracking |
| 5.11 collision avoidance at crossroad | Yes | People/bicycle | Smart city | Collision avoidance |
| 5.12 UAV collision avoidance | Yes | UAV | Smart city | Collision avoidance |
| 5.13 UAV intruder detection | Yes | UAV | Smart city | Detection |
| 5.14 tourist spot monitoring | Yes | People | Smart city | Monitoring |
| 5.15 sleep monitoring | Yes | People | Smart home | Monitoring |
| 5.16 intruder detection – security part | No | People | Smart home | Detection |
| 5.17 health monitoring by coordinate sensing | No | People | Smart home | Monitoring |
| 5.18 health monitoring with service continuity | No | People | Smart home | Monitoring |
| 5.19 sensor group | No |  |  |  |
| 5.20 parking lot detection | Yes | Car | Smart city | Detection |
| 5.21 seamless XR streaming | No |  |  |  |
| 5.22 collision avoidance at smart grid | Yes | People/Car/UAV | Public safety | Collision avoidance |
| 5.23 AMR collision avoidance | Yes | AGV | Smart factory | Collision avoidance |
| 5.24 sport monitoring | Yes | People | Smart home | Monitoring |
| 5.25 human trajectory tracking | Yes | People | Smart home | Positioning/tracking |
| 5.26 Accurate sensing | No | Car | Smart city | Collision avoidance |
| 5.27 positioning for public safety | Yes | People | Public safety | Positioning/tracking |
| 5.28 ADAS | Yes | Car | Smart city | Collision avoidance |
| 5.29 Gesture recognition | Yes | Hand | Smart home | Detection |
| 5.30 Automotive maneuvering out of coverage | No | Car | Smart city | Collision avoidance |
| 5.31 Blind spot detection | No | Car | Smart city | Collision avoidance |
| 5.32 positioning for smart factory | Yes | AGV | Smart factory | Positioning/tracking |

We may have several methods to merge scenarios:

Category #1): to categorize them from the applied scenario’s perspective, then we may have:

- Smart city

- Smart home

- Smart factory

- Public safety

Reason for this categorization is because the applied scenario shares similar affecting factors, e.g. in smart home the affecting factor is mainly human, in smart factory the affecting factor is mainly AGV, in smart city affecting factor can be car, UAV and passenger which can be easily grouped.

Category #2): to categorize them from the application’s perspective, then we have:

- Detection application

- Monitoring application

- Collision avoidance application

- Tracking/positioning application

Reason for this categorization is because different application may focus on different KPI parameters, e.g. detection application focuses more on “missed detection” and “false alarm” that needs the event to be defined, monitoring application focused more on scenario-oriented KPI parameters (rainfall monitoring needs rainfall estimation accuracy, sleep monitoring needs respiration rare accuracy), tracking/positioning application focuses more on how to utilize sensing technology to do positioning by lifting the restriction on positioning target UE. Collision avoidance application and positioning application both focus on “positioning/velocity accuracy” and “resolution of positioning/velocity” in general KPI table, but collision avoidance calls for more stringent accuracy than positioning.

Then we have several options to generate our consolidated KPI table:

- Option 1): One table containing 21 sub-scenarios without merging

- Option 2): Group them into several sub-tables without merging

- Option 3): Group them into several sub-tables with merging, but based on the use case proposers’ agreement.

It is proposed to categorize KPIs based on Option 3) based on categorization #1)

# Y	Consolidated Potential KPIs

Editor’s Note: This clause only lists 3GPP wireless sensing. Whether non-3GPP sensing service needs KPI is FFS.

## y.1	smart city

Editor’s Note:

- Use case 5.2 and 5.7 proposers, is that possible to merge them?

- Use case 5.10, 5.12, 5.13 proposers, is that possible to merge them?

- Use case 5.11, 5.20, 5.28 proposers, is that possible to merge them?

Table y.1-1	Performance requirements of sensing results for smart city scenarios

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 5.3 Rainfall monitoring (NOTE 1) | Outdoor | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 1 min | 10min, application configurable | 5 | 5 |
| 5.2 Intrusion detection on a highway | Outdoor (Highway) | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤5000 | ≤ 0.1 | ≤5 | ≤5 |
| 5.7 Intrusion detection on a railway | Outdoor (Along railway) | 95 | ≤1.5 | N/A | N/A | N/A | N/A | N/A | ˂1500 | ≤ 0.1 | 2 | 2 |
| 5.10 UAV flight trajectory tracing | Outdoor | N/A | N/A | N/A | N/A | N/A | 1m x 1m ~10m x 10m NOTE 2 | 1m/s x 1m/s ~ 10m/s x 10m/s NOTE 3 | 100~1000 NOTE 4 | 1Hz NOTE 5 | 5 | 5 |
| 5.12 Network assisted sensing to avoid UAV collision | Outdoor | [99] | 1 | N/A | 1 NOTE 2 | N/A | <1 NOTE 2 | 1 | 500 | 0.5 | N/A | N/A |
| 5.13 UAV intrusion detection (Level 1) | Outdoor | 95 | ≤10 | ≤10 | N/A | N/A | 10 | [5] | [≤1000] | [≤1] | ≤5 | ≤5 |
| 5.13 UAV intrusion detection (Level 2) | Outdoor | 95 | ≤5 | ≤5 | N/A | N/A | 10 | [5] | [≤1000] | [≤1] | ≤5 | ≤5 |
| 5.14 Tourist spot traffic management | Outdoor | 95 | [≤2] | N/A | N/A | N/A | [1] | [1] | [≤5000] | [≤0.2] | ≤5 NOTE 7 | ≤5 NOTE 7 |
| 5.11 Sensing at crossroads with/without obstacle | Outdoor | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤100 NOTE 8 | ≤ 0.1 | ≤5 | ≤5 |
| 5.20 Parking space determination | Indoor/ outdoor | 95 | 0.5 | 0.5 | 0.1 | N/A | 2.5m perpendicular to the parking space 5m parallel to the parking space | N/A | 1000 | 1 | 1 | 5 |
| 5.28 ADAS [long range Radar] | TBD | [95] | ~[0.1]-[1.3] NOTE 9 | TBD | [±0.03] -[±0.12] m/s NOTE 11 | N/A | [0.4] NOTE 12 | [0.1] –[0.6] NOTE 11 | [50] | [0.05] -[0.2] | [1-10]% | [<1]% |
| 5.28 ADAS [Short range Radar] | TBD | [95] | ~[0.02]-[2.6] NOTE 10 | TBD | [±0.03] -[±0.12] m/s NOTE 11 | N/A | [0.4] NOTE 12 | [0.1] –[0.6] NOTE 11 | [20] | [0.02] –[0.05] | [1-10]% | [<1]% |
| NOTE 1: Rainfall estimation accuracy >= 1mm/h is the parameter describing the closeness of the measured rainfall estimation to its true rainfall value [39]. NOTE 2: To detect the UAV existence (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 4: To realize 1m granularity tracking, when the velocity resolution is 1~10m/s, the maximum corresponding sensing service latency is 0.1~1s. NOTE 5: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 6: The KPI values are sourced from [25] and [40]. NOTE 7: Missed detection or false alarm describes missing to acquire a sensing result or acquiring a wrong sensing result which referring to a target object (a person or a vehicle), in this use case will be missing detect a person or a vehicle, not referring to the number of a crowd of people or vehicles. NOTE 8: The value is sourced from [28]. NOTE 9: Assuming typical max range of 250m, range accuracy of 10cm, azimuth accuracy of ±0.3deg. Positioning accuracy as (Min-Max) within field of view. NOTE 10: Assuming typical max range of 30m, range accuracy of 2cm, azimuth accuracy of ±5deg.Positioning accuracy as (Min-Max) within field of view. NOTE 11: Velocity accuracy and resolution is typically reported for the radial velocity (not absolute H/V velocity). NOTE 12: Range resolution typically reported as 3D ranging distance accuracy. |  |  |  |  |  |  |  |  |  |  |  |  |

## y.2	smart home

Editor’s Note:

- Use case 5.1 and 5.6 proposers, is that possible to merge them?

Table y.2-1	Performance requirements of sensing results for smart home scenarios

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 5.1 Intruder detection in smart home | Indoor | 95 | ≤10 | ≤10 | N/A | N/A | N/A | N/A | <1000 | < 1 | < 5 | < 2 |
| 5.6 intruder detection in surroundings of smart home | Outdoor | 95 | ≤2 | N/A | N/A | N/A | N/A | N/A | ≤1000 | < 1 | < 0.1 | < 5 |
| 5.15 Sleep monitoring (NOTE 1) | Indoor (bedroom) | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 60 | 5 NOTE 2 | 5 NOTE 2 |
| 5.24 Sports monitoring (NOTE 3) | Indoor (living room) | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 1min | N/A | N/A |
| NOTE 1: It monitors people’s respiration, which requires Respiration rate accuracy = 2 times/min (taking 18 times/min as reference), i.e. any detected value in [16,20] satisfies accuracy requirements. NOTE 2: Detect event = “breathing stoppages duration >= 10 seconds” is taken as reference, NOTE 3: Sport can be sit-up. Then it requires Sit-up rate accuracy = 3 times/min (taking 30 times/min as a reference), i.e. any detected value in [27,33] satisfies accuracy requirements. Sport can be push-up. Then it requires Push-up rate accuracy = 4 times/min (taking 40 times/min as a reference), i.e. any detected value in [36,44] satisfies accuracy requirements. |  |  |  |  |  |  |  |  |  |  |  |  |

## y.3	smart factory

Editor’s Note:

- Use case 5.23 and 5.32 proposers, is that possible to merge them?

Table y.3-1	Performance requirements of sensing results for smart factory scenarios

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 5.23 AMR collision avoidance in smart factories (NOTE 1) | 100m | 99 | ≤1 | N/A | 1 | N/A | 1 | 10 | ˂500 | ≥20Hz | N/A | 5 |
| 5.32 Indoor Factory | 100 m2 | 99 | [≤0.5] | N/A | 0.5 | N/A | [0.5] | [0.5] | ≤100 | 0.1 | N/A | N/A |
| NOTE 1: The KPI values are sourced from [47] |  |  |  |  |  |  |  |  |  |  |  |  |

## y.4	public safety

Table y.4-1	Performance requirements of sensing results for public safety scenarios

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| 5.5 sensing for flooding in smart cities | Outdoor | 95 | ≤10 | [≤0.2] NOTE 1 | N/A | N/A | N/A | N/A | ≤ 1min NOTE 2 | < 1min NOTE 2 | < 0.1 | < 3 |
| 5.22 Sensing for the use case in Smart Grid NOTE 3 | Outdoor | 95 | ≤0.7 | N/A | UAV: ≤25 Pedestrian: ≤1.5 Vehicle: ≤15 | N/A | N/A | N/A | ≤5s | ≥10Hz | [≤5] | [≤5] |
| 5.27 Search and Rescue/Apprehend | Outdoor/Indoor | 99 | ≤ 0.5 | ≤ 1.0 | Pedestrian: ≤1.5 | Pedestrian: ≤1.5 | 3 | Horiz: 5 Vert: 5 | ≤1s | ≥10Hz | [≤3] | [≤3] |
| NOTE 1: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. NOTE 2: Description related to NOTE in clause 5.5.1 suggests 2 minute-interval monitoring when the water level of the river rises quickly. NOTE 3: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. The size of the park of Smart Grid depends on the real environment. The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. |  |  |  |  |  |  |  |  |  |  |  |  |