3GPP TSG SA WG 1 Meeting #102 	S1-231298

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Huawei

pCR Title:	Pseudo-CR on FS_Ambient IoT, update Max. allowed end-to-end latency KPI of clause 5.5 intralogistics in automobile manufacturing

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang ZHANG (zhang.shuang2@huawei.com)

Abstract: This document proposes to update max allowed latency KPI to clause 5.5 intralogistics in automobile manufacturing in TR 22.840 v.1.1.0.

1. Introduction

Clause 5.5 has been agreed and captured in TR 22.840, which introduces a communication service that enables highly-efficient intralogistics for automobile manufacturing by realizing Ambient IoT-enabled automatic inventory.

Currently, the KPI value of “Max. allowed end-to-end latency” is captured as “>100 ms” in the KPI Table 5.5.6-1 with note 1 stating:

“This value corresponds to peak reading rate of 100 tags per second. The average tag reading rate is lower.”

2. Reason for Change

This intends to capture the typical reading rate of Ambient IoT devices by the “reader”, however, it should be clarified that deriving Max. allowed end-to-end latency from reading rate is assuming one specific solution.

For inventory application, the time it takes for inventory results to be deliverd to the inventory user/destination is not time-critical. Typically it is in the order of several seconds.

To specify a more appropriate value of “Max. allowed end-to-end latency” for intended inventory service, the value need to be updated to “typically >1 second”.

3. Conclusions

Update KPI Table 5.5.6-1 by updating the value of Max. allowed end-to-end latency from “hundreds ms level” to “typically >1 second”.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

### 5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] The 5G system shall support communication for an Ambient IoT device which is battery-less or with limited energy storage capability.

[PR 5.5.6-002] The 5G system shall support collection of charging information based on different charging policies for Ambient-IoT type of communication, i.e., total number of communication (e.g. data payload) per charging period, or total number of Ambient IoT devices per charging period.

Editor’s Note: 	Additional requirements for this use case are FFS.

[PR 5.5.6-003] The 5G system shall provide the network connection to address the KPIs for the use of Ambient IoT devices for intralogistics in automobile manufacturing, see table 5.3.6.1-1.

Table 5.5.6-1: Potential key performance requirements for the use of Ambient IoT devices for intralogistics in automobile manufacturing

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Automatic Intralogistics in automobile manufacturing | Typically >1 second (note 1) | 99% | NA | <1 kbit/s (note 2) | 96 bits (note 3) | <1,5 Million/km2 (note 4) | <30 meters Indoors | 600 000 m2 (note 5) | NA | NA | NA | NA | [3] m |
| NOTE 1: peak reading rate 100 tags per second. The average tag reading rate is lower. However, tag reading rate is a different aspect from end-to-end latency. As inventory service is not time-critical, the second level latency is appropriate. NOTE 2: This value is calculated as the instant data rate for transmitting 96 bits within 100 ms time period. The need for data transmission is infrequent. NOTE 3: EPC Tag Data standard [5], the length of the EPC number ranges from 96 bits to 496 bits. For intralogistics, EPC length of 96 bits is the most common EPC lengths to satisfy the use case. NOTE 4: Daily around 1 million units of materials are used in the manufacturing area, but they are not used at the same time. NOTE 5: A typical car manufacturing plant takes up to 600 000 m2 in surface. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of the First Change * * * *