3GPP TSG SA WG 1 Meeting #102 	S1-231300

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Huawei,Wiliot

pCR Title:	Pseudo-CR on FS_Ambient IoT, update Transfer Interval KPI table of clause 5.18 Fresh Food Supply Chain

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang ZHANG (zhang.shuang2@huawei.com)

Abstract: This document proposes to add transfer interval KPI to clause 5.18 Fresh Food Supply Chain in TR 22.840 v.1.1.0.

1. Introduction

Clause 5.18 has been agreed and captured in TR 22.840, which introduces a communication service that enables large food suppliers to effectively monitor food supply chain using Ambient IoT. The periodical minitoring takes place throughout the use cycle of Reusable Transport Item (RTI) covering different stages of storage (e.g. fresh produce distribution centre) and transportation. Both in the use case description and in NOTE 1 of KPI Table 5.18.6-1 the per 15 minutes monitoring is mentioned, however the value of transfer interval is missing. Also, Device Speed value is missing.

2. Reason for Change

The value of Transfer interval needs to be added according to the captured NOTE 1 in KPI table to clarify the important periodical monitoring characteristics of the proposed communication service. Also, the RTIs are mobile, the typical speed is 1m/s.

3. Conclusions

Add Transfer Interval and Device Speed values into KPI Table 5.18.6-1.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

### 5.18.6	Potential new requirements needed to support the use case

[PR 5.18.6-1] Based on operator policy, the 5G system shall provide means for an authorised third party to poll a group of multiple ambient IoT devices.

[PR 5.18.6.-2] The 5G system shall be able to provide ambient IoT service with following KPIs

Table 5.18.6-1: Potential key performance requirements for the use of Ambient IoT devices for food supply chain

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for food supply chain | >1 minute | NA | NA | <0.12 bit/s (Note 1) | Typically, < 100 bits (Note 2) | 1.5 Million devices/ km2 (Note 3) | NA | 30,000m2 | 1 m/s | 15 min (Note 1) | NA | NA | NA |
| NOTE 1: Based on sending 1 message of 100 bits once in 15 minutes NOTE 2: If more sensors are used, like humidity or ethylene level, then longer message is required. NOTE 3: This is the highest density inside the distribution center and is based on 50,000 RTIs inside a 30,000m2 distribution center facility. See Figure 5.18.1-2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of the First Change * * * *