3GPP TSG-SA WG1 Meeting #102 	S1-231317r1

Berlin, Germany, 22 - 26 May 2023	(revision of S1-230290)

Title:	Discussion on consolidated requirements of FS_Sensing

Agenda Item:	7.1

Source:	Deutsche Telekom, ZTE

Contact:	Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: This document proposes consolidation on the potential requirements from the use cases of FS_Sensing

### The following potential requirements still have editor’s note and are excluded from proposed consolidation:

[P.R.5.1.6-1] The 5G system shall provide a mechanism for an operator to authorize a UE for sensing, e.g., based on location.

Editor's Note:	Terminology of UE/base station needs to be adapted.

[PR 5.30.6-3] The 5G system shall be able to provide the continuity of V2X 5G wireless sensing between when the UE is served by RAN and when the UE is not served by RAN.

Editor’s note: Requirement [PR 5.30.6-3] is FFS.

### There are various requirements in TR 22.837 v1.0.0 (2023-03) and it is proposed to split them into the following categories:

- Sensing configuration

- 5G wireless sensing service

- Exposure

- Security

- Charging

### Sensing configuration

| Potential Requirements captured in the TR | Comments | Consolidated potential requirements |
| --- | --- | --- |
| [PR 5.2.6-3] The 5G system shall be able to support means to configure the sensing operation of a base station (e.g. authorization, sensing activation and/or deactivation, sensing duration, sensing accuracy, target sensing location area). [PR.5.15.6-1] The 5G system shall support mechanisms to discover and configure a UE and a base station to perform sensing measurements in a certain sensing service location area. [PR 5.9.6-2] Based on operator policy and location area, the 5G system shall be able to provide means to support per-UE authorization for NR-based sensing. [PR 5.20.6-2] The 5G system shall be able to authorize Sensing receiver(s) and Sensing transmitter(s) to participate in a sensing service. [PR 5.8.6-1] The 5G system shall be able to support mechanisms to control UEs and RAN entities for a sensing service. NOTE1: In the requirement above, control can include configuration such as sensing specific policies and settings (e.g., conditions for triggering sensing requests, location, etc.) coordinated amongst UE and RAN entities. [PR 5.8.6-4] Subject to user consent and regulations, based on operator policy, the 5G system shall be able to provide means to authorize and configure a UE for sensing operation (e.g., based on location, time, etc) and for establishing the communication connection needed to assist the sensing service. NOTE 3: The above requirement assumes that the communication connection used for assisting sensing service (e.g. for transferring 3GPP sensing data or sensing results) can include existing communication connection modes such as direct network communication, direct device connection under network coverage and indirect network connection [33]. [PR 5.28.6-1] The 5G system shall be able to configure and authorize UEs supporting V2X applications to perform sensing. | Sensing receiver and sensing transmitter can be used as there is lack of specific difference in existing requirements on configuration and authorization between UEs and base stations. The discovery part of PR.5.15.6-1 seem to be covered in [CPR 7.1.1-25]. PR 5.8.6-1 – is not clear how control is different from configuration and authorization. Some further clarification would be needed if there is a need for separate requirement. PR 5.8.6-4 comes from V2X use case and is also about configuration using the same principle as the other requirements. If there is a need for more specific configuration of V2X UEs, it has to be clearly stated as separate requirement (for example when V2X UE is not served by RAN). If the only purpose is to transfer the 3GPP sensing data or sensing results – then this seems to be covered in [CPR 7.1.1-8]. | [CPR 7.1.1-1] The 5G system shall be able to configure, reconfigure and authorize sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service in certain target sensing service area. NOTE: Such configuration can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc. |
| [PR 5.25.6-1] The 5G system shall be able to configure and authorize sensing for a Sensing device or a group of Sensing devices when using licensed spectrum based on the Sensing device’s location. | The configuration part of [PR 5.20.6-1 (licensed)] (The 5G system shall be able to provide sensing services in licensed and unlicensed spectrum.) and [PR 5.25.6-2] (The 5G system shall be able to enable a Sensing device to perform sensing with licensed band under operator’s control based on the Sensing device’s location.) is merged into [CPR 7.1.1-2]. Therefore additional wording to include the base station aspect and “under operator’s control”. These PRs are also listed bellow. | [CPR 7.1.1-2] The 5G system shall be able to configure and authorize sensing transmitters and sensing receivers for using licensed spectrum based on location and under operator`s control. |
| [PR 5.11.6-4] The 5G system shall support means for a trusted 3rd party application, e.g. a map service provider to configure sensing per location. [P.R 5.13.6-8] The 5G system shall be able to provide a mechanism for network operator to configure and adjust sensing operation (e.g. authorization, sensing area, sensing operation period and sensing operation time window etc.) based on request from a trusted 3rd party. | Do we need a definition of sesing operation? – No, as we have agreed to use sensing service. We already have definition of 5G wireless sensing. Qualcomm’s proposal: Based on request from trusted 3rd party, the 5G system shall be able to "dynamically configure and authorize the sensing operation”. DT: This requirement can be potentially combined with CPR 7.1.1-1. |  |
| [PR 5.30.6-1] The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing operation when not served by RAN. | Phillips, Futurewey, Xiaomi, Qualcomm V2x can be a bit limiting. Ericsson thinks is needed. DT also thinks it is needed. Would it be possible to receive details on a use case where other applications would use sensing outside of network coverage? What exactly would be configured different then spectrum? There seem to be regulatory implications. | [CPR 7.1.1-4] The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing service when not served by RAN. |
| [PR 5.5.6-3] Subject to operator policy, it shall be possible for an authorized third party to configure the 5G system to initiate sensing for disasters or other emergencies (e.g., flooding) in a given geographic area. | This can be combined with the configuration requirement CPR 7.1.1-1. Better not to use disasters – need to be more specific. This may be used in the requirement for prioritization of the sensing service – prioritization for emergency services. There is a need for listing examples of a sensing service. Can be combined with CPR-7.1.1-5. |  |

### 5G wireless sensing service

| Potential Requirements captured in the TR | Comments | Consolidated potential requirements |
| --- | --- | --- |
| [P.R.5.1.6-4] The 5G system shall support UE to perform sensing measurement process using signals received from other UE(s). [PR 5.2.6-1] The 5G system shall be able to support a base station to perform sensing. [PR 5.9.6-1] The 5G system shall be able to provide means to support NR-based sensing in a certain area or location. [PR 5.11.6-1] The 5G system shall be able to support a mechanism to provide available sensing service in a sensing service area location. [P.R 5.14.6-1] The 5G system shall be able to provide means to use base station(s) to perform sensing in certain area. [P.R.5.1.6-2] The 5G system shall support a UE to perform sensing measurement process based on the trusted 3rd party’s request. [PR 5.20.6-1] The 5G system shall be able to provide sensing services in licensed and unlicensed spectrum. | Proposal to consolidate all these requirements into single one. [P.R.5.1.6-2] – 3rd party request is covered in [CPR 7.1.1-25]. Qualcomm proposal: 1. Remove PR.5.1.6-4 (in which section it should go?) 2. Add PR 5.17.6-1and PR 5.20.6-1 3. CPR - Based on regulations, operator policy, the 5G system shall be able to support sensing service using sensing transmitter and receivers. DT: It is not clear why PR 5.17.6-1 (coordination between RAN entities and UEs should not be sensing group specific). It is not clear what is the meaning of “shall be able to support” in QC’s proposal? 5GS is providing the service. Huawei: the note might not be needed. Futureway, Xiaomi, Vivo believes sensing operation and service are different. We will use sensing service… and also in the header. Specific requirements will be discussed. | [CPR 7.1.1-5] The 5G system shall be able to provide 5G wireless sensing service in a sensing service area location using sensing transmitters and sensing receivers. |
| [P.R 5.14.6-3] Subject to regulatory requirements and operator policy, the 5G system shall be able to support the activation and deactivation of the sensing service based on location. [P.R 5.13.6-7] The 5G system shall be able to support the activation and deactivation of the sensing service according to operator’s policy. [PR 5.8.6-6] The 5G system shall be able to provide means for the 5G network to activate and/or deactivate sensing service in the target sensing area based on network conditions (e.g., network-load). | These two requirements can be combined. Operator’s policy seems to be covered in [CPR 7.1.1-1]. Qualcomm’s: The 5G system shall be able to provide means for the 5G network to activate and/or deactivate a sensing service based on operator policy, location or network conditions (e.g., network-load). Futurwey, other conditions apart from location and network conditions… Telefonica: adapt --> dinamically configure? | [CPR 7.1.1-6] Subject to regulation and operator policy, the 5G network shall be able to activate, dynamically reconfigure and deactivate 5G wireless sensing based on location and network conditions (e.g. network load). |
| [PR 5.17.6-2] The 5G system shall support a mechanism for the 5G network to retrieve the wireless sensing capabilities from UEs and RAN entities, and for the UEs and RAN entities to exchange capabilities amongst each other. | Exchanging capabilities among sensing transmitters and sensing receivers seem to be included in the CPR 7.1.1-25. Qualcomm: combined with… [PR 5.8.6-3] -> The 5G system shall be able to support the discovery and exchange of 5G wireless sensing capabilities of sensing transmitters and receivers |  |
| [PR 5.2.6-4] The 5G system shall be able to support means to enable a base station to transfer sensing measurement data to the core network. [PR. 5.3.6-1] The 5G system shall support collection of the NR based sensing measurement data from the base station. [PR. 5.6.6-1] Subject to operator policy, the 5G system shall be able to collect sensing measurement data and yield sensing result from the data for detection of outdoor objects. [PR. 5.7.6-1] Subject to operator policy, the 5G system shall enable the core network to collect and aggregate sensing measurement data from base stations. [PR 5.8.6-2] For a sensing service, the 5G system shall be able to support mechanisms for the UEs and RAN entities to provide 3GPP sensing data. NOTE2: This requirement can cover scenarios making use of information already available in the EPC and E-UTRA (assuming no new functionalities are required in the EPC and E-UTRA). [PR 5.9.6-3] The 5G system shall be able to support means to enable RAN entities and UEs to transfer sensing measurement data to sensing processing entities in the 5G system responsible for processing and aggregation of the sensing measurement data. NOTE2: The “Sensing processing entities” in the above requirement refer to one or more entities in the 5G system responsible for aggregating and processing of sensing measurement data (e.g., core network). [PR 5.11.6-2] The 5G base station shall be able to collect sensing measurement data from requested sensing service area location according to the operator’s policy. NOTE 1: The operator policy means to configure the sensing service area location, real time sensing measurement data collection or periodic collection etc. [P.R 5.13.6-3] The 5G system shall provide mechanisms for an operator to transport sensing data from base station towards the core network. [PR 5.17.6-4] The 5G system shall support a mechanism to provide wireless sensing capable UEs and RAN entities with information of which network entity or destination server to send the sensing measurement data to. [PR 5.2.6-5] The 5G system shall be able to support means to enable the core network to process sensing measurement data for obtaining sensing results. [PR. 5.3.6-2] Based on operator’s policy, the 5G system shall support mechanisms to process the sensing measurement data to derive the sensing results. [P.R 5.13.6-1] The 5G system shall be able to provide a sensing service by using base stations to collect sensing measurements. [PR 5.21.6-1] Subject to user consent and regulatory requirements, based on operator policy, the 5G system shall be able to support secure means for RAN entities and authorized UEs to provide 3GPP sensing data to a 5G network for processing. | There are many requirements for collecting, transporting sensing measurements data. The proposal is to summarize them into [CPR 7.1.1-8]. [PR 5.17.6-4] seem to be a solution-oriented requirement as it clarifies that the sensing transmitter or receiver has to have an address to where to send the info. It would be also possible to have the network element knowing the address of the sensing entity and connecting to retrieve the data. It is more beneficial for the network to control this process of collecting, that is why it is consolidated into network centric approach. Terminology of sensing result must be aligned. Huawei suggest to remove note 1 as it applies to all requirements and make this req. general. If we use 5G wireless sensing in requirements, it seems the note can be removed. | [CPR 7.1.1-8] The 5G system shall be able to collect when needed 3GPP sensing data from sensing receivers and securely provide it to 5G network for processing and deriving sensing results. |
| [PR.5.4.6-1] Subject to user consent and national or regional regulatory requirements, based on operator policy, the 5GS shall support a mechanism to receive uplink non-3GPP sensing data from authorized non-3GPP sensors. NOTE 1: This requirement assumes there is some functionality in the 5GS to discern and interpret the acquired 3GPP and non-3GPP sensing data. [PR 5.21.6-2] Subject to user consent and regulatory requirements, based on operator policy, the 5G system shall be able to collect non-3GPP sensing data from trusted parties. | Samsung: we are missing that the sensor must be authorized. Qualcomm’s proposal: [CPR 7.1.2-15] Subject to user consent, regulation, based on operator’s policy, the 5G system shall be able to support secure mechanism to collect non-3GPP sensing data from trusted parties and provide data to 5G network for processing. Note: The trusted parties can be UE, RAN or trusted 3rd party service provider DT, Samsung: It seems UEs cannot be trusted, how RAN could have non-3GPP sensors? | [CPR 7.1.1-18] Subject to user consent, regulation and operator’s policy, the 5G system shall be able to collect when needed non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network for processing. |
|  | Fix terminology. Proposed to be merged with exposure requirement- CPR 7.1.2-8. |  |
| [P.R 5.10.6-2] The 5G system shall be able to support means to authorize RAN entities and UEs in certain location area generating and reporting sensing measurement data (e.g., related to a UAV position, velocity) to a 5G sensing processing entity. NOTE: The requirement above assumes that the sensing measurement data is post-processed in 5G sensing processing entity which is located within the 5G system. | It seems authorization part is covered in [CPR 7.1.1-1]. Generating and reporting seems to be covered in [CPR 7.1.1-8]. |  |
| [P.R 5.10.6-4] The 5G system shall support energy efficient sensing operations. | Qualcomm will propose a rewording. ZTE and Xiaomi think is needed. |  |
| [PR 5.12.6-1] The 5G system shall be able to provide a sensing service to track one specific target object and the environment around the target object with the assistant information provided by the UE on board the specific target object or authorized 3rd party. NOTE 1: The assistant information for example can be UE position, UE ID. | Ericsson: rewording needed, it needs to be more general. Vivo PR 5.32.6-1 can be combined here. Lenovo think these requirements are different and both needed. It looks like sensing service shall be provided always with assistance information – would be better to use “may” as it is proposed to remove CPR 7.1.1-12. | [CPR 7.1.1-11] The 5G system may provide a sensing service to track one specific target object and the environment around the target object with the assistance information provided by the UE on-board the specific target object or authorized third-party. NOTE 1: The assistance information for example can be UE position, UE ID. |
| [P.R 5.13.6-2] The base station shall be able to sense a target object by obtaining sensing measurement without active involvement of the target object. | This should be the default operation of the sensing service. Ericsson: this req. is not needed. Vivo agrees. | [ |
| [PR 5.17.6-3] The 5G system shall support a mechanism for two or more authorized UEs and/or RAN entities to take part in the wireless sensing of a target, whereby the authorization may be provided based on location. | Authorization part seems to be covered in [CPR 7.1.1-1] and operation in [CPR 7.1.1-8]. |  |
| [PR 5.23.6 -1] The 5G system shall be able to provide the continuity of sensing service for a specific target object, across indoor and outdoor. [PR 5.18.6.1] The 5G system shall support continuity of sensing of a target that may move across a sensing area that may be bigger than the coverage area of a single sensing transmitter. | Ericsson: not needed as req. says we should do sensing in an area. Phillips and OPPO, Vivo, Futurewei, ZTE, Lenovo disagree. (rewording may be needed). Service continuity can be used more as example. | [CPR 7.1.1-13] The 5G system shall support continuity for sensing of specific target object moving across sensing service area location bigger than the coverage of a single sensing transmitter, including across indoor and outdoor. |
| [PR 5.31.6-1] The 5G System shall be able to provide sensing service to track a moving sensing target area. | We may need rewording. Definition is already agreed. Vivo and Vodafone work on this. | [CPR 7.1.1-15] The 5G System shall be able to provide 5G wireless sensing service to track a moving sensing target area. |
| [PR 5.24.6-1] Based on operator policy, the 5G System shall be able to provide the 5G wireless sensing services in case of roaming. |  | [CPR 7.1.1-16] Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming. |
| [PR 5.26.6-1] The 5G system shall be able to determine the confidence level of the sensing results. | Vodafone… needs rewording as is too use cases oriented. KPI table is already with confidence lever, (OPPO, is this needed?). DT: Is it the case that confidence level would be something rather static and coming from deployment and capabilities? It seems more explanation would be needed. |  |
| [PR 5.21.6-3] Subject to user consent and regulatory requirements, based on operator policy, the 5G system should be able to support the combination of the 3GPP sensing data and non-3GPP sensing data to derive combined sensing result. [PR 5.4.6-4] Subject to user consent, network operator policy and national or regional regulatory requirements, the 5GS shall support a mechanism to enable Sensing transmitters and Sensing receivers to acquire 3GPP sensing data to capture information about the nearby environment and for this to be combined with Non-3GPP sensing data to produce a combined sensing result. NOTE 2: This requirement does not imply or allow 3GPP sensing data to be exposed to third parites. This data is considered confidential. [PR 5.27.6-2] The 5G system shall support mechanisms for combining 3GPP sensing data and non-3GPP sensing data (e.g., body cameras.) depending on location, availability of non-3GPP sensing data, and public safety applications. | Ericsson, Vodafone, Telefonica prefer to use “may”. Qualcomm, LG, Lenovo, Xiaomi prefers “should be able to”. | [CPR 7.1.1-19] Subject to user consent, regulation and operator’s policy, the 5G system may support the combination of the 3GPP sensing data and non-3GPP sensing data.. |
| [PR 6.2.2-1] Subject to regulation and operator’s policy, 5G system shall provide prioritization among sensing services. | Some rewording may be needed to clarify what entity would use the prioritization. | [CPR 7.1.1-20] Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services. |
| [PR 5.25.6-3] The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to perform sensing. [P.R.5.1.6-5] The 5G system shall support UE to perform sensing measurement process in licensed or unlicensed band. [PR 5.25.6-2] The 5G system shall be able to enable a Sensing device to perform sensing with licensed band under operator’s control based on the Sensing device’s location. | It is proposed to change “enable” with “configure”. It is only the UE than can work in unlicensed spectrum. There is also requirement to operate the UE for unlicensed [CPR 7.1.1-22]. The configuration part of [PR 5.20.6-1 (licensed)] and [PR 5.25.6-2] is merged into [CPR 7.1.1-2]. How to capture the usage of licensed spectrum? It is captured in CPR 7.1.1-22. | [CPR 7.1.1-21] The 5G system shall be able to configure UEs without 5G coverage to use unlicensed spectrum. |
| [PR 5.30.6-2] Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum. | Unlicensed spectrum seem to be covered in [CPR 7.1.1-21]. Proposal from Qualcomm, Lenovo not to point out specific spectrum – to remove the yellow text. If it is deleted, then 7.1.1-21 would not be needed. Ericsson prefers to have ITS. | [CPR 7.1.1-22] Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum based on location. |
| [PR 5.22.6-2] Based on operator policy, the 5G system may be able to utilize sensing assistance information exposed by a trusted 3rd party to derive the sensing result. NOTE: Sensing assistance information is information that is provided to 5G system by a trusted 3rd party and can be used to derive sensing result. Examples of sensing assistance information are map information, area information, etc. |  | [CPR 7.1.1-23] Subject to operator’s policy, the 5G system may be able to utilize sensing assistance information received by a trusted third-party to derive the sensing result. NOTE: Sensing assistance information is information that is provided to 5G system by a trusted third-party and can be used to derive sensing result. Examples of sensing assistance information are map information, area information, etc. |
| [PR 5.12.6-2] The base stations shall be able to sense multiple specific target objects and their environments at the same time. [PR 5.10.6-1] Based on operator policy, request from UTM and sensing configuration (e.g. sensing area), the 5G system shall be able to support RAN entities and UEs in sensing the characteristics of an airborne object of interest (e.g., UAV), including generating sensing measurement data related to the object’s location and motion metrics (see examples in Table 5.10.6-1). | Ericsson – it is not clear and example would be needed. | [CPR 7.1.1-24] 5G system shall be able to sense the characteristics of an airborne object one or more specific target objects of interest (e.g., UAVs) and their environment. |
| [PR 5.2.6-2] The 5G system shall be able to support means to select suitable base station(s) to perform sensing, e.g. based on the base station’s location, sensing capability, and the sensing service information requested by trusted third party application. [PR 5.8.6-3] The 5G system shall be able to support an authorized UE in the discovery of UEs and RAN entities with the required 3GPP NR RF sensing capabilities for the sensing service. [PR 5.20.6-3] Based on operator’s policy, the 5G system shall enable a trusted 3rd party to request the activation of the sensing service with specific KPI requirement, as well as deactivation of the same service. [PR 5.17.6-2] The 5G system shall support a mechanism for the 5G network to retrieve the wireless sensing capabilities from UEs and RAN entities, and for the UEs and RAN entities to exchange capabilities amongst each other.? | PR 5.8.6-3 is V2X use case (in case that something more specific would be needed in regard to V2X -i.e. outside of network coverage operation). It can be also rewritten to have network centric approach: Based on 3rd party request, the 5G system shall be able to discover sensing transmitter and sensing receiver with required sensing capabilities. Huawei proposes to split the CPR into two. Ericsson thinks the second part is not needed. | [CPR 7.1.1-25] The 5G system shall be able to select suitable sensing transmitter(s) and sensing receiver(s) based on their wireless sensing capabilities, sensing target area and needed KPIs. |
| [PR 5.19.6-1] Based on 3rd party request, the 5G system shall be able to discover a suitable sensing group where sensing transmitters and receivers are within 100m range to be localized within 10cm of accuracy, with accuracy of sensing measurement within 5 ms of synchronization. PR 5.8.6-3 can be placed here and combined with 5.19.6-1. [PR 5.17.6-1] The 5G system shall be able to coordinate wireless sensing among a set of RAN entities and UEs. [PR 5.18.6.2] The 5G system shall support simultaneous wireless sensing of a target by means of multiple sensing devices. [PR 5.20.6-5] The 5G system shall be able to provide a sensing service considering the interference to the Sensing service caused by the sensing operations between multiple Sensing transmitter(s) and Sensing receiver(s). | These [PR 5.17.6-1] and [PR 5.18.6.2] seem to be solution-oriented requirements and in [PR 5.19.6-1] it seems that coordination and simultaneous sensing is contained between the participants of the sensing group (5ms of synchronization). [PR 5.20.6-5] seems to be solution oriented and coordination and synchronization would imply all additional problems (interference) to be considered. Vodafone, Telefonica prefers to have more generic requirement and a note with the values(example). | [CPR 7.1.2-5] The 5G system shall be able to select/discover a suitable sensing group for a sensing service. NOTE: Sensing transmitters and receivers can be within 100m range to be localized within 10cm of accuracy, with accuracy of sensing measurement within 5 ms of synchronization. [to be separate CPR] The 5G system shall be able to provide 5G wireless sensing considering the interference caused by the sensing operations between Sensing transmitters and Sensing receivers. [to be separate CPR] The 5G system shall be able to coordinate 5G wireless sensing among sensing transmitters and sensing receivers. |

### Network Exposure

| Potential Requirements captured in the TR | Comments | Consolidated potential requirements |
| --- | --- | --- |
| [P.R 5.13.6-6] The 5G system shall provide a mechanism controllable by the operator, according to a business agreement, to report sensing result to a trusted 3rd party about a target object and multiple target objects when specific conditions are met. NOTE: These conditions could be the target object distance from the restricted area border less than 10m or entering restricted area. |  | [CPR 7.1.2-1] Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met. NOTE: These conditions can be: the target object(s) distance from the restricted area border (e.g. 10m); entering restricted area. |
|  | Rewording of on-time might be needed or it can be removed. Latency is part of the sensing KPIs and on-time seem to be embedded. Sensing target area is also a KPI. This requirement is proposed to be merged into CPR 7.1.2-8. |  |
| [P.R 5.19.6-2] The 5G system shall allow an authorized AS to discover a sensing group of UEs in the proximity of the UE that is requesting the service from the AS. NOTE: This requirement assumes that a UE requests an AS to discover a set of UEs that have sensing functions that can provide sensing results from RF sensors and sensing measurement data from non-RF sensors to that AS. | It could go to the other two requirements on discovery and consolidate it together. Lenovo prefers to have them separate. | [CPR 7.1.2-4] Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a service from application server. NOTE: This requirement assumes that application in a UE requests an application server to discover a set of sensing group members with sensing capabilities. |
| [PR 5.32.6-1] Based on operator’s policy, the 5G system may provide a mechanism for a trusted third party to provide assisting information about a sensing target. NOTE: The assisting information can include a UE ID attached to or in proximity of the sensing target. |  | [CPR 7.1.2-6] Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide assistance information about a sensing target. NOTE: The assisting information can include a UE ID attached to or in proximity of the sensing target, map information, area information. |
| [PR 5.21.6-4] Subject to user consent and regulatory requirements, based on operator policy, the 5G system shall be able to expose the combined sensing results to a trusted 3rd party service provider. | Potentially this CPR can be combined with CPR 7.1.2-9. QC prefers not to combine it with 7.1.2-8. Maybe a note would be needed for “combined”. Proposed Note: Combined sensing results in this CPR refer to combination and processing of 3GPP and non-3GPP sensing data. Should only the core network expose sensing results? Or UE also can do it in specific cases (V2X)? | [CPR 7.1.2-7] Subject to user’s consent, regulation and operator’s policy, the 5G network should be able to/may/ provide secure means to expose the combined sensing results to a trusted third-party. NOTE: Combined sensing results in this CPR refer to combination and processing of 3GPP and non-3GPP sensing data. |
| [PR 5.11.6-3] The 5G system shall be able to report the sensing result to the trusted 3rd party with refresh rate which is requested by the trusted 3rd party e.g. a map service provider, and controllable by the operator, according to a business agreement. NOTE 2: The sensing result can be the target object’s size, shape, position, moving direction, moving speed, etc. [P.R 5.13.6-4] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to provide a mechanism for a trusted 3rd party to request the sensing service and based on the request, the base station shall be able to operate sensing periodically or continuously in certain location area for a certain amount of time. [PR 5.12.6-3] The 5G system shall be able to provide a mechanism controllable by the operator, according to a business agreement, for a trusted 3rd party to request the sensing service related with a certain target object or multiple target objects of a certain location area. [PR 5.12.6-5] The 5G system shall be able to report sensing result of the environment around a specific target object to a trusted 3rd party. NOTE 2: The sensing result of the environment for example can be its position, the size of obstacles around, and other moving objects nearby. [PR 5.12.6-4] Based on operator policy, the 5G system shall be able to provide a mechanism for a trusted 3rd party to request per location area different sensing services configuration (e.g. sensing KPI, report refresh rate etc.). [PR. 5.5.6-1] Subject to operator policy, the 5G system shall be able to provide sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area to authorized third parties in a timely manner. [PR. 5.5.6-2] Subject to regional or national regulatory requirements and operator policy, the 5G system shall be able to provide its public warning system with a warning notification based on sensing result indicating disasters or other emergencies (e.g., flooding) in a given geographic area in a timely manner. | Should only the core network expose sensing results? Or UE also can do it in specific cases (V2X)? | [CPR 7.1.2-8] 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing for dedicated location with specific parameters (e.g. refresh rate, period of time, sensing KPIs, geographical location) and to receive the respective corresponding sensing results when needed. NOTE: The sensing result can be the target object’s size, shape, position, moving direction, moving speed, obstacles position and size etc. |
| [PR 5.25.6-4] Subject to user consent and national or regional regulation, based on operator policy, the 5G system shall be able to allow a Sensing device to provide sensing results to a trusted third party. [PR 5.2.6-6] Based on operator’s policy, the 5G system shall expose a suitable API to a trusted third party to provide the information regarding sensing results. [PR 5.9.6-4] Based on operator’s policy, the 5G system shall be able to support means to expose sensing results to a trusted 3rd-party application. [PR. 5.7.6-2] Subject to operator policy, the 5G system shall enable the core network to expose a suitable API to provide the information regarding sensing results to authorized third parties. [PR. 5.3.6-3] Based on operator’s policy, the 5G system shall provide mechanisms to expose NR based sensing results with assisted information, e.g. location, to a trusted 3rd party application via the core network. [PR.5.15.6-2] The 5G system shall support mechanisms to derive and expose sensing results to a trusted 3rd party. [P.R 5.14.6-2] Subject to regulatory requirements and operator policy, the 5G system shall be able to expose sensing results to a trusted 3rd party application. [P.R 5.10.6-3] The 5G system shall be able to support means to process the sensing measurement data and expose in real time the sensing results (e.g., related to a UAV position, velocity) from a 5G sensing processing entity to a trusted 3rd party application. [P.R 5.13.6-5] Based on operator’s policy and subject to regulatory requirements, the 5G system shall be able to periodically expose sensing results to a trusted 3rd party application. [PR. 5.22.6-1] Subject to operator policy, the 5G system shall enable the network to expose a suitable API to a authorized third party to provide the information regarding sensing results. [PR.5.27.6-1] The 5G system shall support exposing the information of sensing result (e.g., location, relative location, velocity vectors, relative headings, etc.) to the trusted and secure mission critical applications. [PR 5.1.6-3] The 5G system shall provide mechanisms for an operator to only collect or expose the sensing information requested by a trusted 3rd party according to agreement. [PR 5.4.6-2] Subject to user consent and national or regional regulatory requirements, based on operator policy, the 5GS shall support a mechanism to expose 3GPP sensing results to trusted third parties. [PR 5.4.6-3] Subject to user consent and national or regional regulatory requirements, based on operator policy, the 5GS shall support a mechanism to expose sensing non-3GPP sensing results to trusted third parties. | Direct exposure from device seems to be a specific solution. UE is part of 5GS and general requirement can be stated for exposure. [PR 5.1.6-3] clarifies that operator would only collect or expose sensing information requested by 3rd party. Collecting part seem to be covered in CPR 7.1.1-8. [PR. 5.3.6-3] Is pointing out to assisting information, e.g. location. Should this be mentioned in the CPR? Ericsson proposes to combine this CPR with CPR 7.1.2-8. The consolidated CPR does not cover original intention – it should be 5G system or the UE to expose the sensing results. Lenovo – to use “as needed” instead of “periodically or in real time”. To merge these PRs with the ones above. |  |
| [PR 5.8.6-5] Subject to user consent and regulations, based on operator policy, the 5G system shall be able to support exposure of sensing results and contextual information (e.g. UE location), to a trusted 3rd-party application. NOTE 4: Contextual information is information sent with the sensing results which provide context to the conditions under which the sensing results were derived. This contextual information can be used in scenarios where the sensing result is to be combined with data from other sources. |  | [CPR 7.1.2-10] Subject to operator’s policy, the 5G system shall be able to provide secure means for a trusted third party application to receive sensing results with contextual information. NOTE: Contextual information is information sent with the sensing results which provide context to the conditions under which the sensing results were derived. This contextual information can be used in scenarios where the sensing result is to be combined with data from other sources. |

### Security

| Potential Requirements captured in the TR | Comments | Consolidated potential requirements |
| --- | --- | --- |
| [PR 5.16.6-1] The 5G system shall provide a mechanism to protect identifiable information that can be derived from the sensing measurements data from eavesdropping. | Eavesdropping – defined in SA3. | [CPR 7.1.3-1] The 5G system shall provide a mechanism to protect identifiable information that can be derived from the 3GPP sensing data from eavesdropping. |
| [PR.6.1.2-1] The 5G system shall limit sending the sensing results only to third party authorized to receive that sensing results. |  | [CPR 7.1.3-2] The 5G system shall limit exposing the sensing results only to third party authorized to receive those sensing results. |
| [PR.6.1.2-2] The 5G system shall support encryption and integrity protection of the sensing result, to protect the data inside the 5G system and when used. [PR 5.23.6 -2] The 5G system shall be able to provide a secure mechanism to ensure sensing result data privacy within the sensing service area. [PR.5.27.6-3] The 5G system shall support security and protection of the 3GPP sensing data, non-3GPP sensing data, and 3GPP sensing results. [PR.5.27.6-4] The 5G system shall provide a secure sensing service for Mission Critical Services. | Proposal to merge CPR 7.1.3-1 into this one. When used to be understood as when expose the data. Proposal from QC for having a general statement and then with a note to give examples of security. | [CPR 7.1.3-3] The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system. |
| [PR.6.1.2-3] The 5G system shall support appropriate level of sensing for both situations where consent can be obtained from the sensing targets, and where it cannot. | There might be a need for additional clarification and rewording. | [CPR 7.1.3-4] The 5G system shall support appropriate level of 5G wireless sensing for both situations where consent can be obtained from the sensing targets, and where it cannot. |
| [PR 6.1.2-4] Subject to regulation, the 5G system shall obtain user consent when sensing results and user identification are brought together for further processing. | Should it be a subject to regulation or in general? | [CPR 7.1.3-5] Subject to regulation, the 5G system shall obtain user’s consent when sensing results and user identification are brought together for further processing. |

### Charging

| Potential Requirements captured in the TR | Comments | Consolidated potential requirements |
| --- | --- | --- |
| [PR 5.2.6-7] The 5G system shall be able to support charging data collection for the sensing services (e.g. considering service type, sensing accuracy, target area, duration) requested by a trusted third-party application. [PR 5.28.6-2] The 5G system shall be able to collect charging information for UEs supporting V2X applications when performing sensing. [PR 5.20.6-4] The 5G system shall be able to support charging for the sensing services (e.g. considering service type, sensing accuracy, target area, duration). | According to Daniel it is not a good idea to use “to support charging”. | [CPR 7.1.4-1] The 5G system shall be able to collect charging data for the 5G wireless sensing service (e.g. considering service type, sensing accuracy, target area, duration) requested by a trusted third-party application. |

### 3. Based on the discussion it is proposed to agree the following consolidated requirements in S1-231318.