3GPP TSG-SA WG1 Meeting #102 	S1-231369

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231350, 1103)

Title:	FS_Metaverse Requirement Consolidation

Agenda Item:	7.3

Document for:	Approval

Source:	Samsung, Huawei

Contact:	Erik Guttman

erik.guttman@samsung.com

Abstract: This pCR proposes a consolidation of requirements for the FS_Metaverse study.

Introduction

This consolidation only considers

-	requirements agreed and in TR 22.856-100;

-	with aligned terminology (based on terminology in 22.856, with alignment throughout the TR as proposed at SA1 101);

-	for potential requirements without editor's notes questioning whether the PR can be included, etc.

-	NOTE numbering is only included in the context of the single CPR and PR. The NOTE numbering of the original PR is not maintained for ease of reading the CPR table.

After SA1 102, the agreed requirements from that meeting (with no remaining Editor's Notes and FFS statements) will be treated as shown below to create a proposed consolidation of requirements for SA1 102.

changes in 1369 from 1350

- the introduction is much shorter, removing text that is no longer accurate after the changes.

- extensive rewording for clarity

- extensive merging of requirements

- moves to different sections

- removal of two sections: digital entities, XR service enablers for location agnostic service experience => requirements are moved to other subclauses

- controversial changes which are not resolved

> use of the term 'IMS' in 3d avatar call requirements => may complicate or exclude support of non-IMS services

> use of the term 'container' => implies a certain technical solution

> restatement of 'digital entity' requirements are unclear (rather than remove the PR, I offer the rapporteur's best effort attempt)

> some potential requirements seem like introductory text to KPIs, please see comments

> some potential requirements are so high level they were removed

To see the original comments and the guidance that lead to the changes in this pCR, please see the following link:

https://10.10.10.10/SA/SA1/Inbox/draft/S1-23zzzz-was1350_QC_HW.DOCX

Proposal

It is proposed to agree to the following change to TR 22.856, 1.0.0.

Start of changes

# 8	Consolidated potential requirements and KPIs

## 8.1	Consolidated potential requirements

### 8.1.A	Localized Mobile Metaverse Service Functionality

This clause consolidates requirements from clauses 5.1, 5.4 5.5, and 5.27.

Table 8.1.A-1 – Localized Mobile Metaverse Service Functionality Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR A.1] | The 5G system shall enable third parties to provide information to the 5GS concerning the availability of spatial anchors. | [P.R.-5.1.6-1] | Reworded for clarity, use the term spatial anchor instead of text. |
|  |  |  | Reworded for clarity Merged into A.6, now A.2 |
|  |  |  | P.R.-5.1.6-2 is split into two requirements. Merged into A.6, now A.2 |
|  |  |  | reworded for clarity Merged into A.6, now A.2 |
| [CPR A.2] | Subject to operator policy, the 5G system shall be able to establish, and expose to a third party, a spatial anchor, i.e. an association between a physical location (a point or volume in three dimensional space) and service information. NOTE: Service information can include information to enable users to discover and access services, e.g. type of service, URLs, configuration data, the distance between the user and the spatial anchor, etc. | [PR-5.4.6-2] [PR-5.1.6.2] [PR-5.1.6.3] | reworded for clarity merged in requirements |
| [CPR A.3] | Subject to operator policy, the 5G system shall enable an authorized third party to request the information associated with a specific spatial anchor. NOTE: How the service and location information is used by the third party to access a mobile metaverse server and the AR media itself is out of scope of this requirement. | [PR-5.4.6-4] | reworded for clarity |
| [CPR A.4] | Subject to operator policy, the 5G system shall provide an authorized third party a means to define who is authorized to access spatial anchor information and to manage the spatial anchor(s), e.g. add, remove or modify spatial anchors. | [PR-5.4.6-5] | Split from [PR-5.4.6-5] |
|  |  |  | Split from [PR-5.4.6-5] merged back into CPR A.4 (was A.8 before renumbering) |
| [CPR A.5] | Subject to operator policy and regulatory requirements, the 5GS shall support mechanisms for an authorized UE to provide sensing data, e.g. to produce or modify a spatial map to an authorized recipient of that sensing data. NOTE: The sensing data could be received and processed by the UE's HPLMN to enable services including spatial mapping and localization, which can be exposed to third parties. | [PR5.5.6.1-1] [PR5.5.6.1-2] [PR5.5.6.2-2] [PR5.5.6.2-3] |  |
|  |  |  | covered by the note in A.5 (was A.10) |
| [CPR A.6] | Subject to operator policy and regulatory requirements, the 5GS shall support mechanisms to expose a spatial map or derived localization information from that map to authorized third parties. | [PR5.5.6.1-3] | Notes are not needed |
|  |  |  | Note is not needed merged into in A.5 (was A.10) |
|  |  |  | Moved to D |
|  |  |  | Moved to D |
|  |  |  | Moved to D, merged into D.6 |

Next change

### 8.1.B	Avatar call functionality

This clause consolidates requirements from clauses 5.3, 5.6, 5.9, 5.11, 5.16, 5.22 and 5.26.

Table 8.1.B-1 – Avatar Call Functionality Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR B.1] | The 5G system shall support 5G CN to provide the IMS with real-time feedback in support of conversational XR communication among multiple users simultaneously, including transfer of animated digital representation of users (e.g. avatar) and audio media. NOTE: The feedback can include information such as network condition, achieved QoS. Such information can be used by the IMS, for example, to trigger the codec negotiation. | [PR 5.3.6.2-1] | Reword for clarity Whether to state IMS or 5G system is not yet agreed. |
| [CPR B.2] | Subject to user consent, the IMS shall support multimedia conversational communications between two or more users including transfer of real time 3D avatar media and audio media. NOTE 1: 3D avatar media can be transmitted on both uplink and downlink. NOTE 2: Confidentiality of the data used to produce the 3D avatar (e.g. from the UE cameras, etc.) is assumed. | [P.R. 5.11.6-1] [PR 5.22.6-1] [P.R. 5.11.6-2] [P.R. 5.11.6-3] | Whether to state IMS or 5G system is not yet agreed. Merged these two PRs. Suggest to replace “animated user digital representation” with “3D avatar media” to be aligned with other CPRs. |
|  |  |  | These PR can be reflected in the corresponding KPI requirements. [Huawei] The suggestion is to move these PRs into the KPI clause. [Rapporteur] This text can be used in the non-normative 'general' section above the KPIs, effectively to describe the services the KPIs enable, when we produce CRs or a TS for this feature. |
|  |  |  | Moved to D, merged into D.6 |
|  |  |  | Moved to D, merged into D.6 |
|  |  |  | Moved to D |
|  |  |  | Aligned the term “Service Hosting Environment” Moved to D |
|  |  |  | The NOTE is not needed. covered in B.2, NOTE 1 |
|  |  |  | covered in B.2, NOTE 2 |
| [CPR B.3] | OPTION 1 Subject to user consent, the 5G system shall support multimedia conversational communications between two or more users including a means to change media types between audio/video and avatar media. OPTION 2 Subject to user consent, the 5G system with IMS shall support change of media types between video and avatar media for parties of an IMS call. | [P.R. 5.11.6-4] | The note is not necessary. Whether to state IMS or multimedia conversational communications is not yet agreed. |
| [CPR B.4] | The 5G system with IMS shall support transcoding between media such as text, GTT, video and 3D avatar media. NOTE 1: Text, video or other media could allow a party to control the appearance of its avatar, e.g. to express behavior, movement, affect, emotions, etc. NOTE 2: The transcoding of media enables 3D avatar communication, e.g. in scenarios in which UE participating in the IMS call does not support e.g. FACS, encoding avatar media, presenting avatar media, etc. | [P.R. 5.11.6-5] [P.R.-5.26.6-2] [P.R.-5.26.6-3] [P.R.-5.26.6-4] | Whether to state IMS or not is not yet agreed. |
|  |  |  | Merged into B.5, was B.13 |
|  |  |  | Merged into B.5, was B.13 |
| [CPR B.5] | The 5G system shall support the encoding of sensor data capturing the facial expression and movement and gestures of a person, in a standard form. | [P.R.-5.26.6-1] [PR 5.16.6.2-5] [PR 5.16.6.2-6] |  |
|  |  |  | merged into B.4 (was B.10) |
|  |  |  | merged into B.4 (was B.10) |
|  |  |  | merged into B.4 (was B.10) |

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  |  |  | MOVED TO D |
|  |  |  | Clarified wording MOVED TO D |
|  |  |  | what does the round trip latency refer to? Between the UE and an AS? See the addition of the clarifying NOTE. Qualcomm: Move to KPI part…if not covered already…better to express/convert as E2E latency [Rapporteur] The requirement text here can be used as a introduction text (informative) ahead of the KPIs when we move to the normative specification phase. |

Start of changes

### 8.1.D	Coordination and operation of mobile metaverse services

This clause consolidates requirements from 5.7, 5.8, 5.14.

Table 8.1.D-1 – Coordination and operation of mobile metaverse services Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  |  |  | Merged below to D.1 (was D.2) |
| [CPR D.1] | Subject to operator policy, the 5G system shall support a mechanism that enables flexible adjustment of communication services based on e.g. the type of devices (e.g., wearables), or communication duration (e.g. more than one hour), such that the services can be operated with reduced energy utilization. NOTE: Metaverse service experience over an extended period of time (e.g. 2h) requires significant power consumption by the UE. In some cases, a device with no external power supply cannot sustain downloading and rendering of media over a long interval, e.g. for the duration of an entire feature film or athletic event. | [PR 5.7.6-1] [PR 5.7.6-2] | reworded for clarity and to merge former D.1 |
|  |  |  | Qualcomm: This is a general requirement that is already satisified. |
| [CPR D.3] | The 5G system shall support suitable APIs to securely provide information of a user to an immersive mobile metaverse service when the user accesses the service. | [PR 5.14.6-1] | We must clarify what 'information of a user' means. |
|  |  |  | What does 'user assets' mean? How can treatment of info stored by services be in scope of 3GPP? MOVED TO G |
| [CPR D.4] | The 5G system shall be able to provide a means to associate, coordinate and update data flows related to one or multiple UEs to support e.g. digital twin applications provided by the mobile metaverse service. | [PR 5.20.6-1] |  |
|  |  |  | Merged above |
|  |  | [PR 5.20.6-3] | Qualcomm: Very specific, can be covered by general CPR above |
| [CPR D.5] | Subject to operator policy and user consent, the 5G system shall be able to expose network performance information (e.g., predicted bitrate, latency, for one or more users) to an authorized 3rd party metaverse application. NOTE: The network performance information can be per UE and take into account all available 5G access network types. | [PR 5.25.6-1] [PR 5.9.6.2] |  |
|  |  |  | merged into CPR D.6 below |
| [CPR D.6] | Subject to operator policy, the 5G system shall support a mechanism to enable one or more authorized third party(ies) to coordinate the transmission of multiple service data flows delivered to/from one or more UE(s). Multiple UEs may be associated with one user/location or different users at different locations. NOTE: Coordinated transmission refers to the ability to provide an acceptable level of user experience for a given service, e.g. based on latency and synchronization constraints (due to multiple sources or long distance between UEs/users). | [PR 5.27.6-3] [PR 5.9.6.1] [PR 5.3.6.2-4] [PR 5.25.6-2] | New merged CPR from diverse requirements |
| [CPR D.7] | The 5G system shall enable the coordination of diverse media, transmitted to a UE from one or more mobile metaverse services associated with a physical location, to be combined to form a localized service experience. | [P.R.-5.1.6-4] [PR-5.4.6-1] | reworded for clarity, merged MOVED to D from A.5 |
| [CPR D.8] | Subject to operator policy, the 5G system shall support exposure mechanisms enabling a trusted third party to determine one or more subscribers to whom mobile metaverse media can be distributed in a resource efficient manner. | [PR 5.27.6-1] | Moved to D from A.14 |
| [CPR D.9] | Subject to operator policy, subject to user consent, the 5G system shall support mechanisms to provide resource efficient communication of third party mobile metaverse media to one or more subscribers. | [PR 5.27.6-2] | Moved to D from A.15 |
| [CPR D.10] | The 5G system shall provide a means to expose predicted network conditions, in particular, latency, between remote users. | [PR 5.9.6.2] | Moved to D, was B.5 |
| [CPR D.11] | The 5G system shall provide a means to support the distribution, configuration, and execution of a predictive model associated to a remote user in a Service Hosting Environment. | [PR 5.9.6.3] | Aligned the term “Service Hosting Environment” Moved to D, was B.6 |
| [CPR D.12] | The 5G system shall provide a means to synchronize multiple service data flows (e.g., heart rate, video, audio) of multiple UEs associated with Critical HealthCare services. | [PR 5.10.6-2] | Moved to D, was C.1 |
| [CPR D.13] | The 5G system shall provide a mechanism to support coordination and synchronization of multiple data flows transmitted via one UE or different UEs, subject to synchronization thresholds provided by third party (i.e. a quantitative bound to the extent different data flows can diverge in synchronization.) | [PR 5.12.6-1] | Moved to D, was C.1 |

Next change

### 8.1.F	Security and Privacy aspects of mobile metaverse services

This clause consolidates requirements from 5.18, 5.19, 5.24, aspects of 5.3 and 5.5.

Table 8.1.F-1 – Security and Privacy aspects of mobile metaverse services Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR F.1] | Subject to regulatory requirements, operator policies and user consent, the 5G system shall be able to support mechanisms to expose to a trusted third party (e.g. the conference focus) the result of authenticating a user identity to a UE. NOTE: Authenticating a user identity to a UE at the terminal side is out of 3GPP scope. | [PR 5.3.6.2-2] |  |
| [CPR F.2] | Subject to operator policy and relevant regional and national regulation, the 5GS shall support mechanisms to authorize Spatial Localization Service. | [PR5.5.6.2-1] |  |
| [CPR F.3] | Subject to national/regional regulations, and user consent, the 5G System shall be able to process and expose information from UEs related to user’s location, user’s body, and user’s environment, e.g., user’s home, user’s immediate vicinity. NOTE: This requirement does not affect the ability of regulatory services, e.g., legal intercept service, to access such information without consent of the user. | [5.19.6-1] |  |
| [CPR F.4] | Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall support mechanisms to identify an avatar and associate the avatar with a subscriber (i.e. the owner of the avatar). | [PR 5.18.6-1] [PR 5.24.6-1] |  |
| [CPR F.5] | Subject to operator policy and national or regional regulation, the 5G system shall support different communication security mechanisms according to the security requirements of different services. | [PR 5.18.6-2] |  |
| [CPR F.6] | Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall be able to authorize the avatar to be used in mobile metaverse services. | [PR 5.24.6-2] |  |
| [CPR F.7] | Subject to regulatory requirements, user’s consent and operator’s policy, the 5G system shall provide time-bound authorization services for an avatar to be used in mobile metaverse services. | [PR 5.24.6-3] |  |
| [CPR F.8] | OPTION 1 Remove the requirement. OPTION 2 The 5G system shall be able to authenticate a digital entity. OPTION 3 Subject to regulatory requirements and operator policy and subscriber consent, the 5G system shall provide a means for a third party to access 3GPP services on behalf of the subscriber. | [PR 5.17.6-1] | Moved to F from E.1 IT IS NOT CLEAR WHAT THIS REQUIREMENT MEANS - PROPOSAL TO THE LEFT |
| [CPR F.9] | The 5G system shall be able to provide a means to identify an authorized digital representation (e.g. avatar) associated with a subscriber. | [PR 5.17.6-2] | Moved to F from E.2 Reworded for clarity |

Next change

### 8.1.G	Digital Asset Management

This clause consolidates requirements from 5.18, 5.19, 5.24, aspects of 5.3 and 5.5.

Table 8.1.G-1 – Digital Asset Management Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR G.1] | Subject to user consent, operator policy, and regulatory requirements, the 5G system shall be able to support handling of a digital asset container to store the information associated with a user. | [PR 5.13.6-1] [PR 5.15.6-1] [PR 5.16.6.2-1] | Qualcomm: Remove the term 'container'? suggest: information |
| [CPR G.2] | Subject to user consent, operator policy, and regulatory requirements, the 5G system shall provide mechanisms to allow users to securely access and update their information in the digital asset container associated with this user. NOTE: For CPR G.1 and CPR G.2, the user could be a human user using a UE with a certain subscription, or an application running on or connecting via a UE, or a device behind a gateway UE. The user could also be a third party, e.g. an enterprise customer having service level agreement with the operator and interacting with the 3GPP network via an application server. | [PR 5.13.6-1] [PR 5.15.6-1] [PR 5.16.6.2-1] | Qualcomm: Remove the term 'container'? suggest: information |
| [CPR G.3] | Subject to user consent, the 5G system shall be able to allow a trusted third party to retrieve the information of a digital asset container associated with a user, e.g. when the user accesses a specific application. | [PR 5.13.6-2] [PR 5.15.6-3] | Qualcomm: Remove the term 'container'? suggest: information |
| [CPR G.4] | Subject to regulatory requirements and operator’s policy, the 5G system shall provide secure means to authorize the use of digital asset information by a user (e.g. digital assets belonging to a third party customer). | [PR 5.16.6.2-2] [PR 5.13.6.5] | Reworded for clarity |
|  |  |  | merged into G.3 |
| [CPR G.5] | The 5G system shall provide mechanisms to certify the authenticity of the information of a digital asset container associated with a user. | [PR 5.13.6-4] |  |
|  |  |  | merged above, and specific security threats are not in scope of SA1. |
| [CPR G.8] | The 5G system shall be able to collect charging information per UE for managing the digital asset container associated with a user (e.g. typically a human user with a certain subscription). | [PR 5.16.6.2-3] | MOVED to H |
| [CPR G.9] | The 5G system shall be able to collect charging information per application for managing the digital asset container for the third party (e.g. typically an enterprise customer having service level agreement with the operator). | [PR 5.16.6.2-4] | MOved to H |
| [CPR D.5] | The 5G system shall support mechanisms to adapt the user assets and information stored by one immersive mobile metaverse service with the information needed, or requested, by another immersive XR media service. | [PR 5.14.6-2] | Moved from D.5 |

Next change

### 8.1.H	Charging requirements for mobile metaverse services

This clause consolidates requirements from aspects of 5.3, 5.4, 5.5, 5.11, 5.26 and 5.27.

Table 8.1.H-1 – Consolidated Requirements on charging for mobile metaverse services

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR H.1] | The 5G system shall be able to collect charging information per UE or per application for the use of IMS based conferencing services. | [PR 5.3.6.2-3] |  |
| [CPR H.2] | The 5G system shall be able to collect charging information for the establishment or management of an association between a physical location and service information, where a third party creates, deletes or changes a spatial anchor or associated service information. | [P.R.-5.4.6-6] |  |
| [CPR H.3] | The 5G system shall be able to collect charging information associated with the network operator exposure of spatial anchors to authorized third parties, and of service information associated with spatial anchors. NOTE: The preceding requirements assumes that exposure of network anchors and associated service information can be a service provided by a network operator to third parties. | [P.R.-5.4.6-7] |  |
| [CPR H.4] | The 5G system shall support the collection of charging information associated with the exposure of a spatial map or derived localization information to authorized third parties. | [PR5.5.6.1-4] |  |
| [CPR H.5] | The 5G system shall support the collection of charging information associated with the production or modification of a spatial map on behalf of an authorized third party. | [PR5.5.6.1-5] |  |
| [CPR H.6] | The 5G system shall support the collection of charging information associated with exposing spatial location service information to authorized third parties. | [PR5.5.6.2-4] |  |
| [CPR H.7] | The 5G system shall support collection of charging information associated with initiating and terminating an IMS-based 3D avatar call. | [P.R. 5.11.6-6] |  |
| [CPR H.8] | The 5G system shall be able to collect charging information for transcoding services associated with IMS-based avatar call. | [P.R.-5.26.6-5] |  |
| [CPR H.9] | The 5G system shall be able to collect charging information associated with distribution of third party mobile metaverse media to one or more subscribers. | [PR 5.27.6.4] |  |
| [CPR G.8] | The 5G system shall be able to collect charging information per UE or per application, related to the use of digital asset container associated with a user (e.g. typically a human user with a certain subscription). | [PR 5.16.6.2-3] [PR 5.16.6.2-4] | Moved from G.8 Qualcomm: Remove the term 'container'? |
|  |  |  | Merged into the above. |

End of changes