3GPP TSG SA WG 1 Meeting #102		 S1-231394

Berlin, Germany, 23-27 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0686 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AI-ML KPIs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML-MT |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last meeting (SA1#101), there is a CR S1-230110 was agreed to change the latency from 2ms to 15ms. However it is missed to change the “experieced data rate” i.e. 144 Mbit/s = 0.27 Mbyte/15ms. This CR is to correct it. Meanwhile, TS22.261v19.2.0 also needs be aligned with this change |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise UL experience data rate KPI for image recognition, to 144 Mbit/s, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KPI values remain unclear or not fully justified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================== First Change =====================

## 7.10	KPIs for AI/ML model transfer in 5GS

The 5G system shall support split AI/ML inference between UE and Network Server/Application function with performance requirements as given in Table 7.10-1.

Table 7.10-1 KPI Table of split AI/ML inference between UE and Network Server/Application function

| # Uplink KPI |  |  |  |  | # Downlink KPI |  |  |  | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # Max allowed UL end-to-end latency | # Experienced data rate | # Payload size | # Communication service availability | # Reliability | # Max allowed DL end-to-end latency | # Experienced data rate | # Payload size | # Reliability |  |
| 15 ms | 144 Mbit/s | # 0.27 MByte | # 99.999 % | # 99.9 % |  |  |  | # 99.999 % | # Split AI/ML image recognition |
| 100 ms | 1.5 Mbit/s |  |  |  | # 100 ms | # 150 Mbit/s | 1.5 MByte/‌frame |  | # Enhanced media recognition |
|  | # 4.7 Mbit/s |  |  |  | 12 ms | 320 Mbit/s | 40 kByte |  | Split control for robotics |
| NOTE 1: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |  |  |

The 5G system shall support AI/ML model downloading with performance requirements as given in Table 7.10-2.

Table 7.10-2 KPI Table of AI/ML model downloading

| # Max allowed DL end-to-end latency | # Experienced data rate # (DL) | # Model size | # Communication service availability | # Reliability | # User density | # # of downloaded AI/ML models | # Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.1Gbit/s | 138MByte | 99.999 % | 99.9% for data transmission of model weight factors; 99.999% for data transmission of model topology |  |  | AI/ML model distribution for image recognition |
| 1s | 640Mbit/s | 80MByte | 99.999 % |  |  |  | AI/ML model distribution for speech recognition |
| 1s | 512Mbit/s(see note 1) | 64MByte |  |  |  | Parallel download of up to 50 AI/ML models | Real time media editing with on-board AI inference |
| 1s |  | 536MByte |  |  | up to 5000~ 10000/km2 in an urban area |  | AI model management as a Service |
| 1s | 22Mbit/s | 2.4MByte | 99.999 % |  |  |  | AI/ML based Automotive Networked Systems |
| 1s |  | 500MByte |  |  |  |  | Shared AI/ML model monitoring |
| 3s | 450Mbit/s | 170MByte |  |  |  |  | Media quality enhancement |
| NOTE 1: 512Mbit/s concerns AI/ML models having a payload size below 64 MB. TBD for larger payload sizes. NOTE 2: Communication service availability relates to the service interfaces, and reliability relates to a given system entity. One or more retransmissions of network layer packets can take place in order to satisfy the reliability requirement. |  |  |  |  |  |  |  |

The 5G system shall support Federated Learning between UE and Network Server/Application function with performance requirements as given in Table 7.10-3.

Table 7.10-3: KPI Table of Federated Learning between UE and Network Server/Application function

| # Max allowed DL or UL end-to-end latency | # DL experienced data rate | # UL experienced data rate | # DL packet size | # UL packet size | # Communication service availability | # Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1s | 1.0Gbit/s | 1.0Gbit/s | 132MByte | 132MByte |  | Uncompressed Federated Learning for image recognition |
| 1s | 80.88Mbit/s | 80.88Mbit/s | 10Mbyte | 10Mbyte | TBD | Compressed Federated Learning for image/video processing |
| 1s | TBD | TBD | 10MByte | 10MByte |  | Data Transfer Disturbance in Multi-agent multi-device ML Operations |

## ================ End of Changes ====================