3GPP TSG SA WG 1 Meeting #102 	S1-231364

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231150)

Source:	Qualcomm

pCR Title:	Update to Use Case on Ambient IoT for airport terminal / shipping port

Draft Spec:	3GPP  TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: Updating KPIs for the use case in clause 5.7

1. Introduction

Updating KPI table

2. Reason for Change

Adding values for positioning service availabillity and position accuracy KPIs.

Removing the comparators from the KPI values

Changing the end to end latency to range 1sec to 10sec

Changing units of device speed from Km/hr to Km/h

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0

* * * First Change * * * *

### 5.7.6	Potential New Requirements needed to support the use case

[PR 5.7.6-001] The 5G system shall be able to support means to discover and locate Ambient-IoT devices in a certain geographical area, e.g. at cell level.

[PR 5.7.6-002] The 5G system shall be able to provide communication with Ambient-IoT devices with the following KPIs :

Table 5.7.6-1:  KPI Requirements for Airport Terminal/Shipping Port Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Airport Terminal/ Shipping Port | 1s to 10s | 99% | NA | NA | 256 bits (UL) (Note 1) | 100 devices/1km2 | 50m (indoor) | 1-10km2 (Note 2) | 3 to 10km/h | NA | NA | 90% | cell level |
| Note 1: 128 bits for the Electronic Product Code (EPC) of the tracked object and additional 128 bits assumed for control / other data (e.g., location-related). Note 2: As an example, Newark Airport size ~8km2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *