3GPP TSG SA WG 1 Meeting #102 	S1-231408

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231238)

Source:	Xiaomi

pCR Title:	Pseudo-CR on Changing Ranging to relative positioning in the use case 10

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Chunhui Zhu, zhuchunhui@xiaomi.com

Abstract: It is proposed to change Ranging to relative positioning in the use case 10.

1. Introduction

TS 22.261 v19.2.0 has the following definitions:

Ranging: refers to the determination of the distance between two UEs and/or the direction of one UE from the other one via direct device connection.

relative positioning: relative positioning is to estimate position relatively to other network elements or relatively to other UEs.

The use case ‘5.10 Use case on Ranging for Ambient IoT’ in TR 22.840 v1.1.0 has the sentence:

Ranging refers to the determination of the distance between two UEs or more UEs and/or the direction and/or relative positioning of one UE (i.e., Target UE) from another UE (i.e. Reference UE) via PC5 interface.

2. Reason for Change

The use case ‘5.10 Use case on Ranging for Ambient IoT’ actually requires to determine the distance between the UE and the Ambient IoT device, while the Ambient IoT device is not a UE and the PC5 interface may not be re-used, so the term Ranging is not suitable in this use case.

As defined in TS 22.261 v19.2.0, relative positioning is to estimate position relatively to other network elements or relatively to other UEs. It means the distance determination between the UE and the Ambient IoT device can be regarded as relative positioning.

3. Conclusions

It is proposed to change Ranging to relative positioning in the use case 5.10 in TR 22.840 v1.1.0.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

## 5.10	Use case on relative positioning for Ambient IoT

### 5.10.1	Description

Ambient Internet of Things (Ambient IoT) is an IoT service with an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (i.e., using a capacitor). It can enable communication with IoT devices without conventional power source and/or avoids human intervention for recharging or replacing. An Ambient IoT device can harvest energy from energy source from Radio, solar, light, motion/vibration, heat, pressure, or any other power sources.

Note: Energy harvesting is out of scope of this TR.

Relative positioning is to estimate position relatively to other network elements or relatively to other UEs..

### 5.10.2	Pre-conditions

Tom buys an Ambient IoT device which has a 3GPP subscription and is equipped with 3GPP radio technology. There is a relative positioning APP in Tom’s UE. The relative positioning application server and clients records the UE and Ambient IoT device’s relative positioning application layer ID.

Tom pastes the device on his key.

In a morning, Tom gets up and wants to walk in the park nearby.

### 5.10.3	Service Flows

- Tom wants to take his key, but cannot find the key in the room. He takes out his UE to initiate a relative positioning request for the Ambient IoT device by using the relative positioning APP. The 5G system authenticates the UE and the Ambient IoT, and authorizes the UE and the Ambient IoT device to perform the relative positioning.

- The UE sends the radio to activate the Ambient IoT device in the room and the relative positioning request with the relative positioning application layer ID of the UE as the requester and the Ambient IoT device as the target.

- After receiving and storing enough energy, the Ambient IoT device responds the UE’s relative positioning request with its relative positioning application layer ID. From the APP, Tom’s UE displays that his key is still in the room.

- After the relative positioning discovery procedure above, the UE continues sending the radio to the Ambient IoT device, and the Ambient IoT device receives and stores the energy for continuous relative positioning.

- The UE performs relative positioning to find the direction and relative positioning to the Ambient IoT device.

### 5.10.4	Post-conditions

Tom finds his key under the bed.

### 5.10.5	Existing features partly or fully covering the use case functionality

None

### 5.10.6	Potential New Requirements needed to support the use case

[PR 5.10.6-001] 5G system shall be able to support an authorized UE to perform Ambient IoT relative positioning between the UE and specific Ambient IoT devices.

[PR 5.10.6-002] The 5G system shall be able to provide relative positioning services for Ambient IoT with the performances requirements reported in Table 5.10.6.1-1.

Table 5.10.6-1: Performance requirements for relative positioning service for Ambient  IoT

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Finding Items in a home | NA | NA | NA | NA | NA | 20 Ambient IoT devices/ (100m2) | 10m IC/PC/OOC | NA | Static/ Moving (<1m/s) | 500ms | NA | 95 % | 100 cm, |
| NOTE: The KPI values in this table are relaxed compared to Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels, TS22.261 V19.2.0 with the consideration in a home use case. |  |  |  |  |  |  |  |  |  |  |  |  |  |