3GPP TSG SA WG 1 Meeting #102 	S1-231411

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231291)

Source:	Huawei

pCR Title:	Pseudo-CR on FS_Ambient IoT, remove Editor’s Note by adding KPIs to clause 5.16 automated supply chain distribution

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang Zhang (zhang.shuang2@huawei.com)

Abstract: This document provides a TP to TR 22.840 v.1.1.0 to remove the Editor’s Note from clause 5.16.6 on the use case of automated supply chain distribution. The revision includes the changes to address received comments.

1. Introduction

The use case of “Ambient IoT service for automated supply chain distribution” has been agreed and captured in TR 22.840, which includes the use case description, potential requirtements and some KPI values such as communication service availability. In Clause 5.16.6, there is an EN for capturing additional KPI values. It also means further clarification is expected to provide more details on the proposed KPI values.

2. Reason for Change

To address the EN, the following updates are includes in the pCR:

- provide additional information on the white goods private warehousing;

- new potential KPI values are proposed:

- indoor communication range of 20m, taking into account typically around 10000 square feet storage area of a private warehouse;

- outdoor communication range of 500m, considering reusing the existing site of macro base-stations;

- Since the indoor positioning accuracy [3m] has been captured for many meeting cycles, during which period this value has been discussed and accepted elsewhere such as in similar other use cases, the square brackets can be removed.

3. Conclusions

Add the additional KPI values in clause 5.16.6 and remove square brackets for the indoor positioning accuracy, whereby removing the corresponding Editor’s Note.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	International Energy Agency (IEA), "World Energy Outlook 2020". Available online: https://www.iea.org/reports/world-energy-outlook-2020

[3] 	Motlagh, N.H.; Mohammadrezaei, M.; Hunt, J.; Zakeri, B. "Internet of Things (IoT) and the Energy Sector." Energies 2020, 13, 494.

[4] 	Pereira F., Correia R., Carvalho N.B. "Passive Sensors for Long Duration Internet of Things Networks." Sensors 2017;17:2268. doi: 10.3390/s17102268.

[5] 	https://www.gs1.org/standards/rfid.

[6]	3GPP TS 22.368: "Service requirements for Machine-Type Communications (MTC)".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.011: “Service accessibility”.

[10]	5G for smart manufacturing: https://www.gsma.com/iot/wp-content/uploads/2020/04/2020-04_GSMA_SmartManufacturing_Insights_On_How_5G_IoT_Can_Transform_Industry.pdf

[11] 	 https://forcetechnology.com/en/articles/batteryless-electronics-energy-harvesting

[12] 	R. Ding and B. Xing, "Comparative Research on the Way of Energy Harvesting of the Wireless Sensor Network Nodes," 2013 6th International Conference on Intelligent Networks and Intelligent Systems (ICINIS), 2013

[13] 	P. D. Hung et al., "A Self-Powered Wireless Gas Sensor Node Based on Photovoltaic Energy Harvesting," 2021 Symposium on VLSI Circuits, 2021.

[14] 	GS1: "EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard (Release 2.1) "

[15]                      EPC Tag Data standards version 1.1 Rev.1.24

[16] 	Noghabaei S M, Radin R L, Savaria Y, et al. A high-efficiency ultra-low-power CMOS rectifier for RF energy harvesting applications[C]//2018 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2018: 1-4

[17] 	Wu Z, Zhao Y, Sun Y, et al. A Self-Bias Rectifier with 27.6% PCE at-30dBm for RF Energy Harvesting[C]//2021 IEEE International Symposium on Circuits and Systems (ISCAS). IEEE, 2021: 1-5.

[18] 	Valenta C R, Durgin G D. Harvesting wireless power: Survey of energy-harvester conversion efficiency in far-field, wireless power transfer systems[J]. IEEE Mi crowave Magazine, 2014, 15(4): 108-120.

[18] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[20] 	Green M, Dunlop E, Hohl‐Ebinger J, et al. Solar cell efficiency tables (version 57)[J]. Progress in photovoltaics: research and applications, 2021, 29(1): 3-15.

[21] 	Mishu M K, Rokonuzzaman M, Pasupuleti J, et al. An adaptive TE-PV hybrid energy harvesting system for self-powered iot sensor applications[J]. Sensors, 2021, 21(8): 2604.

[22]	Prauzek M, Konecny J, Borova M, et al. Energy harvesting sources, storage devices and system topologies for environmental wireless sensor networks: A review[J]. Sensors, 2018, 18(8): 2446.

[23] 	Kim S, Vyas R, Bito J, et al. Ambient RF energy-harvesting technologies for self-sustainable standalone wireless sensor platforms[J]. Proceedings of the IEEE, 2014, 102(11): 1649-1666.

[24] 	H. S. Kim, J. -H. Kim, and J. Kim, ‘‘A review of piezoelectric energy harvesting based on vibration,’’ Int. J. Precision Eng. Manuf., vol. 12, no. 6, pp. 1129–1141, Dec. 2011.

[25] 	Hoang T, Ferin G, Bantignies C, et al. Aging assessment of piezoelectric energy harvester using electrical loads[C]//Journal of Physics: Conference Series. IOP Publishing, 2019, 1407(1): 012078.

[26] 	https://www.usda.gov/foodwaste/faqs

[27] 	https://www.fda.gov/food/new-era-smarter-food-safety#:~:text=Welcome%20to%20the%20New%20Era,reducing%20the%20number%20of%20illnesses

[28] 	Alam AU, Rathi P, Beshai H, Sarabha GK, Deen MJ. Fruit Quality Monitoring with Smart Packaging. Sensors (Basel). 2021 Feb 22;21(4):1509. doi: 10.3390/s21041509. PMID: 33671571; PMCID: PMC7926787.

[29] 	Taoukis, Petros & Nychas, George-John. (2000). Use of time-temperature integrators and predictive modeling for shelf life control of chilled fish under dynamic storage conditions. International journal of food microbiology. 53. 21-31. 10.1016/S0168-1605(99)00142-7.

[30] 	Mishu, M.K., Rokonuzzaman, M., Pasupuleti, J., Shakeri, M., Rahman, K.S., Hamid, F.A., Tiong, S.K. and Amin, N., 2020. Prospective efficient ambient energy harvesting sources for iot-equipped sensor applications. Electronics, 9(9), p.1345.

[31] 	Bagchi, S., Abdelzaher, T.F., Govindan, R., Shenoy, P., Atrey, A., Ghosh, P. and Xu, R., 2020. New frontiers in IoT: Networking, systems, reliability, and security challenges. IEEE Internet of Things Journal, 7(12), pp.11330-11346.

[32] 	O’Neill, V. and Soh, B., 2022. Improving Fault Tolerance and Reliability of Heterogeneous Multi-Agent IoT Systems Using Intelligence Transfer. Electronics, 11(17), p.2724.

[33] 	Katanbaf, M., Jain, V. and Smith, J.R., 2020. Relacks: Reliable backscatter communication in indoor environments. Proceedings of the ACM on Interactive, Mobile, Wearable and Ubiquitous Technologies, 4(2), pp.1-24.

[34] 	3GPP TR 23.501 release-17 - https://www.3gpp.org/ftp/Specs/archive/23_series/23.501/23501-h50.zip

[35] 	3GPP TR 38.300 release-17 - https://www.3gpp.org/ftp/Specs/archive/38_series/38.300/38300-h10.zip

[36] 	https://www.mokosmart.com/use-iot-fire-detector-sensor/

[37]  	https://www.5gamericas.org/wpcontent/uploads/2019/07/5G_Americas_URLLLC_White_Paper_ Final__updateJW.pdf

[38] 	TS 22.261 https://www.3gpp.org/ftp/Specs/archive/22_series/22.261/22261-j00.zip

[39] 	 	    TR 22.866: "enhanced Relays for Energy Efficiency and Extensive Coverage".

[40] 		     		Global RFID Sensor Market - Global Industry Analysis, Size, Share, Growth, Trends

and Forecast 2021-2031, Transparency Market Research, June 2021

[41]		     		Sensors: Technologies and Global Markets (https://www.bccresearch.com/market-

research/instrumentation-and-sensors/sensors-technologies-markets-report.html),BCC

Research

[42]		     		Berckmans, D. Precision livestock farming technologies for welfare management in

intensive livestock systems. Rev. Sci. Tech. Off. Int. Epiz. 2014, 33, 189–196.

[43]	Precision Agriculture for Crop and Livestock Farming - Brief Review, Animals 2021

[44]                	Journal of Dairy Science - Comparison of 2 systems of pasture allocation on milking intervals and total daily milk yield of dairy cows in a pasture-based automatic milking system, American Dairy Science Association, 2013

[45]	Grazing and automation, Proceedings 4th Meeting European Grassland Federation WG “Grazing”, 2015

[46] 	Grazing Methods, UK Agriculture Food and Environment, https://grazer.ca.uky.edu/content/grazing-methods-which-one-you#:~:text=Strip%20Grazing%20%E2%80%93%20This%20technique%20involves,a%20new%20allocation%20of%20forage.

[47]                	Nora O’Donovan, https://www.independent.ie/business/farming/dairy/dairy-    advice/putting-a-size-on-paddocks-30447793.html

[48]	A. van den Pol-van Dasselaar, P. W. Blokland, et al, Beweidbare oppervlakte en weidegang op melkveebedrijven in Nederland (Pasture area and grazing on dairy farms in the Netherlands), Livestock Research Wageningen, 2015

[49]	Laura Paine, A Summary of Dairy Grazing Practices in Wisconsin

[50]	R. J. Rendell, Designing Paddocks on an Irrigated Dairy Farm, Victoria, Australia

[51]	The EU pig meat sector, European Parliament

[52]	http://www.bestgenetics.com.cn/EN/PigFarm1

[53]		    			Lisette. E. van der Zande, et al, Individual detection and tracking of group housed pigs

in their home pen using computer vision, Front. Anim. Sci. 2:669312. doi:

10.3389/fanim.2021.669312, April 2021

[54]	Carol Souza de Silva, Wageningen University, the impact of Pen Size and Stocking Density on behaviour and welfare of growing pigs

[55]	Xue Li, Xia Xiong, et al, Effects of stocking density on growth performance, blood parameters and immunity of growing pigs, December, 2020

[56] 	https://www.concreteconstruction.net/projects/infrastructure/so-many-manholes-so-little-time_o#:~:text=U.S.%20EPA%20estimates%20the%20number,from%20100%20to%20500%20feet.

[57]	http://news.cjn.cn/24hour/wh24/201101/t1276749.htm

[58] 	https://www.dibblecorp.com/insights/manhole-infrastructure-assessments/

[59] 	https://www.francebleu.fr/infos/faits-divers-justice/un-homme-retrouve-mort-dans-une-bouche-d-egout-a-saint-nazaire-1666456765

[60]	https://guancha.gmw.cn/2022-05/16/content_35739227.htm

[61]	Intelligent manhole cover national standard, GB/T 41401, 2022, the Ministry of Housing and Urban-Rural Development of the People's Republic of China

[62]	MEMS gauge pressure sensor NSPGD1 Novosense, liquid level measurement, https://www.novosns.com/Public/Uploads/uploadfile/files/20220726/-897.pdf

[63]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[64]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[65] 	Sung-Pil CHANG, Jinkyo CHOO, Values of Bridge in the Formation of Cities

[66]	https://www.thepaper.cn/newsDetail_forward_4643273

[67] 	https://news.sina.cn/gn/2021-12-19/detail-ikyamrmy9954352.d.html

[68] 	https://www.nytimes.com/2020/12/22/world/europe/genoa-bridge-collapse.html

[69]	https://www.theguardian.com/world/2022/oct/31/india-bridge-collapse-death-toll-rises-rescue-efforts-continue

[70]	Role of the Local Government in Monitoring and Maintenance Bridges, IAPA Proceedings Conference, 2019

[71]	https://elastisense.com/structural-health-monitoring-bridges

[72]	GB/T 39559.2-2020 National Specification for operational monitoring of urban rail transit facilities – Part 2: Bridge

[73]	Tilt sensor, Clinometer LE-60, http://www.tuoluoyi.com/upfile/201610/2016100853701905.pdf

[74]	https://www.analog.com/media/en/technical-documentation/data-sheets/adxl375.pdf

[75]	Big data analysis for bridges health monitoring, Tongji University, 2017

[76]	Beijing Dongsha bridge safe operation monitoring, http://www.ccea.zju.edu.cn/_upload/article/files/64/e6/a824c7d04932af200750fbd9e690/fb0a5e9c-a1e7-47b8-bdb4-82b584d1c52f.pdf

[77]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[78]	https://www.rfidjournal.com/esls-are-becoming-more-affordable-for-retailers

[79]	https://www.newswire.ca/news-releases/sobeys-pilots-smart-cart-the-first-intelligent-grocery-shopping-cart-803013190.html

[80]                     https://en.wikipedia.org/wiki/Electronic_paper

[81]	F. Palacio, et al. (2006). "Active RFID tag with sensing capabilities and low power consumption".

[82]	T. O. John, H. C. Ukwuoma, S. Danjuma and M. Ibrahim, "Energy consumption in wireless sensor network", Energy, vol. 7, no. 8, 2016.

[83]	Tuna, G., and V. C. Gungor. "Energy harvesting and battery technologies for powering wireless sensor networks." Industrial Wireless Sensor Networks. Woodhead Publishing, 2016. 25-38.

[84]	M. Zeinali and J. Thompson, "Impact of Compression and Small Cell Deployment on NB-IoT Devices Coverage and Energy Consumption with a Realistic Simulation Model", Sensors. no. 19, 2021.

[y1]	Average size of a warehouse (https://www.nobroker.in/forum/what-is-the-average-size-of-a-warehouse/)

* * * Second Change * * * *

### 5.16.2	Pre-conditions

Company X is a well-known home appliance manufacturer. Apart from manufacturing various home appliances such as washing machine, television, intelligent wardrobe and so on, it also provides customized and personalized services to its customers such as sales agents. Different sales agents may order different types of home appliances with different quantities according to the demand of the region they sale to. Company X operates its own private warehouses to store manufactured goods until they are ordered. Owned by Company X, the private warehouses are customized according to its own needs, and they are typically located close to production plants. The warehouse management provides insight in inventory and by exchanging inventory information with other functional areas (e.g. logistics, trade) the overall business performance is improved. Compared with public warehousing, the typical storage area of a private warehouse is smaller, typically around 10000 square feet. This is representative according to known statistics of warehouse area dimensions [y1]. Based on these typical dimensions, indoor communication range around 20m is sufficient to provide connectivity inside the warehouse storage area.

The southern regions in Country C have many economically developed cities, while western regions consist of some developing cities. Therefore, Company X has designed different types of washing machines to meet various demands. For example, type A washing machine is intelligent, equipped with several laundry modes according to the colour, texture, material and shape of the clothes. Type B washing machine is a power saving and water saving washing machine, which achieves washing clothes with high efficiency and low power and water consumption. Sale agent A from southern region may order 100 type A washing machines and 50 type B washing machines, while sale agent B from western region may order 30 type A washing machines and 90 type B washing machines. Based on the orders from different sale agents, Company X will package the corresponding types of washing machines with corresponding quantity together. Afterwards, different packages for different sale agents will be loaded to transportation vehicles accordingly for different destinations.

Company X has a service level agreement with service provider Y to deploy 5G network with sufficient coverage in the service area to enable the communication of Ambient IoT devices with the network.

Before the transportation vehicle enters the loading stack for product transportation, it is necessary to inventory the products to be transported in the warehouse in order to ensure that the products have been recorded. During the loading process, it is required to ensure that the products are loaded to the corresponding transportation vehicles. Moreover, during the transportation, Company X may also need to monitor the transportation route by tracking the products in order to ensure that the products are transported on the right route. Otherwise, the products will be transported to the wrong place which is time and cost consuming.

* * * Third Change * * * *

### 5.16.3	Service Flows

- Company X generates in its logistics management system a list of unique IDs for Ambient IoT devices to be used for products such as washing machines. Since these washing machines are going to be distributed to different sales agents for product sales across public areas, Company X may require an authentication and authorization mechanism before Ambient IoT devices start to communication to ensure that certain level of security could then be enforced to prevent the washing machines information from being obtained by any other untrusted third-party companies. In this case, an authentication and authorization mechanism is needed for these Ambient IoT devices.

- Ambient IoT devices are attached onto washing machines.

- Before transporting the washing machines, Company X’s inventory system can request the 5G network to perform inventory and report the information of Ambient IoT devices which were attached on the washing machines in order to confirm that the types and the number of washing machines to be transported are accurate.

- Ambient IoT devices harvest power from the environment. Based on the request from the Company X’s inventory system, the 5G network transmits signals intending to start inventory process. When detecting the signals from the 5G network the Ambient IoT devices can react by responding to connect to the 5G network.

- When the Ambient IoT devices access to the 5G network successfully, the 5G network obtains the information of the Ambient IoT devices and send it to Company X’s inventory system.

- Based on the orders from several sale agents, different types of washing machines will be packaged together.

- When the washing machines are loaded to the transportation vehicle, the Company X’s inventory system can request the 5G network to perform positioning services. Since the transportation vehicles stop at fixed positions, with the positioning services, the Company X’s inventory system can monitor whether the package of the washing machines is loaded to the corresponding transportation vehicles. Since the size of the transportation vehicles is around 10m*3m, the position service should support around 3m to 5m accuracy to meet the demand for location.

- When the packages of washing machines are in transit, Company X’s inventory system can request the 5G network to periodically perform inventory and location reporting (e.g. cell level granularity) of Ambient IoT devices which were attached onto the washing machines in order to make sure that the washing machines are still on the right route. As the transportation vehicles may go to different destinations via different highways, mobile network operator O can deploy Ambient IoT services at highway toll station to perform inventory and locate the route accordingly. Because of the request from the Company X’s inventory system, the Ambient IoT devices perform authentication and authorization mechanism before sending the information in order to ensure that it is the trusted inventory system that performs the inventory of the Ambient IoT devices.

- Based on the request from Company X’s inventory system, the 5G network periodically transmits signals intending to start inventory process and record the relationship between the Ambient IoT devices and the location information. As the purpose of identifying the location during transportation is to crosscheck whether goods are on the correct routes, cell-level accuracy of location is sufficient. The 5G network reports the location information to the Company X’s inventory system.

- According to the report from the 5G network, Company X’s inventory system can locate these washing machines with the location information. If the location information is not aligned with the right route, the Company X may confirm with the staff to check whether there are mistakes. In this way, Company X can find the mistakes as soon as possible in order to decrease the cost loss.

- When the washing machines are transported to the corresponding sale agents, Company X’s inventory system can request the 5G network to perform inventory and report the information of Ambient IoT devices which were attached on the washing machines in order to confirm that the washing machines are transported to the right place.

* * * Fourth Change * * * *

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-001] The 5G system shall be able to support a mechanism to authenticate and authorize Ambient IoT devices.

[PR 5.16.6-002] The 5G system shall optimize mobility management support for non-stationary Ambient IoT devices that are unable to initiate communication towards the network.

[PR 5.16.6-003] The 5G System shall allow an operator to manage (e.g. provision, authenticate, authorise, etc.) Ambient IoT devices that have limited or no power source.

[PR 5.16.6-004] The 5G System shall be able to provide suitable and secure means to report to an authorized third-party the location of Ambient IoT devices.

[PR 5.16.6-005] The 5G System shall provide the network connection to address the KPIs for the use of Ambient IoT devices for automated supply distribution, see table 5.16.6-1.

Table 5.16.6-1: Potential key performance requirements for the use of Ambient IoT devices for automated supply chain distribution

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT devices for automated supply distribution |  | >10 s |  | 99% |  | NA |  | <100 bit/s |  | Typically, <100 bytes |  | <1,5 Million/km2 |  | 30m indoor (note 1), 400m outdoor |  | 600 000 m2 |  | NA |  | NA |  | NA |  | NA |  | 3 m (Indoor, 90% confidence level and in horizontal) , cell-level outdoor |  |
| NOTE 1: The storage area of a private warehouse is typically around 10000 square feet, which is smaller than that of a typical public warehouse. |  |  |  |  |  |  |  |  |  |  |  |  |  |