3GPP TSG SA WG 1 Meeting #102 	S1-231075

Berlin, Germany, 22 - 26 May 2023	(revision of S1-23xxxx)

Title:	Discussion on consolidated KPIs of FS_Sensing

Agenda Item:			7.1

Source:			Deutsche Telekom AG, Nokia, Ericsson

Contact:			Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: This document proposes consolidation on the KPIs from the use cases of FS_Sensing

- The study has 32 use cases. Among which, there are 21 use cases having KPIs, 11 use cases do not have KPI table. According to application needs, these use cases can be categorized into the following scenarios: object detection and tracking, environment monitoring and motion monitoring:

| Use cases | KPI? | Sensed object | Application |
| --- | --- | --- | --- |
| 5.1 intruder detection at home | Yes | People | Object detection and tracking |
| 5.2 intruder detection at highway | Yes | People/animal | Object detection and tracking |
| 5.3 rainfall monitoring | Yes | Rainfall | Environment monitoring |
| 5.4 transparent sensing | No |  |  |
| 5.5 flood monitoring | Yes | Water | Environment monitoring |
| 5.6 intruder detection at yard | Yes | People/animal | Object detection and tracking |
| 5.7 intruder detection at railway | Yes | People/animal | Object detection and tracking |
| 5.8 Automotive Maneuvering | No | Car | Object detection and tracking |
| 5.9 AGV tracking | No | AGV | Object detection and tracking |
| 5.10 UAV trajectory tracing | Yes | UAV | Object detection and tracking |
| 5.11 collision avoidance at crossroad | Yes | People/bicycle | Object detection and tracking |
| 5.12 UAV collision avoidance | Yes | UAV | Object detection and tracking |
| 5.13 UAV intruder detection | Yes | UAV | Object detection and tracking |
| 5.14 tourist spot monitoring | Yes | People | Environment monitoring |
| 5.15 sleep monitoring | Yes | People | Motion monitoring |
| 5.16 intruder detection – security part | No | People | Object detection and tracking |
| 5.17 health monitoring by coordinate sensing | No | People | Motion monitoring |
| 5.18 health monitoring with service continuity | No | People | Motion monitoring |
| 5.19 sensor group | No |  |  |
| 5.20 parking lot detection | Yes | Car | Object detection and tracking |
| 5.21 seamless XR streaming | No |  |  |
| 5.22 collision avoidance at smart grid | Yes | People/Car/UAV | Object detection and tracking |
| 5.23 AMR collision avoidance | Yes | AGV | Object detection and tracking |
| 5.24 sport monitoring | Yes | People | Motion monitoring |
| 5.25 human trajectory tracking | Yes | People | Object detection and tracking |
| 5.26 Accurate sensing | No | Car | Object detection and tracking |
| 5.27 positioning for public safety | Yes | People | Object detection and tracking |
| 5.28 ADAS | Yes | Car | Object detection and tracking |
| 5.29 Gesture recognition | Yes | Hand | Motion monitoring |
| 5.30 Automotive maneuvering out of coverage | No | Car | Object detection and tracking |
| 5.31 Blind spot detection | No | Car | Object detection and tracking |
| 5.32 positioning for smart factory | Yes | AGV | Object detection and tracking |

It is proposed within these scenarios to further group the KPIs of these use cases based on similarity of their values and complexity in the specific scenarios.

- Majority of the use cases can be considered as object detection and tracking where different service levels can be recognised based on similarity of the values in the tables.

- Sensing service level 1: Use case 5.1 and 5.13 values are exacly the same except range resolution and velocity resolution, therefore could be combined into intruder detection sensing service level 1 with additional clarifications with notes. The values for note 2 and note 3 coming from use case 5.13 are actually clarified in use case 5.10 – from there the text of the notes is taken.

- Sensing service level 2: Values for use cases 5.6 and 5.13 (one of the levels in it) seem to be quite similar, therefore combined.

- Sensing service level 3: Avoid collision use cases are combined here (5.2, 5.7, 5.10, 5.11, 5.12, 5.23).

- Sensing service level 4: More demanding values and use cases (5.20, 5.22, 5.25, 5.27, 5.32) are combined here.

- Sensing service level 5: More demanding values for ADAS (5.28) are combined here. All the values are in brackets and reference would be needed.

For the merged use cases the most demanding values without brackets have been taken. When there is big difference, more values are shown with additional text for clarification – these of course can be also merged.

- There are use cases that can be categorized as environment monitoring: tourist spot monitoring (5.14), Rainfall monitoring (5.3) and flooding (5.5).

- These are use cases that can be categorized as motion monitoring: sleep monitoring (5.15), sports monitoring (5.24), gesture recognition (5.29).

- Based on this clasification it is proposed to agree the following consolidated KPI table for the FS_Sensing study:

| Scenario | Sensing service level | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 1 (use cases 5.1; 5.13 – level1) | Object to be detected indoor: Human, object to be detected outdoor: UAV | 95 | ≤10 | ≤10 | N/A | N/A | 10 NOTE 2 | 10 NOTE 3 | <1000 | < 1 | < 5 | < 2 | intruder detection in smart home, UAV intrusion detection |
|  | 2 (use cases 5.13 – level2, 5.6) | Object to be detected outdoor: Human, UAV | 95 | ≤5 | ≤5 | N/A | N/A | 10 NOTE 2 | 10 NOTE 3 | [≤1000] | [≤1] | ≤5 | ≤5 | UAV flight route intrusion detection, intruder detection in surroundings of smart home |
|  | 3 (use cases 5.2, 5.7, 5.10, 5.11, 5.12, 5.23) | Factory (100m2), crossroad, highway, railway NOTE 4 Object to be detected: Animal, Human, UAV, Vehicle | 95 | ≤1 | N/A | 1 NOTE 5 | N/A | <1 NOTE 5 NOTE 8 | 1m/s x 1m/s NOTE 9 | ≤100 ≤1000 NOTE 6 NOTE 10 | ≥0.05 ÷ ≤ 0.1 ≤ 1 NOTE 11 | ≤2 | ≤2 | pedestrian/animal intrusion detection on a highway/railway, sensing at crossroads with/without obstacle, UAV flight trajectory tracing UAV collision avoidance, AMR collision avoidance in smart factories |
|  | 4 (use cases 5.20, 5.22, 5.25, 5.27, 5.32) | Public area safety, factory NOTE 7 Object to be detected: Animal, Human, UAV, AGV, Vehicle | 95 | 0.5 | 0.5 | 0.1 Pedestrian: ≤1.5 Vehicle: ≤15 | N/A Pedestrian: ≤1.5 | 0.5m | 5 x 5 for factories 0.5 may be needed | 250 | 0.25 ≤ 1 | 1 | 5 | Parking Space Determination, UAVs/vehicles/pedestrians detection near Smart Grid equipment, immersive experience based on sensing, public safety search and rescue or apprehend, integrated sensing and positioning in factory hall |
|  | 5 (use cases 5.28) | ADAS -TBD Object to be detected: Vehicle | [95] | ~ [0.1]-[1.3] ; for short range radar ~[0.02]-[2.6] | TBD | [±0.03] -[±0.12] m/s | N/A | [0.4] | [0.1] – [0.6] | [50] | [0.05] -[0.2] | [1-10] % | [<1] % | ADAS |
| Environment monitoring | 1 (use case 5.14) | Object to be detected outdoor: Human | 95 | [≤2] | N/A | N/A | N/A | [1] | [1] | 1000 NOTE 10 | [≤0.2] | 5 | 5 | traffic management |
|  | 2 (use cases 5.3 and 5.5.) | Rainfall monitoring and flooding NOTE 14 Object to be detected: Rain | 95 | ≤10 | [≤0.2] NOTE 15 | N/A | N/A | N/A | N/A | 1 min | 1<10min, application configurable | < 0.1 | < 3 | rainfall monitoring, flooding monitoring |
| Motion monitoring | 1 (use cases 5.15, 5.24, 5.29) | Indoor human motion -sleep monitoring NOTE 12, sports monitoring NOTE 13, gesture recognition Motion on human, gesture | 95 | N/A | N/A | 0.1 valid for gesture recognition | 0.1 valid for gesture recognition | N/A | N/A | 60s | 60, ≤0.1 for gesture recognition | 5 | 5 | sleep monitoring, sports monitoring, gesture recognition |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: To detect the UAV existance (e.g., for intrusion detection), the sensing resolution of distance is 10m [25]. NOTE 3: To detect the UAV existence, the sensing resolution of velocity is 10m/s [25]. NOTE 4: The typical size (Length x Width x Height) of UAV is 1.6m x 1.5m x 0.7m, the typical size of pedestrian is 0.5m x 0.5m x 1.75m, and the typical size of engineering vehicle is 7.5m x 2.5m x 3.5 m. NOTE 5: The KPI values for UAVs are sourced from [25] and [40] and for factories are sourced from [47]. NOTE 6: The value 100 ms is sourced from [28] and is valid for sensing at crossroads. NOTE 7: The safe distance between pedestrian/vehicle and transmission station/line is 0.7m/0.95m [46]. The size of the park of Smart Grid depends on the real environment. NOTE 8: To track the UAV flying (e.g., for collision detection and warning), the sensing resolution of distance is 1m [25]. NOTE 9: To track the UAV flying, the sensing resolution of velocity is 1m/s [25]. NOTE 10: To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1s. NOTE 11: Echodyne MESA-DAATM has approximate 1Hz scan rate [40]. NOTE 12: Additional KPI on motion rate accuracy of 2 times/min (0.033 Hz). NOTE 13: Additional KPI on motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) NOTE 14: Rainfall estimation accuracy is1 mm/h[39] and describes the closeness of the measured rainfall estimation to its true rainfall value. NOTE 15: This value is for the water level. Description related to NOTE in clause 5.5.1 suggests 0.01 m. [≤0.2] is derived from the water level where people feel difficulty in walking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex:	Sensing KPIs captured in TR 22.387 v1.0.0

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Intruder detection in smart home | Indoor | 95 | ≤10 | ≤10 | N/A | N/A | N/A | N/A | <1000 | < 1 | < 5 | < 2 |
| Pedestrian/animal intrusion detection on a highway | Outdoor (Highway) | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤5000 | ≤ 0.1 | ≤5 | ≤5 |
| Rainfall monitoring (NOTE 2) | outdoor | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 1 min | 10min, application configurable | 5 | 5 |
| sensing for flooding in smart cities | Outdoor | 95 | ≤10 | [≤0.2] | N/A | N/A | N/A | N/A | ≤ 1min NOTE 3 | < 1min NOTE 3 | < 0.1 | < 3 |
| intruder detection in surroundings of smart home | Outdoor | 95 | ≤2 | N/A | N/A | N/A | N/A | N/A | ≤1000 | < 1 | < 0.1 | < 5 |
| Intrusion detection on a railway | Outdoor (Along railway) | 95 | ≤1.5 | N/A | N/A | N/A | N/A | N/A | ˂1500 | ≤ 0.1 | 2 | 2 |
| UAV flight trajectory tracing | Outdoor | N/A | N/A | N/A | N/A | N/A | 1m x 1m ~10m x 10m | 1m/s x 1m/s ~ 10m/s x 10m/s | 100~1000 | 1Hz | 5 | 5 |
| Sensing at crossroads with/without obstacle | Outdoor | 95 | ≤1 | N/A | N/A | N/A | N/A | N/A | ≤100 | ≤ 0.1 | ≤5 | ≤5 |
| Network assisted sensing to avoid UAV collision | Outdoor | [99] | 1 | N/A | 1 | N/A | <1 NOTE 2 | 1 | 500 | 0.5 | N/A | N/A |
| UAV intrusion detection (Level 1) | Outdoor | 95 | ≤10 | ≤10 | N/A | N/A | 10 | [5] | [≤1000] | [≤1] | ≤5 | ≤5 |
| UAV intrusion detection (Level 2) | Outdoor | 95 | ≤5 | ≤5 | N/A | N/A | 10 | [5] | [≤1000] | [≤1] | ≤5 | ≤5 |
| Tourist spot traffic management | Outdoor | 95 | [≤2] | N/A | N/A | N/A | [1] | [1] | [≤5000] | [≤0.2] | ≤5 | ≤5 |
| Sleep monitoring (NOTE 3) | Outdoor (bedroom) | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 60 | 5 | 5 |
| Parking space determination | Indoor/ outdoor | 95 | 0.5 | 0.5 | 0.1 | N/A | 2.5m perpendicular to the parking space 5m parallel to the parking space | N/A | 1000 | 1 | 1 | 5 |
| Sensing for the use case in Smart Grid | Outdoor | 95 | ≤0.7 | N/A | UAV: ≤25 Pedestrian: ≤1.5 Vehicle: ≤15 | N/A | N/A | N/A | ≤5s | ≥10Hz | [≤5] | [≤5] |
| AMR collision avoidance in smart factories | 100m | 99 | ≤1 | N/A | 1 | N/A | 1 | 10 | ˂500 | ≥20Hz | N/A | 5 |
| Sports monitoring (NOTE 4) | Indoor (living room) | 95 | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 1min | N/A | N/A |
| Immersiveexperience | Indoor | 95 | 0.5 | 0.5 | 0.1 | N/A | 0.5 | N/A | 250 (Granularity of field is 1.5m x 1.5m) | 0.25 | 5 | 5 |
| Search and Rescue/Apprehend | Outdoor/Indoor | 99 | ≤ 0.5 | ≤ 1.0 | Pedestrian: ≤1.5 | Pedestrian: ≤1.5 | 3 | Horiz: 5 Vert: 5 | ≤1s | ≥10Hz | [≤3] | [≤3] |
| ADAS [long range Radar] | TBD | [95] | ~[0.1]-[1.3] | TBD | [±0.03] -[±0.12] m/s | N/A | [0.4] | [0.1] –[0.6] | [50] | [0.05] -[0.2] | [1-10]% | [<1]% |
| ADAS [Short range Radar] | TBD | [95] | ~[0.02]-[2.6] | TBD | [±0.03] -[±0.12] m/s | N/A | [0.4] | [0.1] –[0.6] | [20] | [0.02] –[0.05] | [1-10]% | [<1]% |
| Gesture recognition | Indoor | N/A | FFS | FFS | 0.1 | 0.1 | FFS | [0.3] | FFS | ≤0.1 | FFS | FFS |
| Indoor Factory | 100 m2 | 99 | [≤0.5] | N/A | 0.5 | N/A | [0.5] | [0.5] | ≤100 | 0.1 | N/A | N/A |
| NOTE 1: The terms in Table 7.2-1 are found in Section 3.1. NOTE 2: Additional KPI on rainfall estimation accuracy (for a target confidence level) of 1mm/h. NOTE 3: Additional KPI on motion rate accuracy of 2 times/min (0.033 Hz) NOTE 4: Additional KPI on motion rate accuracy of 3 times/min (0.05Hz) and 4 times/min (0.07 Hz) |  |  |  |  |  |  |  |  |  |  |  |  |