3GPP TSG SA WG 1 Meeting #102 	S1-231451

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231436)

Source:	Qualcomm, CATT

pCR Title:	Update to Use Case on  Gesture Recognition for Application Navigation and Immersive Interaction

Draft Spec:	3GPP  TR 22.837 v1.0.0

Agenda item:	7.1

Document for:	Approval

Contact:	Lola Awoniyi-Oteri oawoniyi@qti.qualcomm.com

Qing Wan wanqing1@cictmobile.com

Abstract: Updating the Use Case on  Gesture Recognition for Application Navigation and Immersive Interaction

1. Introduction

The revisions are introduce to update the use case with reasonable KPIs, a PR and remove EN and FFS from the KPI table

2. Reason for Change

This use case is revised with the following changes:

- Removing an EN in clause 5.29.6

- Proposing 1  PR in 5.29.6 (PR 5.29.6-1]) to enable processing of sensing data at the UE with sensing assistance information .

- Updating the service flow in 5.29.5 to discuss processing of sensing data at the UE

- Updating the KPIs (Position accuracy, missed detection rate, false alarm, confidence level, maximum sensing service latency and range resolution) with requirements that are achievable within 5Gtechnologies.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.837 v1.0.0.

* * * First Change * * * *

### 5.29.3	Service Flows

Social Media Navigation service with Gesture Recognition

Step 1: Sitting in this room, Jose is reading through social media posts using his smartphone, to navigate to either the previous or next posts. Jose waves his hand in the air from left to right or from right to left.

Step 2: Jose’s smartphone detects the hand gesture using 5G wireless sensing using the RAN entity, UE or both. The smartphone and RAN entity can send sensing data (with extracted gesture features such as range and Doppler of the detected gesture) to the 5G network.

Step 3: 5G network then aggregates and processes the information collected from the UE and RAN entity to detect Jose’s gesture and provides the sensing results to Jose’s smartphone which is shared with the social media application and used for the navigation of the post.

Step 2b: Alternatively, the Jose’s smartphone detects the hand gesture using sensing signals and associated information to generate 3GPP sending data and further processing to generate sensing results.

Immersive interaction service with Gesture Recognition

Step 1: Bob launches the XR application in his room, at which moment one avatar is generated to represent him in the virtual world of the XR application, and the immersive interaction service is activated as shown in Fig 5.29.3-1.

Step 2: When Bob sees a basketball flying toward him, he bends the fingers to catch the ball and throws it back. The characteristics of the gesture (e.g. range, doppler shift) are detected using NR sensing signals of UEs, the RAN entity or both.

Step 3: 5G network (e.g. 5GC) collects the 3GPP sensing data from UEs, RAN entity or both, process the data and exposes the sensing result (e.g. 3D position, velocity) to XR application. In parallel, the non-3GPP sensing data obtained from the XR device is transmitted to the application platform transparently through UE to 5GC.

Step 4: The gesture and hand movement are recognized and the basketball bounces to another direction as a sensing result. The entire course will be presented on Bob’s headset as that the basketball is caught by one hand of Bob’s avatar and thrown back.

* * * Next Change * * * *

### 5.29.6	Potential New Requirements needed to support the use case

[PR 5.29.6-1] Subject to regulation, operator’s policy, user consent and under operator’s control, the 5G system shall be able to authorize a UE to securely process locally the 3GPP sensing data collected by the same UE and sensing assistance information to derive sensing results.

[PR 5.29.6-2] The 5G system shall be able to provide sensing with the following KPIs:

Table 5.29.6-1	Performance requirements of sensing results for gesture recognition

| Scenario | Sensing service area | Confidence level [%] | Motion rate accuracy | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Gesture recognition | Indoor | 95 | N/A | 0.2 NOTE 5 | 0.2 NOTE 5 | 0.1 | 0.1 | 0.375 NOTEs 2 and 3 | 0.3 NOTE 6 | [5] NOTE 4 | ≤0.1 | ≤5 | ≤5 |
| NOTE 1: The terms in Table 5.29.6-1 are found in Section 3.1. NOTE 2: The KPIs above assume the use of pre-defined gestures for determining a user’s gestures. Also, the sensing result for this type of gesture recognition allows basic hand gestures and not finger gestures recognition. NOTE 3: The range resolution is determined assuming 400 MHz bandwidth which is supported in 5G mmWave networks using formular C/2*B where C is the speed of light, B is the bandwidth. NOTE 4: The value is derived from TS22.261 [33] clause 7.11 about max allowed end-to-end latency for immersive multi-modal KPIs. NOTE 5: Positioning accuracy KPIs are based on minimum supported positioning accuracy in 5G systems [TS 22.261] NOTE 6: Velocity resolution info. for gesture monitoring is taken from the reference [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *