3GPP TSG SA WG 1 Meeting #102 	S1-231482

Berlin, Germany, 22 – 26 May 2023	   (revision of S1-231165, 427, 454)

Source:	vivo

pCR Title:	Resolve EN regarding motion rate accuracy

Draft Spec:	3GPP TR22.837

Agenda item:	7.1

Document for:	Approval

Contact:	Amy Zhang, amy.zhang@vivo.com

Abstract: This document provides a text proposal to resolve the EN.

1. Introduction

Resolve EN: delete EN and mention this parameter is only useful for use case 5.15 and use case 5.24

2. Reason for Change

EN about how to name “motion rate accuracy” is left to be resolved.

Modify motion rate accuracy to more specifically to human motion rate accuracy.

Corresponding KPI parameter modifications in Tables of use cases 5.15 and 5.24

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.837 v1.0.0.

* * * First Change * * * *

## 3.1	Terms

Human motion rate accuracy describes the closeness of the measured value of the human body movement frequency caused by part(s) (e.g. chest) of the target object (i.e. human body) to the true value of the human body movement frequency.

* * * Second Change * * * *

### 5.15.6	Potential New Requirements needed to support the use case

[PR.5.15.6-1] The 5G system shall support mechanisms to discover and configure a UE and a base station to perform sensing measurements in a certain sensing service location area.

[PR.5.15.6-2] The 5G system shall support mechanisms to derive and expose sensing results to a trusted 3rd party.

[PR.5.15.6-3] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

Table 5.15.6-1	Performance requirements of sensing results for contactless sleep monitoring

| Scenario | Sensing service area | Confidence level [%] | Human motion rate accuracy [Hz] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sleep monitoring | Outdoor (bedroom) | 95 | 0.033 (NOTE 2) | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 60 | 5 (NOTE3) | 5 (NOTE3) |
| NOTE 1: The terms in Table 5.15.6-1 are found in Section 3.1. NOTE 2: Respiration rate = 18 times/min as reference, any detected value in [16,20] satisfies accuracy requirements, 0.033Hz corresponds to 2 times/min NOTE 3: Detect event = “breathing stoppages duration >= 10 seconds” as reference |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * *  Third Change * * * *

### 5.24.6	Potential New Requirements needed to support the use case

[PR 5.24.6-1] Based on operator policy, the 5G System shall be able to provide the 5G wireless sensing services in case of roaming.

[PR.5.24.6-2] The 5G system shall be able to provide 5G wireless sensing service with the following KPIs:

Table 5.24.6-1	Performance requirements of sensing results for sports monitoring

| Scenario | Sensing service area | Confidence level [%] | Human motion rate accuracy [Hz] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency[ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |
| Sports monitoring | Indoor (living room) | 95 | 0.05 (NOTE 2) 0.07 (NOTE 3) | N/A | N/A | N/A | N/A | N/A | N/A | 60s | 1min | N/A | N/A |
| NOTE 1: The terms in Table 5.24.6-1 are found in Section 3.1. NOTE 2: Sit-up rate = 30 times/min as reference, 0.05Hz corresponds to 3 times/min NOTE 3: Push-up rate = 40 times/min as reference, 0.07Hz corresponds to 4 times/min |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *