3GPP TSG SA WG 1 Meeting #102 	S1-231484

Berlin, Germany, 22 - 26 May, 2023	(revision of S1-231362, S1-231273 & S1-231226)

Source:	Huawei, DENSO

pCR Title:	Pseudo-CR on resolving the Editor’s Note in the Use case for out-of-coverage 5G Wireless sensing for automotive maneuvering and navigation service

Draft Spec:	3GPP TS 22.837 v1.0.0

Agenda item:	7.1

Document for:	Approval

Contact:	Edward Hall (edward.hall@huawei.com)

Chuting Yao (yaochuting@huawei.com)

Abstract: This paper proposes to remove the Editor’s Note in 5.30.

1. Introduction

In the use case of sensing for automotive maneuvering and navigation service when not served by RAN, NR wireless sensing is utilized to assist automotive maneuvering. There is an Editor’s Note on a potential requirement to introduce service continuity.

2. Reason for Change

In the use case of sensing for automotive maneuvering and navigation service when not served by RAN, NR wireless sensing is utilized to assist automotive maneuvering. In order to resolve the Editor’s Note on [PR 5.30.6-3], this paper proposes to remove [PR 5.30.6-3].

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.837 v1.0.0.

* * * First Change * * * *

## 5.30	Use case on sensing for automotive maneuvering and navigation service when not served by RAN

### 5.30.1	Description

Consider the scenario defined in section 5.8, where NR wireless sensing is utilized to assist automotive maneuvering, i.e. sensing results play an important role in making the maneuvering decisions. However, in this use case the vehicles are not served by RAN when the Sensing activity is expected to occur, where UE is not served by RAN of the its authorized PLMN’s RAN entities, and therefore RAN entities are unable to be involved. UEs collecting the sensing measurement process in this scenario have to be able to operate when UE is not served by RAN.

### 5.30.2 	Pre-conditions

Refer to 5.8, where Bob’s vehicle determines the need for sensing service. Bob’s vehicle integrates a UE supporting V2X application ie. Bob’s vehicle supports V2X. Sensing can be performed by 5G Wireless sensing. Unlike in clause 5.8, the vehicles are not served by RAN and perform 5G Wireless sensing without help of the network entities. Bob’s vehicle has been provisioned by his home operator to be able to perform 5G Wireless sensing when not served by RAN. There may be other vehicles or RSU (road side unit) helping the 5G Wireless sensing of the Bob’s vehicle.

### 5.30.3		Service Flows

1. Bob is driving from urban to rural countryside. As his vehicle is operational, in motion, and attempting to assist his driving using ADS, Bob’s vehicle is performing sensing using 5G Wireless sensing.

2.Bob drives his vehicle outside the coverage area of its mobile network. The 5G Wireless sensing continues providing assistance to Bob’s driving.

3. Bob’s vehicle approaches Joe’s vehicle.

4. Bob’s vehicle becomes aware of Joe’s vehicle by Bob’s onboard Sensing receiver(s) receiving 5G Wireless sensing reflected by Joe’s vehicle.

5. Bob’s vehicle applies this new object information to the local ADS function to ensure a safe, collision-free, drive.

### 5.30.4	Post-conditions

Bob’s vehicle is able to drive with high reliability by utilizing 5G sensing service when not served by RAN.

### 5.30.5		Existing features partly or fully covering the use case functionality

None.

### 5.30.6		Potential New Requirements needed to support the use case

[PR 5.30.6-1] The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing operation when not served by RAN.

[PR 5.30.6-2] Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum.

* * * End of Change * * * *