3GPP TSG-SA WG1 Meeting #102 	S1-231516

Berlin, Germany, 22 - 26 May 2023	revision of S1-231106

Title:	CR on mobility between naval non-public networks

Agenda Item:	7.12

Source:	b<>com, Airbus, Novamint

Contact:	Mathieu Lagrange, mathieu.lagrange@b-com.com

Abstract: This change request provides a use case related to supporting mobility between naval non-public networks.

## x.2	Use case Mobility in a group of interconnected standalone naval non-public networks

### x.2.1	Description

This use case deals with self-organizing, broadband, low latency, connectivity solution between ships in a fleet. It is an extension of the use case x.1 on interconnection of standalone naval non-public networks.

This network architecture, unusual in the world of public cellular networks, will however have to support all the mobility functions supported by terrestrial networks. In particular, a terminal will have to be able to move freely from one vessel to another, either in standby or while being actively involved in a communication. This movement should not be perceptible to the user.

The situation described in this use case is depicted on the figure below:

### x.2.2	Pre-conditions

User A terminal is loaded with a USIM that is registered to SNPN A.

SNPA A has local coverage on-board ship A and SNPN B has coverage on-board ship B.

Service B is offered by an application B hosted on ship B. Application B has 5G connectivity through SNPN B.

User A has needed credentials to access service B.

### x.2.3	Service Flows

- User A on-board ship A and attached to the on-board SNPN A establishes a PDU session to access service B.

- User A takes a speed boat, equipped with necessary communication equipment to deploy locally SNPN around the speed boat and to enable connectivity with any vessels in the fleet through wireless links. Once it is deployed, the on-board SNPN selects the SNPNs in range, identifies and authenticates itself to those SNPNs. The SNPN on board the speed boat is then added in the group of interconnected SNPNs through wireless links.

- As the speed boat moves away from ship A, radio measurements made by user A terminal trigger the 5G system to handover the ongoing communication from SNPN A to the SNPN on-board the speed boat.

- At this point, user A terminal, although under radio coverage of the SNPN in the speed boat, keeps consuming service B through an interconnection path made of speed boat SNPN + SNPN A + SNPN D + SNPN B..

- Somewhere along the path from ship A to ship B, as radio conditions on the 5G link to ship A worsen and as the speedboat approaches ship D, the SNPN on-board the speed boat selects SNPN D as a new anchoring point to the group of interconnected standalone non-public networks. Based on the new resulting topology, the 5G system then seamlessly reconfigures the interconnection path from user A’s UE to service B so that it can still continue to fulfil QoS required by service B.. The interconnection path is now made of speed boat SNPN + SNPN D + SNPN B.

- Later on, as the speedboat approaches ship B, the on-board tacSNPN directly connects to SNPN B in the same anchoring point selection and interconnection path switching process as previously described to reduce communication latency.

- From the point in time user A gets on board the speedboat and gets to ship B, his communication with service B is maintained. There is no noticeable alteration of the communication along the path from ship A to ship B.

- When user A arrives at his destination on ship B, his communication is seamlessly handed over from the SNPN on-board the speedboat to one of the cells on SNPN B. Once connected to SNPN B, he can still access to service B and other local services according to authorizations he’s entrusted with.

### x.2.4	Post-conditions

User A is attached to SNPN B and still in communication with service B.

### x.2.5	Existing features partly or fully covering the use case functionality

Editor’s note : section left for further study

### x.2.6	Potential New Requirements needed to support the use case

[PR.5.x.6-004]: The 5G system shall be able to support discovery, addition and authentication, or detachment of standalone non-public networks in a group of interconnected SNPNs through wireless links without interrupting ongoing communications.

[PR.5.x.6-002]: The 5G system shall be able to support measurements allowing to change the interconnection points of a standalone non-public network in a group of interconnected SNPNs through wireless links

[PR.5.x.6-003]: The 5G system shall be able support seamless handover of communications between newly collocated and interconnected standalone non-public networks.

Editor’s note : Performance requirements are left for further study