3GPP TSG SA WG 1 Meeting #102 	S1-231523

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231139)

Source:	CATT

pCR Title:	Terminology Adaption and Alignment

Draft Spec:	3GPP TR 22.851 v1.1.0

Agenda item:	7.4

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This document proposes to adapt the new defined term to associated requirements in TR22.851.

1. Introduction

Editorial change to adapt new defined terms to the potential requirements of TR.

2. Reason for Change

Editorial change: Adapt the terms to the PRs, align the definition

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.851 v1.1.0

* * * First Change * * * *

5.1.6	Potential New Requirements needed to support the Use Case

[PR 5.1.6-001] The 5G system shall be able to support network sharing with indirect connection between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks.

[PR 5.1.6-002] The 5G system shall be able to support means for Participating Operators to provide their operator name to a registered UE, for display to the user.

* * * Next Change * * * *

5.2.6	Potential New Requirements needed to support the Use Case

[PR 5.2.6-001] The 5G core network shall be able to support collection of charging information associated with a UE accessing a Shared NG-RAN  using Indirect Network Sharing.

[PR 5.2.6-002] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall support service continuity for UEs that are moving between different Shared NG-RANs or between a Shared NG-RAN and a non-shared RAN (managed by Hosting and Participating Operators).

[PR 5.2.6-003] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall be able to minimize the impact to the user experience (e.g. QoS, QoE) of UEs in an active communication moving between different Shared NG-RANs or between a Shared NG-RAN and a non-shared RAN (managed by Hosting and Participating Operators).

[PR 5.2.6-004] In case of Indirect Network Sharing, the 5G system shall support a mechanism for a UE to access the subscribed PLMN services when entering a Shared NG-RAN.

* * * Next Change * * * *

5.3.6	Potential New Requirements needed to support the Use Case

[PR 5.3.6-001] In case of indirect network sharing, the 3GPP system shall support mechanisms to minimize service interruptions for UEs that are moving between different Shared RANs or between a Shared RAN and a non-shared RAN (managed by Hosting and Participating Operators).

[PR 5.3.6-002] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall support access control for a UE accessing a Shared NG-RAN.

[PR 5.3.6-003] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall be able to select an appropriate radio access network (i.e. 4G, 5G) for a UE.

[PR 5.3.6-004] In case of Indirect Network Sharing and subject to Hosting and Participating Operators’ policies, the 3GPP system shall support a mechanism to enable a UE with a subscription to a Participating Operator to access an authorized Shared NG-RAN.

* * * Next Change * * * *

5.4.6	Potential New Requirements needed to support the Use Case

[PR 5.4.6-001] The 5G system shall enable Shared NG-RAN of a Hosting Operator with the indirect connection between Shared NG-RAN and a Participating NG-RAN Operator’s core network to provide services for inbound roaming users.

NOTE:  Inbound roaming users mentioned above refer to the subscribers of a foreign operator having a roaming agreement with one participating operator.

* * * Next Change * * * *

5.6.6	Potential New Requirements needed to support the Use Case

[PR 5.6.6-001] Subject to national or regional regulatory and operator policies, the 5G system shall support emergency call from a UE accessing a Shared NG-RAN via Indirect Network Sharing.

[PR 5.6.6-002] Subject to national or regional regulatory and operator policies, the 5G system shall provide to the Participating Operator the location information of an emergency caller accessing a Shared NG-RAN via Indirect Network Sharing.

* * * Next Change * * * *

5.8.6	Potential New Requirements needed to support the Use Case

[PR 5.8.6-001] Subject to regulatory requirements and mutual agreement between the Participating Operators and the Hosting Operator, the 5G system shall be able to support PWS when connectivity is provided via Indirect Network Sharing.

[PR 5.8.6-002] Subject to regulatory requirements and mutual agreement between the Participating Operators and the Hosting Operator, Shared NG-RAN in Indirect Network Sharing shall be able to broadcast PWS messages originated from the core network of the Hosting Operator.

* * * End of Changes * * * *