3GPP TSG- SA1 Meeting # 102 	S1-231555

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231544, 1440, 1128, 0448)

Title:	Consolidation requirements on FS_EnergyServ

Agenda Item:	7.10

Source:	China Mobile, Samsung

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Abstract: This pCR provides consolidation requirements on FS_EnergyServ.

Energy consumption as service criteria: 5.1, 5.5

Different energy states of network elements and network functions: 5.2

Monitoring and measurement related to energy efficiency: 5.1, 5.3, 5.4, 5.9, 5.10, (5.6, 5.8, 5.9 with FFS)

Information exposure related to energy consumption: 5.3, 5.4, 5.5, 5.9, 5.10, (5.6, 5.7, 5.8, 5.9 with FFS)

6	Consolidated Potential Requirements

Editor's Note: The following consolidated potential requirements will be revised and updated as discussion going on to cover updates in existing use cases and potential requirements in new use cases.

6.1	Energy consumption as service criteria

This subclause contains the requirements related to energy consumption as service criteria and supporting energy credit limit for specific service.

Table 6.1-1 – Consolidated requirements on energy consumption as service criteria

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.1-1 | Subject to operator’s policy, the 5G system shall support subscription policies and means to enforce the policy that define a maximum energy consumption rate for services without QoS criteria. NOTE x: The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services. | PR 5.1.6-1, PR 5.1.6-2 | merge |
| CPR 6.1-2 | The 5G network shall support a means to define maximum energy consumption rate with specific granularities (which include subscriber granularity, network slice granularity). | PR 5.1.6-3 |  |
| CPR 6.1-3 | Subject to operator’s policy, the 5G system shall support subscription policies that define a maximum energy credit limit for services. NOTE x: The definition of subscription is in TS 21.905. | P.R 5.5.6-1 | Definition of subscription is in TS 21905 maximum energy credit limit: a policy establishing an upper bound on the quantity of energy used by the 5G system to provide services provided to a specific subscriber.(clause 3.1) |
| CPR 6.1-4 | Subject to operator’s policy, the 5G system shall support a means to associate energy consumption with charging information based on subscription policies. | P.R 5.5.6-2 | Charging aspect |
| CPR 6.1-5 | Subject to operator’s policy, the 5G system shall support a mechanism to perform energy consumption credit limit control for services. NOTE x: The result of the credit control is not specified by this requirement. NOTE x: Credit control [18] compares against a credit control limit. It is assumed charging events are assigned a corresponding energy consumption and this is compared against a policy of energy credit limit. It is assumed there can be a new policy to limit energy consumption allowed. | P.R 5.5.6-4 |  |

6.2	Different energy states of network elements and network functions

This subclause contains the requirements related to different energy states of network elements and network functions and dynamic changes.

Table 6.2-1 – Consolidated Requirements on different energy states of network elements and network functions

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | The 5G system shall support different energy states of network elements and network functions. | PR 5.2.6-1 |  |
| CPR 6.2-2 | 5G system shall support dynamic changes of energy states of network elements and network functions. NOTE x: This requirement also include the condition when providing network elements or functions to an authorised 3rd party, the dynamic changes can be based on pre-configured policy (the time of changing energy states, which energy state map to which level of load, etc.) | PR 5.2.6-2 |  |
| CPR 6.2-3 | The 5G system shall support different charging mechanisms based on the different energy states of network elements and network functions. | PR 5.2.6-3 | Charging aspect |
| NOTE x: These requirements assume it is possible that there is new energy states of network elements and network functions. |  |  |  |

6.3	Monitoring and measurement related to energy efficiency

This subclause contains the requirements of monitoring and measurement related to energy efficiency .

Table 6.3-1 –Consolidated Requirements on monitoring and measurement related to energy efficiency

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to operator's policy, the 5G network shall support energy consumption monitoring at per network slice and per subscriber granularity. NOTE x: Energy consumption monitoring as described in the preceding requirement is done by means of averaging or applying a statistical model. The requirement does not imply that some form of 'real time' monitoring is required. | PR 5.1.6-4 |  |
| CPR 6.3-2 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to acquire energy consumption information of the network functions serving this 3rd party, independently from NG-RAN deployment scenarios. | PR.5.3.6-1 |  |
| CPR 6.3-3 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to acquire energy consumption information of the NPN serving this 3rd party, including the shared network function(s) which is (are) serving the NPN. | PR.5.4.6-1 |  |
| CPR 6.3-4 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to acquire the ratio of renewable energy used to provide dedicated communication service to this 3rd party on periodic basis. NOTE x: The reporting period could be set, e.g., on monthly or yearly basis. | PR.5.9.6-2 |  |
| CPR 6.3-5 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to acquire the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s network energy consumption, and the carbon intensity of operator’s network. NOTE x: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE x: The granularity of reporting (e.g., per month) is not discussed in this work. | PR 5.10.6-1 |  |

Editor's Note: The potential requirements PR 5.6.6-1, PR 5.8.6-1, PR 5.9.6-1 will be considered to consolidation after the ENs are resolved.

6.4	Information exposure related to energy consumption

This subclause contains the requirements related to information exposure related to energy consumption.

Table 6.4-1 – Consolidated Requirements on information exposure related to Energy Usage

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information of the network functions serving this 3rd party, independently from NG-RAN deployment scenarios to authorized 3rd parties. | PR.5.3.6-1 |  |
| CPR 6.4-2 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information of the NPN serving this 3rd party, including the shared network function(s) which is (are) serving the NPN to authorized 3rd parties. | PR.5.4.6-1 |  |
| CPR 6.4-3 | Subject to operator’s policy, the 5G system shall support a means to expose energy consumption to authorized third parties for services, including energy consumption information related to the condition of energy credit limit (e.g. when the energy consumption is reaching the energy credit limit). | P.R 5.5.6-3 |  |
| CPR 6.4-4 | Subject to operator’s policy and agreement with 3rd party, the 5G system shall be able to provide to authorised 3rd parties the reporting of the ratio of renewable energy used to provide dedicated communication service to this 3rd party on periodic basis. NOTE x: The reporting period could be set, e.g., on monthly or yearly basis. | PR.5.9.6-2 |  |
| CPR 6.4-5 | Subject to user consent, operator policy and regulatory requirements, the 5G system shall be able to provide a mechanism to expose to the authorized third parties the energy efficiency information (e.g., including the estimated carbon emissions) related to a subscriber based on the subscriber’s data volume over a specific period of time, the operator’s network energy consumption, and the carbon intensity of operator’s network. NOTE x: The carbon intensity of operator’s network can be provided by an authorized third party and can vary based on locations. NOTE x: The granularity of reporting (e.g., per month) is not discussed in this work. | PR 5.10.6-1 |  |

Editor's Note: The potential requirements PR 5.6.6-1, PR 5.7.6-1, PR 5.7.6-2, PR 5.8.6-2, PR 5.9.6-1 will be considered to consolidation after the ENs are resolved.