3GPP TSG SA WG 1 Meeting #102 	S1-231702

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231568, S1-231383, S1-231087)

Source:	Novamint, Huawei, ETRI, Nokia, Gatehouse, China Telecom, CATT

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.865

Draft Spec:	3GPP TR 22.865 1.0.0

Agenda item:	7.7

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in S1-231086.

1. Introduction

The “Consolidated Requirements” of TR 22.865 are currently not included in version 1.0.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.865, capturing a set of final consolidated potential requirements, based on associated discussion paper and proposal in  S1-231382.

The document  was updated into S1-231383 to merge the documents S1-231072 & S1-231224.

Then S1-231383 was revised and split into 3 documents:

- S1-231568/1702 – potential CPR agreeable at this meeting (present document)

- S1-231569 - potential CPR for further discussion at this meeting

- S1-231570 - potential CPR for next meeting

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.865 v1.0.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | Subject to operator’s policies, a 5G system with satellite access shall be able to support Store & Forward Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data when feeder link between the satellite and the ground segment is available. | [PR 5.1.6-001] [PR.5.3.6-001] [PR 5.4.6-001] |  |
| CPR 6.2-2 | A 5G system with satellite access shall be able to inform a UE whether Store & Forward Satellite operation is applied. | [PR 5.1.6-002] |  |
| CPR 6.2-3 | Subject to operator’s policies, a 5G system with satellite access supporting Store and Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period. | [PR 5.1.6-003] [PR 5.2.6-002] |  |
| CPR 6.2-4 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota. | [PR 5.1.6-004] [PR 5.2.6-003] [PR.5.3.6-003] [PR 5.4.6-004] |  |
| CPR 6.2-5 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to store and forward operation (e.g. forwarding priority, acknowledgment policy). | [PR 5.1.6-005] [PR 5.2.6-004] |  |
| CPR 6.2-6 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to inform an authorized UE about how long the data received from the UE will be stored before being delivered. | [PR.5.16.6-001] |  |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication (without going through the ground network) are listed in the table below

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support communication between UEs via one or multiple serving satellites without going through the ground network. NOTE: UEs can be connected via their HPLMN or a VPLMN. | [PR 5.6.6-003] [PR 5.7.6-001] |  |
| CPR 6.3-2 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites). | [PR 5.6.6-004] [PR 5.14.6-001] |  |
| CPR 6.3-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of the communication between UEs not going through the ground network when the communication path between UEs extends from one to several satellites (through inter satellite links). | [PR 5.14.6-002] |  |
| CPR 6.3-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of the communication between UEs using satellite access without going through the ground network. | [PR 5.8.6-001] |  |
| CPR 6.3-5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of communication (e.g. services including unicast, multicast, broadcast) using satellite access without going through the ground network. | [PR 5.8.6-002] |  |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE registered to a HPLMN or a VPLMN independently of the UE’s GNSS capability. | [PR 5.11.6-001] [PR 5.11.6-002] |  |
| CPR 6.4-2 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies, based on the information from reliable and trusted sources) in order to provide services. | [PR 5.11.6-003] |  |
| CPR 6.4-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. | [PR 5.13.6-001] [PR 5.12.6-001] |  |
| CPR 6.4-4 | A 5G system with satellite access shall be able to provide to a UE using only satellite access with information on positioning services (e.g. supported positioning performance). | [PR 5.12.6-002] |  |
| CPR 6.4-5 | A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE or network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS). | [PR 5.13.6-002] |  |

## 6.5	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below

Table 6.5-1 – Consolidated Requirements for general aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.5-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. | [PR 5.5.6-001] [PR 5.5.6-002] [PR 5.15.6-001] |  |
| CPR 6.5-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite. | [PR 5.9.6-001] |  |

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-1 | Subject to operator’s policies, a 5G system with satellite access supporting Store & Forward Satellite operation shall be able to support forwarding of the stored data received from authorized UEs using S&F operation from one satellite to another satellite through Inter-Satellite Links while preserving security of the data. | [PR.5.3.6-002] |  |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-1 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites). | [PR 5.4.6-005] [PR 5.4.6-006] |  |