3GPP TSG-SA WG1 Meeting #101 	S1-231511

Berlin, Germany, 22 – 26 May, 2023

Source:	OPPO

pCR Title:	Consolidation on KPI table of AIML-Ph2

Draft Spec:	3GPP TR 22.840

Agenda item:	7.6

Document for:	Approval

Contact:	Yang Xu (xuyang@oppo.com)

Abstract: it is proposed to make CPR mapping table for consolidation of AIML-Ph2.

Discussion

Based on all functional requirements captured in the latest version of TR 22.876v020, it made a mapping table for consolidation.

****************Start of Change*************

8	Consolidated potential requirements and KPIs

8.2	Consolidated KPIs

8.2.1			Split AI/ML operation between AI/ML endpoints

Table 8.1-1 KPI Table of Split AI/ML operation between AI/ML endpoints for AI inference by leveraging direct device connection

| Max allowed UL end-to-end latency (NOTE 1) | UL Payload size (Intermediate data size) | UL Experienced data rate | Service area dimension | Communication service availability | Reliability | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 2–100 ms | 0.1-1.5 Mbyte for each frame | 4.8-720 Mbps |  |  |  | Proximity-based worktask offloading for Remote driving, AR displaying/gaming, remote-controlled robotics, video recognition and One-shot object recognition |
| 10 ms | 0.000016 – 1.6 MByte (8 bits data format) | 0.128 - 1.28 Gbps | 900 m2 (30 m x 30 m) | 99.999 % | 99.999 % | Local AI/ML model split on factory robots |
| 10 ms | 0.000064 – 6.4 Mbyte (32 bits data format) | 0.512 - 5.12 Gbps |  |  |  | Local AI/ML model split on factory robots |
| NOTE 1: For AIML model splitting, the UL intermediate data size is much bigger than DL result data size. |  |  |  |  |  |  |

8.2.2			AI/ML model/data distribution and sharing by leveraging direct device connection

Table 8.1-1 KPI Table of AI/ML model/data distribution and sharing by leveraging direct device connection

| Max allowed DL end-to-end latency (NOTE 1) | Experienced DL data rate | DL Payload size | Communication service availability | Remark |
| --- | --- | --- | --- | --- |
| 1s | 0.14 - 1.92 Gbit/s | 18 - 240 MByte | 99.9 % | AI Model Transfer Management through Direct Device Connection |
| 3s | 5 – 81.33 Mbyte/s | 15 – 244 MByte | - | transfer learning for trajectory prediction |
| NOTE 1: the model distribution mainly requires an KPI for DL data transmission NOTE 2: The AI/ML model data distribution is for a specific application service |  |  |  |  |

8.2.3			Distributed/Federated Learning by leveraging direct device connection

Table 8.1-1 KPI Table of Distributed/Federated Learning by leveraging direct device connection

| Ul/DL Payload size (NOTE 1) | Maximum UL/DL latency | Experienced UL/DL data rate | Reliability | Remark |
| --- | --- | --- | --- | --- |
| 132 MByte | 2-3 s | 352 – 528 Mbit/s |  | Direct device connection assisted Federated Learning (Uncompressed model) |
| 132 MByte |  |  |  | Asynchronous Federated Learning via direct device connection |
| 18 – 50 MByte | 1 s | 140 – 220 Mbit/s | 99.99% |  |
| NOTE 1: For Distributed/federated learning, the UL and DL data size are almost the same due to the training gradient (UL) and global model (DL) are corresponding to the same model and have the same number of parameters |  |  |  |  |

*******************End of Change*****************