3GPP TSG SA WG 1 Meeting #102 	S1-231747

Berlin, Germany, 22 - 26 May 2023	(revision of S1-231490, S1-231368, S1-231318)

Title:	Pseudo-CR on Consolidated requirements of FS_Sensing

Agenda item:	7.1

Source:	Deutsche Telekom AG, ZTE

Contact:	Vasil Aleksiev, vasil dot aleksiev at magenta dot at

Abstract: Based on DP S1-231317, this pCR proposes to agree consolidated requirements section of the FS_Sensing study.

# 7	Consolidated potential requirements and KPIs

## 7.1	Consolidated functional requirements

Table a – Sensing configuration and authorization Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | Subject to regulation, the 5G system shall be able to configure and/or authorize sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service. NOTE: Such configuration and authorization can be based on e.g. sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data. | PR 5.2.6-3 PR.5.15.6-1 PR 5.9.6-2 PR 5.20.6-2 PR 5.8.6-1 PR 5.28.6-1 |  |
|  | The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing service when not served by RAN. | PR 5.30.6-1 |  |

Table b – 5G Wireless sensing service Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall be able to provide 5G wireless sensing service in a sensing service area location using sensing transmitters and sensing receivers. | PR 5.2.6-1 PR 5.9.6-1 PR 5.11.6-1 P.R 5.14.6-1 PR 5.1.6-2 |  |
|  | Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g. network load). | P.R 5.14.6-3 P.R 5.13.6-7 PR 5.8.6-6 |  |
|  | Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network for processing. | PR.5.4.6-1 PR 5.21.6-2 |  |
|  | The 5G system shall support continuity for 5G wireless sensing service (e.g. for sensing a moving object). | PR 5.23.6 -1 PR 5.18.6.1 |  |
|  | Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming. | PR 5.24.6-1 |  |
|  | Subject to user consent, regulation, and operator’s policy, the 5G system should support the combination of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result. | PR 5.21.6-3 PR 5.4.6-4 PR 5.27.6-2 |  |
|  | Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services (e.g. prioritizing between communication and sensing services). | PR 6.2.2-1 |  |
|  | The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service. | PR 5.25.6-3 PR 5.1.6-5 PR 5.25.6-2 |  |
|  | Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum. | PR 5.30.6-2 |  |
|  | Subject to operator’s policy, the 5G system may be able to use sensing assistance information received from a trusted third-party to derive the sensing result. | PR 5.22.6-2 |  |

Table c – Network exposure Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met. NOTE: These conditions could be e.g. the target object distance from the restricted area border or entering restricted area. | PR 5.13.6-6 |  |
|  | Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a service from application server. | PR 5.19.6-2 |  |
|  | Subject to operator’s policy, the 5G network may enable secure means for a trusted third party to provide assistance information about a sensing target. | PR 5.32.6-1 |  |
|  | The 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g. refresh rate, period of time, sensing KPIs, geographical location) and to receive the respective corresponding sensing results. | PR 5.11.6-3 PR 5.13.6-4 PR 5.12.6-3 PR 5.12.6-5 PR 5.12.6-4 PR 5.5.6-1 PR 5.5.6-2 |  |
|  | Subject to operator’s policy, the 5G system shall be able to provide secure means for a trusted third-party application to receive sensing results with contextual information. | PR 5.8.6-5 |  |

Table d – Security Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall provide a mechanism to protect identifiable information that can be derived from the 3GPP sensing data from eavesdropping. | PR 5.16.6-1 |  |
|  | The 5G system shall limit the exposure of the sensing results only to third party authorized to receive that sensing results. | PR 6.1.2-1 |  |
|  | The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system. | PR 5.27.6-4 PR 5.27.6-3 PR 5.23.6 -2 PR 6.1.2-2 |  |
|  | The 5G system shall support appropriate sensing KPIs of 5G wireless sensing for both situations where consent can be obtained from the sensing targets, and where it cannot. | PR 6.1.2-3 |  |