3GPP TSG SA WG 1 Meeting #102 	S1-231812

Berlin, Germany,  22 - 26 May 2023	(revision of S1-231785)

Source:	Qualcomm, Futurewei, Wiliot, KPN

pCR Title:	Update to Use Case on Finding Lost Item

Draft Spec:	3GPP  TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: Updating KPIs in clause 5.8

1. Introduction

Updating use case by updating KPIs.

2. Reason for Change

- Adding values for postioning service availabillity and position accuracy KPIs

- Removed the comparators on the KPI values

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0

* * * First Change * * * *

### 5.8.6	Potential New Requirements needed to support the use case

[PR.5.8.6-001] The 5G system shall be able to assist an Ambient IoT device with discovery and communication with 5GS entities that can provide location related information. [PR 5.8.6-002] Based on operator policy, the 5G system shall be able to support authorization of UEs communicating with an Ambient IoT device.

[PR.5.8.6-003] The 5G system shall be able to support means to support RAN entities and authorized UEs to communicate with Ambient IoT devices and transfer related information to other 5G system entities (e.g., core network) / servers.

[PR.5.8.6-004] The 5G system shall be able to provide a mechanism to protect the privacy of information (e.g., location and identity) exchanged during communication with an Ambient IoT device.

NOTE 1:	This requirement refers to communication between Ambient IoT devices and 5G System entities (e.g., core network, RAN entities), application servers or authorized UEs.

[PR 5.8.6-005] The 5G system shall be able to support a UE to authenticate an Ambient IoT device.

[PR 5.8.6-006] The 5G system shall be able to support Ambient IoT devices with the following KPIs:

Table 5.8.6-1:  KPI Requirements for “Finding Remote Lost Item” Service

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Remote lost item finding (Indoor) | 5s | 99% (Note 1 ) | NA | NA | 256 bits (Note 2 ) | 750 devices/100m2 (Note 3) | 10m | NA | NA | NA | NA | 90% | ~3m |
| Remote lost item finding (Outdoor) | 5s | 99% (Note 1) | NA | NA | 256 bits (Note 2) | 750 devices/100m2 (Note 3) | 100m | NA | NA | NA | NA | 90% | ~10m |
| Note 1: Service can be potentially provided by both multiple UEs and RAN entities. Note 2: 64bits corresponds to 20 digits in decimal number. 20 digits is assumed for the length of tag ID. Additional 192bits were assumed for control and other data (e.g., location information, IP address of server). Note 3: 100m2 space can be packed with about 250 large size baggage (with H=0.7m, W=0.5m). Assuming 3 layers of baggage stacked gives = 250x3 = 750 |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *