3GPP TSG-SA1 Meeting #103	S1-232037

# Gothenburg, Sweden, 21 - 25 August 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.865 | CR | 0002 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update of consolidation for TR 22.865 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Novamint, Nokia, Nokia Shanghai Bell, ETRI, Sateliot, Gatehouse, CATT, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GSAT_Ph3 |  |  |  |  |  | Date: |  |  | 2023-8-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Several potential requirements agreed at the previous meeting have not been consolidated yet in “Consolidated Requirements” of TR 22.865 version 19.0.0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding CPRs based on PRs agreed at the previous meeting and which could not have been consolidated subsequently (based on S1-231703). Resolving comment in CPR 6.2-7 conducting to the merge of CPR 6.2-6 and CPR 6.2-2 Some additional modifications based on inputs received: using the term “UE-Satellite-UE communication”, using S&F Satellite operation… |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below.

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.2-1 | Subject to operator’s policies, a 5G system with satellite access shall be able to support S&F Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data once the feeder link between the satellite and the ground segment becomes available. | [PR 5.1.6-001] [PR.5.3.6-001] [PR 5.4.6-001] |  |
| CPR 6.2-2 | A 5G system with satellite access shall be able to inform an authorised UE whether S&F Satellite operation is applied and to provide information associated (e.g. estimated delivery time to the authorised 3rd party). | [PR 5.1.6-002] [PR.5.16.6-001] |  |
| CPR 6.2-3 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period. | [PR 5.1.6-003] [PR 5.2.6-002] |  |
| CPR 6.2-4 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota. | [PR 5.1.6-004] [PR 5.2.6-003] [PR.5.3.6-003] [PR 5.4.6-004] |  |
| CPR 6.2-5 | A 5G system with satellite access supporting S&F Satellite operation shall be able to support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to store and forward operation (e.g. forwarding priority, acknowledgment policy). | [PR 5.1.6-005] [PR 5.2.6-004] |  |
| CPR 6.2-6 | A 5G system with satellite access supporting Store & Forward Satellite operation shall be able to provide information associated (e.g. estimated delivery time) to an authorized UE. | [PR.5.16.6-001] |  |
| CPR 6.2-6 | A 5G system with satellite access shall be able to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide information associated (e.g. estimated delivery time to the authorised UE). | [PR 5.2.6-001] [PR 5.2.6-005] | Missing similar mention of the estimated delivery time for MO use case (in CPR 6.2-6) |
| CPR 6.2-7 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs. NOTE: It is assumed that the constellation knows which satellite has a feeder link available. However, this is outside the scope of 3GPP. | [PR.5.1.6-008] [PR.5.3.6-002] |  |
| CPR 6.2-8 | Subject to operator’s policies, a 5G system with satellite access supporting the S&F Satellite operation shall be able to support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available. | [PR 5.1.6-007] [PR 5.2.6-007] |  |

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication are listed in the table below.

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not. | [PR 5.6.6-003] [PR 5.7.6-001] [PR 5.14.6-001] [PR 5.6.6-005] |  |
| CPR 6.3-2 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites). | [PR 5.6.6-004] [PR 5.14.6-002] |  |
| CPR 6.3-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the communication path between UEs extends to additional satellites (through ISLs). | [PR 5.14.6-003] |  |
| CPR 6.3-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication | [PR 5.8.6-001] |  |
| CPR 6.3-5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast). | [PR 5.8.6-002] |  |
| CPR 6.3-6 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when one UE communication path moves between a direct network connection via 5G terrestrial access network and an indirect network connection via a relay UE (using satellite access). NOTE: It is assumed that the 5G terrestrial access network and the satellite access network belong to the same operator. | [PR 5.10.6-002] |  |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below.

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE independently of the UE’s GNSS capability. | [PR 5.11.6-001] [PR 5.11.6-002] |  |
| CPR 6.4-2 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies) in order to provide services. | [PR 5.11.6-003] |  |
| CPR 6.4-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. NOTE: UE may or may not have GNSS capabilities. | [PR 5.13.6-001] [PR 5.12.6-001] |  |
| CPR 6.4-4 | A 5G system with satellite access shall be able to provide to a UE using only satellite access with information on positioning services (e.g. supported positioning performance). NOTE: UE may or may not have GNSS capabilities. | [PR 5.12.6-002] |  |
| CPR 6.4-5 | A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE and network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS). NOTE: UE may or may not have GNSS capabilities. | [PR 5.13.6-002] |  |
| CPR 6.4-6 | Subject to regulatory requirements, a 5G system with satellite access shall be able to provide positioning services (e.g. with the availability of 99%, the accuracy of several kilometres) independently of UE’s GNSS capability when the UE is using only satellite access. NOTE: The regulatory requirements for positioning (e.g. service requirements of Public Safety by GSMA [22]) could be taken into account. | [PR 5.13.6-003] |  |

## 6.5	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below.

Table 6.5-1 – Consolidated Requirements for other aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.5-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. | [PR 5.5.6-001] [PR 5.5.6-002] [PR 5.15.6-001] |  |
| CPR 6.5-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite. | [PR 5.9.6-001] |  |

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-1 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to preserve security of the data stored and forwarded. | [PR.5.3.6-002] [PR.5.1.6-008] |  |
| CPR 6.6-2 | A 5G system with satellite access supporting S&F Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F Satellite operation. | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] [PR 5.1.6-006] [PR 5.2.6-006] |  |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the UE-Satellite-UE communication, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a relay UE (using satellite access assuming that the 5G system with satellite access is authorized to assign spectrum resources for the communication between remote UE and relay UE). | [PR 5.6.6-001] [PR 5.10.6-001] |  |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-1 | A 5G system with satellite access supporting S&F Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites). | [PR 5.4.6-005] [PR 5.4.6-006] |  |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, for UE-Satellite-UE communication. | [PR 5.6.6-002] [PR 5.7.6-002] |  |

End Change