3GPP TSG SA WG 1 Meeting #103 	S1-232038

Gothenburg, Sweden, 21 - 25 August 2023

Source:	Novamint, ETRI, Nokia, Nokia Shanghai Bell, Sateliot, Gatehouse, China Telecom, CATT

pCR Title:	Discussion on normative work of 5GSat Ph3

Draft Spec:	3GPP TR 22.865 1.1.0

Agenda item:	7.7

Document for:	Discussion

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document proposes to discuss in which section(s) of 22.261 the consolidated potential requirements of TR 22.865 can be included.

1. Introduction

TR 22.865 version 19.0.0 includes several consolidated requirements in a dedicated section and split in different topics. The purpose of this document is to identify where each of the CPR can be included in 22.261

2. Reason for Change

Identify where to add normative requirements related to 5GSAT_Ph3 in 22.261 based on the consolidation made in TR 22.865 v19.0.0 and update coming out of this meeting.

The assumption is that there will be a dedicated Satellite access section

3. Proposal

It is proposed to implement the different CRs for normative requirements related to 5GSAT_Ph3 in the relevant sections of 22.261 based on the following identification

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the 5G system with satellite access.

## 6.2	Store & Forward Satellite operation

The potential requirements corresponding to the support of Store & Forward Satellite information are listed in the table below.

Table 6.2-1 – Consolidated Requirements for Store & Forward Satellite operation

| CPR # | Consolidated Potential Requirement | Original PR # | Clause of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.2-1 | Subject to operator’s policies, a 5G system with satellite access shall be able to support S&F Satellite operation for authorized UEs e.g. store data on the satellite when the feeder link is unavailable; and forward the data once the feeder link between the satellite and the ground segment becomes available. | [PR 5.1.6-001] [PR.5.3.6-001] [PR 5.4.6-001] | New clause Either 6.x Satellite Access 6.x.8 Store & Forward Satellite operation or 6.45 Store & Forward Satellite operation 6.45.2 Requirements |
| CPR 6.2-2 | A 5G system with satellite access shall be able to inform an authorised UE whether S&F Satellite operation is applied and to provide information associated (e.g. estimated delivery time to the authorised 3rd party). | [PR 5.1.6-002] [PR.5.16.6-001] | Same as above |
| CPR 6.2-3 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period. | [PR 5.1.6-003] [PR 5.2.6-002] | Same as above |
| CPR 6.2-4 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota. | [PR 5.1.6-004] [PR 5.2.6-003] [PR.5.3.6-003] [PR 5.4.6-004] | Same as above |
| CPR 6.2-5 | A 5G system with satellite access supporting S&F Satellite operation shall be able to support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to store and forward operation (e.g. forwarding priority, acknowledgment policy). | [PR 5.1.6-005] [PR 5.2.6-004] | Same as above |
| CPR 6.2-6 | A 5G system with satellite access shall be able to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide information associated (e.g. estimated delivery time to the authorised UE). | [PR 5.2.6-001] [PR 5.2.6-005] | Same as above |
| CPR 6.2-7 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through Inter-Satellite Links. NOTE: It is assumed that the constellation knows which satellite has a feeder link available. However, this is outside the scope of 3GPP. | [PR.5.1.6-008] [PR.5.3.6-002] | Same as above |
| CPR 6.2-8 | Subject to operator’s policies, a 5G system with satellite access supporting the S&F Satellite operation shall be able to support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available. | [PR 5.1.6-007] [PR 5.2.6-007] | Same as above |

Result: In addition, it is proposed to add a small description in this new section and to include Annex A (Informative) of TR 22.865 as informative Annex in 22.261

## 6.3	UE-Satellite-UE communication

The potential requirements corresponding to the support of UE-Satellite-UE communication are listed in the table below.

Table 6.3-1 – Consolidated Requirements for UE-Satellite-UE communication

| CPR # | Consolidated Potential Requirement | Original PR # | Clause of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not. | [PR 5.6.6-003] [PR 5.7.6-001] [PR 5.14.6-001] [PR 5.6.6-005] | Dedicated New clause 6.x Satellite Access 6.x.9 UE-Satellite-UE communication Or 6.3 Multiple access technologies 6.3.2.3 Satellite access or 6.4 Resource efficiency 6.4.2.1 General? |
| CPR 6.3-2 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the UE communication path moves between serving satellites (due to the movement of the UE and/or the satellites). | [PR 5.6.6-004] [PR 5.14.6-002] | Dedicated New clause 6.x Satellite Access 6.x.3 Service continuity or 6.2.3 Service continuity requirements |
| CPR 6.3-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when the communication path between UEs extends to additional satellites (through ISL). | [PR 5.14.6-003] | Dedicated New clause 6.x Satellite Access 6.x.3 Service continuity or 6.2.3 Service continuity requirements |
| CPR 6.3-4 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide QoS control of a UE-Satellite-UE communication | [PR 5.8.6-001] | Dedicated New clause 6.x Satellite Access 6.x.9 UE-Satellite-UE communication Or 6.7 Priority, QoS, and policy control 6.7.2 Requirements or 6.5 Efficient user plane 6.5.2 Requirements |
| CPR 6.3-5 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast). | [PR 5.8.6-002] | Dedicated New clause 6.x Satellite Access 6.x.9 UE-Satellite-UE communication Or 6.3 Multiple access technologies 6.3.2.3 Satellite access or 6.13 Flexible broadcast/multicast service 6.13.2 Requirements |
| CPR 6.3-6 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support service continuity (with minimum service interruption) of a UE-Satellite-UE communication when one UE communication path moves between a direct network connection via 5G terrestrial access network and an indirect network connection via a relay UE (using satellite access). NOTE: It is assumed that the 5G terrestrial access network and the satellite access network belong to the same operator. | [PR 5.10.6-002] | Dedicated New clause 6.x Satellite Access 6.x.3 Service continuity or 6.2.3 Service continuity requirements or 6.9 Connectivity models 6.9.2.5 Satellite and Relay UEs |

## 6.4	GNSS independent operation & positioning enhancements for satellite access

The potential requirements corresponding to the support of GNSS independent operation & positioning enhancements for satellite access are listed in the table below.

Table 6.4-1 – Consolidated Requirements for GNSS independent operation & positioning enhancements for satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Section of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to provide services to an authorized UE independently of the UE’s GNSS capability. | [PR 5.11.6-001] [PR 5.11.6-002] | Dedicated New clause 6.x Satellite Access 6.x.2 General or 6.3 Multiple access technologies 6.3.2.3 Satellite access |
| CPR 6.4-2 | Subject to the regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to determine the location of a UE using only satellite access (e.g. based on 3GPP positioning technologies) in order to provide services. | [PR 5.11.6-003] | Same as above |
| CPR 6.4-3 | Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall be able to support 3GPP positioning methods for UEs using only satellite access. NOTE: UE may or may not have GNSS capabilities. | [PR 5.13.6-001] [PR 5.12.6-001] | Dedicated New clause 6.x Satellite Access 6.x.10 Positioning aspects for satellite access or 6.27 Positioning services 6.27.2 Requirements |
| CPR 6.4-4 | A 5G system with satellite access shall be able to provide to a UE using only satellite access with information on positioning services (e.g. supported positioning performance). NOTE: UE may or may not have GNSS capabilities. | [PR 5.12.6-002] | Same as above |
| CPR 6.4-5 | A 5G system with satellite access shall be able to support negotiation of positioning methods, between UE and network, according e.g. to 3GPP RAT and UE positioning capability, the availability of non-3GPP positioning technologies (e.g. GNSS). NOTE: UE may or may not have GNSS capabilities. | [PR 5.13.6-002] | Same as above |
| CPR 6.4-6 | Subject to regulatory requirements, a 5G system with satellite access shall be able to provide positioning services (e.g. with the availability of 99%, the accuracy of several kilometres) independently of UE’s GNSS capability when the UE is using only satellite access. NOTE: The regulatory requirements for positioning (e.g. service requirements of Public Safety by GSMA [22]) could be taken into account. | [PR 5.13.6-003] | Same as above |

## 6.5	Other aspects for satellite access

The potential requirements corresponding to the support of enhancements of other aspects of satellite access are listed in the table below.

Table 6.5-1 – Consolidated Requirements for other aspects of satellite access

| CPR # | Consolidated Potential Requirement | Original PR # | Clause of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.5-1 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support an efficient communication path and resource utilization for a UE using only satellites access, e.g. to minimize the latencies introduced by satellite links involved. | [PR 5.5.6-001] [PR 5.5.6-002] [PR 5.15.6-001] | Dedicated New clause 6.x Satellite Access 6.x.6 Efficient user plane or 6.5 Efficient user plane or 6.4 Resource efficiency 6.4.2.1 General |
| CPR 6.5-2 | Subject to regulatory requirements and operator’s policies, a 5G system with satellite access shall be able to support collection of information on usage statistics and location of the UEs that are connected to the satellite. | [PR 5.9.6-001] | Dedicated New clause 6.x Satellite Access 6.x.2 General or 6.3 Multiple access technologies 6.3.2.3 Satellite access |

## 6.6	Security aspects

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for security aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Clause of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.6-1 | Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to preserve security of the data stored and forwarded. | [PR.5.3.6-002] [PR.5.1.6-008] | 8 Security 8.9 Data security and privacy |
| CPR 6.6-2 | A 5G system with satellite access supporting S&F Satellite operation shall be able to support mechanisms to authorize a UE to use the S&F Satellite operation. | [PR.5.3.6-004] [PR 5.4.6-003] [PR 5.4.6-002] [PR 5.1.6-006] [PR 5.2.6-006] | 8 Security 8.4 Authorization |
| CPR 6.6-3 | A 5G system with satellite access shall be able to support mechanisms to authorize the UE-Satellite-UE communication, based on e.g., location information and subscription. NOTE: UEs can use satellite access directly or via a relay UE (using satellite access assuming that the 5G system with satellite access is authorized to assign spectrum resources for the communication between remote UE and relay UE). | [PR 5.6.6-001] [PR 5.10.6-001] | 8 Security 8.4 Authorization |

## 6.7	Charging aspects

The potential requirements corresponding to the charging aspects are listed in the table below.

Table 6.7-1 – Consolidated Requirements for charging aspects

| CPR # | Consolidated Potential Requirement | Original PR # | Clause of TS 22.261 identified |
| --- | --- | --- | --- |
| CPR 6.7-1 | A 5G system with satellite access supporting S&F Satellite operation shall be able to collect charging information per UE or per application (e.g., number of UEs, data volume, duration, involved satellites). | [PR 5.4.6-005] [PR 5.4.6-006] | 9 Charging aspects 9.1 General 9.4 Satellite Access |
| CPR 6.7-2 | A 5G system with satellite access shall be able to collect charging information for a UE registered to a HPLMN or a VPLMN, for UE-Satellite-UE communication. | [PR 5.6.6-002] [PR 5.7.6-002] | 9 Charging aspects 9.1 General 9.4 Satellite Access |