3GPP TSG SA WG1 Meeting #103 	S1-232055

Goteborg, Sweden, 21 - 25 August 2023	(revision of S1-23xxxx)

Source:	Xiaomi

pCR Title:	Pseudo-CR on 5.2 Requirements – 5.2.1 General subsection

Draft Spec:	3GPP TS 22.137

Agenda item:	x.x

Document for:	Approval

Contact:	Chendong7@xiaomi.com

Abstract: This document introduces 5.2.1 General subsection of 5.2 Requirements in TS 22.137

1. Introduction

This pCR introduced 5.2.1 General subsection of 5.2 Requirements into the new TS on Sensing.

2. Reason for Change

Based on the discussion in the study report TR 22.837, some requirements on general have been introduced. Here we capture these requirements in the 5.2.1 General subsection of 5.2 Requirements into TS 22.137.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137.

* * * First Change * * * *

### 5.2.1	General

The 5G system shall be able to provide 5G wireless sensing service in a sensing service area location using sensing transmitters and sensing receivers.

Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g. network load).

Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network for processing.

The 5G system shall support continuity for 5G wireless sensing service (e.g. for sensing a moving object).

Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the combination of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services (e.g. prioritizing between communication and sensing services).

The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service.

Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum.

* * * End of Changes * * * *