3GPP TSG SA WG 1 Meeting #103 	S1-232097

Gothenburg, Sweden, 21 - 25 August 2023

Source:	Novamint

pCR Title:	Pseudo-CR on consolidated requirements for TR 22.848

Draft Spec:	3GPP TR 22.848 version 0.1

Agenda item:	7.12

Document for:	Approval

Contact:	Thierry Bérisot (tberisot@novamint.com)

Abstract: This document includes a text proposal for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

1. Introduction

The “Consolidated Requirements” of TR 22.848 are currently not included in version 0.1.0

2. Reason for Change

adding texts and tables for section Consolidated Requirements of TR 22.848, capturing a set of final consolidated potential requirements

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.848 v 0.1.0.

===============================  All new Text ================================

# 6	Consolidated Requirements

## 6.1	Introduction

The following requirements represent a consolidation of the various potential requirements captured in the above use cases related to the interconnect of SNPN.

## 6.2	Scalable SNPN Interconnect with dynamic connections

The potential requirements corresponding to the support of Scalable SNPN Interconnect with dynamic connections are listed in the table below

Table 6.2-1 – Consolidated Requirements for Scalable SNPN Interconnect with dynamic connections

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |

## 6.3	Interconnect between SNPNs

The potential requirements corresponding to the support of interconnect between SNPNs are listed in the table below

Table 6.3-1 – Consolidated Requirements for interconnect between SNPNs

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.3-1 | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms to enable interconnect between standalone non-public networks. | [PR 5.2.6-001] |  |
| CPR 6.3-2 | The 5G system shall support a mechanism for an authorised UE from a SNPN to interconnect with another SNPN for which the UE is authorized to access to. NOTE: All types of services (e.g., voice, VoIP, IoT, mission critical communications, IMS, Video Streaming) shall be supported by the interconnect between SNPNs | [PR 5.5.6-002] |  |
| CPR 6.3-3 | The 5G system shall support a mechanism for an authorised UE from a SNPN to interconnect with another SNPN only for a defined duration. | [PR 5.7.6-004] | To merge with above |
| CPR 6.3-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support interconnect of standalone non-public networks through links established on a temporary basis forming a group of interconnected Standalone Non-Public Networks with a variable topology. | [PR 5.3.6-001] |  |
| CPR 6.3-5 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support routing of user data through Standalone Non-Public Networks interconnected. | [PR 5.3.6-002] |  |
| CPR 6.3-6 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of Standalone Non-Public Networks in a group of interconnected Standalone Non-Public Networks. | [PR 5.3.6-003] |  |
| CPR 6.3-7 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support per UE communication, the establishment, reconfiguration and release of user data transmission path between two Standalone Non-Public Networks in a group of interconnected Standalone Non-Public Networks. | [PR 5.3.6-004] |  |
| CPR 6.3-8 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a user subscribed to one Standalone Non-Public Network to consume a service offered by another Standalone Non-Public Network in a group of interconnected Standalone Non-Public Networks. | [PR 5.3.6-005] |  |
| CPR 6.3-9 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support a mechanism to exploit interconnection links metrics (hop-count, jitter, latency, packet loss, capacity ...) as a criterion to route data between two distant Standalone Non-Public Networks in a group of interconnected Standalone Non-Public Networks. | [PR 5.3.6-006] |  |
| CPR 6.3-10 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of Standalone Non-Public Networks in a group of interconnected Standalone Non-Public Networks without interrupting ongoing communications. | [PR 5.4.6-001] |  |
| CPR 6.3-11 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support seamless handover of UE communications between interconnected Standalone Non-Public Networks with overlapping radio coverage. | [PR 5.4.6-002] |  |
| CPR 6.3-12 | The 5G system shall support for a SNPN to restrict the interconnect based on the type of communication the UE want to use the SNPN for | [PR 5.7.6-003] |  |
| CPR 6.3-13 | The 5G system shall support a secure mechanism for a SNPN to remotely provision credentials of a uniquely identifiable and verifiably UE | [PR 5.7.6-005] |  |
| CPR 6.3-14 | The 5G system shall support a mechanism for an authorised UE to adapt the subscription to interconnect of SNPN to the capabilities of the UE. | [PR 5.7.6-006] |  |
| CPR 6.3-15 | The 5G system shall support an automatic and secure on-boarding of a user trying to interconnect with another SNPN than the ones he is registered to and authorised for. | [PR 5.8.6-001] |  |
| CPR 6.3-16 | The 5G system shall support for a SNPN to allow or to restrict the interconnect of a UE based on the type of SNPN the UE is primary registered to (e.g., from public safety) or based on the UE capability. | [PR 5.8.6-002] |  |
| CPR 6.3-17 | The 5G system shall support for a SNPN to continue to allow or to stop the interconnect of a UE coming from another SNPN when this SNPN is not available anymore. | [PR 5.8.6-003] |  |
| CPR 6.3-18 | Subject to regulatory requirements and SNPN operators’ policy, the 5G system shall support a mechanism for 2 SNPNs from different countries to interconnect between each other in the same area | [PR 5.8.6-004] |  |
| CPR 6.3-19 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support discovery, addition and authentication, or detachment of Standalone Non-Public Networks in a group of interconnected Standalone Non-Public Networks without interrupting ongoing communications. | [PR 5.4.6-001] |  |

## 6.4	Network Selection

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.4-1 – Consolidated Requirements for network selection

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.4-1 | Subject to SNPNs operators’ agreement, the 5G system shall be able to support mechanisms for a selection of 5GS endpoints for SNPNs’ interconnect. | [PR 5.2.6-002] |  |
| CPR 6.4-2 | The 5G system shall support for an authorised UE to be able to select and switch between SNPNs based on the availability and strength of the signal, without any disruption to the ongoing private network services. | [PR 5.5.6-003] |  |
| CPR 6.4-3 | The 5G system shall support for an authorised UE to be able to select the optimal network based on factors such as coverage, capacity, relation with the network and quality of service. | [PR 5.6.6-001] |  |
| CPR 6.4-4 | The 5G system shall support that the network selection process during interconnect between SNPNs to be automatic and transparent to the user, without requiring manual intervention. | [PR 5.6.6-002] |  |
| CPR 6.4-5 | The 5G system shall support for a SNPN to inform authorised UE about the type of communications supported by the SNPN. | [PR 5.7.6-001] |  |
| CPR 6.4-6 | The 5G system shall support for an authorised UE to perform an automatic discovery to determine the capabilities of the SNPNs available and the type of applications supported by the SNPNs. | [PR 5.7.6-002] |  |

## 6.5	Service continuity

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.5-1 – Consolidated Requirements for service continuity

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.5-1 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to support service continuity between interconnected Standalone Non-Public Networks with overlapping radio coverage. | [PR 5.4.6-002] |  |
| CPR 6.5-2 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall enable packet loss to be minimized during mobility between interconnected Standalone Non-Public Networks with overlapping radio coverage for some or all connections associated with a UE. | [PR 5.4.6-003] |  |
| CPR 6.5-3 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall minimize interruption time during mobility between interconnected Standalone Non-Public Networks with overlapping radio coverage for some or all connections associated with a UE. | [PR 5.4.6-004] |  |
| CPR 6.5-4 | Subject to regulatory requirements and SNPN operators’ policy, a 5G system shall support service continuity (with minimum service interruption) of a private network communication during interconnect between 2 SNPNs. NOTE: The SNPNs should support the transfer of UE session information, including IP address and other relevant parameters, from the first SNPN to the second SNPN, to maintain continuity of service during interconnect. | [PR 5.5.6-004] |  |

## 6.6	QoS & performances

The potential requirements corresponding to the QoS & performances aspect are listed in the table below.

Table 6.6-1 – Consolidated Requirements for QoS a,d performances aspect

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.6-1 | The 5G system shall support mechanisms for SNPNs to be able to ensure a consistent level of service quality, including low latency, high throughput, and minimal packet loss, to support the private network services and applications during interconnect between SNPNs. | [PR 5.5.6-005] |  |
| CPR 6.6-2 | The 5G system shall support mechanisms for a SNPN to be able to monitor and manage the private network services, including network performance, traffic volume, and resource utilization for an authorized UE having interconnected from another SNPN. | [PR 5.5.6-006] |  |
| CPR 6.6-3 | The 5G system shall support mechanisms for the two SNPNs to be able to ensure that the UE receives a consistent and reliable quality of service, regardless of the SNPN being used. NOTE: This may require the use of network slicing and QoS mechanisms to prioritize and optimize the traffic flows. | [PR 5.6.6-005] |  |
| CPR 6.6-4 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to provide the required QoS (e.g. reliability, end-to-end latency, and bandwidth) and support prioritization of resources for a service offered by a Standalone Non-Public Network that is consumed by users attached to other interconnected Standalone Non-Public Networks. | [PR 5.x.6-001] |  |
| CPR 6.6-5 | Based on SNPN configuration and subject to SNPN operator’s policy, the 5G system shall be able to monitor the delivered QoS of a service consumed by one or several users attached to different Standalone Non-Public Networks inside a group of interconnected Standalone Non-Public Networks. | [PR 5.x.6-002] |  |

## 6.7	Security aspect

The potential requirements corresponding to the security aspect are listed in the table below.

Table 6.7-1 – Consolidated Requirements for security aspect

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.7-1 | The 5G system shall support for a SNPN to be able to provide a secure authentication mechanism to ensure that only authorized UEs can access the private network services. NOTE: The authentication should be based on mutual authentication and should include encryption of user data to prevent eavesdropping and tampering. | [PR 5.5.6-001] |  |

## 6.8	Charging aspect

The potential requirements corresponding to the charging aspect are listed in the table below.

Table 6.8-1 – Consolidated Requirements for charging aspect

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR 6.8-1 | The 5G system shall be able to collect charging information per UE or per application when interconnecting with another SNPN (e.g., number of UEs, data volume, which SNPN the UE is primary subscriber to, which services, duration of the use of the visited network). | [PR 5.6.6-003] |  |
| CPR 6.8-2 | The 5G system shall support a mechanism to ensure accurate and timely charging of the services used by the UE when interconnecting with another SNPN. | [PR 5.6.6-004] |  |