3GPP TSG-SA WG1 Meeting # 103 	S1-232119

# Gothenborg, Sweden, Aug 21, 2023 - Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.856 | CR | 0005 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on the consolidate requirements for coordination and synchronization of data flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Metaverse |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA1 102 meeting, PR 5.12.6-1 are not consolidated : [PR 5.12.6-1] The 5G system shall provide a mechanism to support coordination and synchronization of multiple data flows transmitted via one UE or different UEs, subject to synchronization thresholds provided by third party (i.e. a quantitative bound to the extent different data flows can diverge in synchronization.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the PR in the consolidated requirements, clause 7.1.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreed PR not be consolidated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of CHANGE

7.1.3	Operational efficiency, exposure, and coordination of mobile metaverse services

Table 7.1.3-1 – Operational efficiency, exposure, and coordination of mobile metaverse services Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| [CPR 3.1] | Subject to operator policy, the 5G system shall support a mechanism that enables flexible adjustment of communication services based on e.g. the type of devices (e.g., wearables), or communication duration (e.g. more than one hour), such that the services can be operated with reduced energy utilization. NOTE: Metaverse service experience over an extended period of time (e.g. 2h) requires significant power consumption by the UE. In some cases, a device with no external power supply cannot sustain downloading and rendering of media over a long interval, e.g. for the duration of an entire feature film or athletic event. | [PR 5.7.6-1] [PR 5.7.6-2] |  |
| [CPR 3.2] | The 5G system shall be able to provide a means to associate and coordinate data flows related to one or multiple UEs e.g. associated with the same object in digital twin applications provided by the mobile metaverse service. | [PR 5.20.6-1] [PR 5.20.6-2] [PR 5.20.6-3] |  |
| [CPR 3.3] | Subject to operator policy, regulatory requirements and user consent, the 5G system shall be able to expose network performance information (e.g., observed or predicted bitrate, latency or packet loss) related to one or more users to an authorized third party metaverse application. NOTE: The network performance information can be per UE and take into account all available access network types, i.e. 3GPP and non-3GPP. | [PR 5.25.6-1] [PR 5.9.6.2] |  |
| [CPR 3.4] | Subject to operator policy, the 5G system shall support a mechanism to enable one or more authorized third party(ies) to coordinate multiple service data flows delivered to/from one or more UE(s). Multiple UEs may be associated with one user/location or different users at different locations potentially using different access networks, i.e. 3GPP and non-3GPP. NOTE 1: Coordination refers to the ability to provide an acceptable level of user experience for a given service, e.g. based on latency and synchronization constraints (due to multiple sources or long distance between UEs/users). NOTE 2: It is not assumed that it is always possible to coordinate and provide the same capabilities regardless of whether 3GPP or non-3GPP access is used. | [PR 5.27.6-3] [PR 5.9.6.1] [PR 5.3.6.2-4] [PR 5.25.6-2] [PR 5.10.6-2] |  |
| [CPR 3.5] | The 5G system shall enable the coordination of diverse media, transmitted to a UE from one or more mobile metaverse services associated with a physical location, to be combined to form a localized service experience. | [PR 5.1.6-4] [PR 5.4.6-1] |  |
| [CPR 3.6] | Subject to operator policy, the 5G system shall support exposure mechanisms enabling an authorized third party to determine one or more subscribers to whom mobile metaverse media can be distributed in a resource efficient manner. | [PR 5.27.6-1] |  |
| [CPR 3.7] | Subject to operator policy, subject to user consent, the 5G system shall support a means to provide resource efficient communication of third party mobile metaverse media to one or more subscribers. | [PR 5.27.6-2] |  |
| [CPR 3.X] | The 5G system shall provide a mechanism to support coordination and synchronization of multiple data flows transmitted via one UE or different UEs, subject to synchronization thresholds provided by third party (i.e. a quantitative bound to the extent different data flows can diverge in synchronization.) | [PR 5.12.6-1] |  |

End of CHANGE