3GPP TSG-SA WG1 Meeting #103 	S1-232136

Goteborg, Sweden, 21 – 25 August 2023

Source:	OPPO, Apple

pCR Title:	Overview for Ambient IoT TS

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	xuweijie@oppo.com, Yang Xu (xuyang@oppo.com)

Abstract: This document provides the scope section of TS 22.xxx.

****************Start of Change *************

4	Overview

In the 5G era, various LPWA technologies such as MTC, NB-IoT, and RedCap have been developed to fulfil the increasing demand from verticals. These LPWA technologies have achieved low cost, low power and massive connections and can meet requirements of many applications. However, there are still many use cases and applications that cannot be addressed. For example, a conventional battery-powered device cannot be deployed in extreme environmental conditions (e.g. high pressure, extremely high/low temperature, humid environment). Also, conventional battery devices cannot be used where maintenance-free devices are required (e.g. where the devices are inaccessible and it is not possible to replace the device battery). Finally, ultra-low complexity, very small device size/form factor (e.g. thickness of mm), longer life cycle, etc. are required for mass market use cases.

Ambient power-enabled IoT is a promising technology to fulfil the unmet market requirements stated above. An ambient power-enabled IoT device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor) and the energy is provided through the harvesting of radio waves, light, motion, heat, or any other suitable power source. Ambient IoT devices are expected to have lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices (e.g. NB-IoT/eMTC devices) with an emphasis on improving network efficiency. Ambient IoT devices can be maintenance free and can have long life span (e.g. more than 10 years).

This technical specification describes the ambient IoT technology service requirements as part of the 5G system to enable new services and use cases. Ambient IoT has the potential to benefit a large number of verticals, e.g. smart manufacturing, logistics and warehousing, smart grid, agriculture, and smart home by providing functionalities that fulfil the needs of use cases such as inventory taking, sensor data collection, asset tracking and actuator control. Therefore, a new kind of IoT service for the verticals will be enabled by combining ambient power-enabled IoT with cellular networks, vastly benefitting the 3GPP ecosystem.

*******************End of Change*****************