3GPP TSG-SA WG1 Meeting #103 	S1-232138

Goteborg, Sweden, 21 – 25 August 2023

Source:	OPPO

pCR Title:	Input to new TS - Ambient IoT performance requirements

Draft Spec:	3GPP TS 22.XXX

Agenda item:	7.2.2

Document for:	Approval

Contact:	xuweijie@oppo.com, Yang Xu (xuyang@oppo.com)

1. Introduction

This pCR introduces descriptions and requirements of ambient IoT performance in section 6.

2. Reason for Change

Overall descriptions and performance requirements need to be described.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 22.XXX.

****************Start of Change *************

6	Performance requirements

6.1	General

Ambient IoT service can be applied for 4 categorization of use cases, namely inventory, sensor data collection, tracking and actuator control. The corresponding performance is required for each of the use case categorization.

6.2	Ambient power-enabled IoT performance requirements

The 5G system shall be able to provide ambient IoT services with the performance requirements in the following Table 6.2.1-1, Table 6.2.2-1, Table 6.2.3-1, Table 6.2.4-1.

6.2.1 KPIs for inventory

Table 6.2.1-1 KPIs for inventory

|  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | inventory or asset management | Typically, seconds level | 99% | NA | <2 Kbps | 96/256 bits | <1,5 Million/km2 | 30~50m indoor, 200m~400m outdoor | 1-10km2 | 3~10km/h | NA | NA | NA | 3 m indoor, cell-level outdoor |  |

6.2.2 KPI for sensor data collection

Table 6.2.2-1 KPIs for sensor data collection

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Room envirionment monitoring (e.g. domicile, machinery) | 1-20 s | 99 % | NA | <1 kbit/s | < 100 bits | NA | [10-30]m | NA | Stationary | NA | NA | NA | NA |  |
|  |  | Indoor agriculture and husbandry | >10 s | 99.9% | NA | <1 kbit/s | Typically, [< 100 bytes] | 1 per m2 | 30 - 200 m | 6000 m2~30,000m2 | Quasi-stationary | 15 minutes to half an hour | NA | NA | NA |  |
| outdoor |  | Smart grid | 1 s | 99% | NA | < 1kbit/s | Typically, < 100 bytes | < 10,000 /km2 | Typically 50-200 meters | [several km2 up to 100 000 km2] | Stationary | 5-15 min | NA | NA | several 10 m |  |
|  |  | Outdoor husbandry and logistics | Typically, > tens of seconds | 99% | NA | <500 bit/s | Typically, [< 100 bytes] | <5200 devices / km2 | [300 m - 500 m] | 430000 m2 | NA | 15 min | NA | NA | NA |  |
|  |  | Smart city | 10 s - 30 s | 99% | NA | <1 kbit/s | Typically, [< 100 bytes] | <1000 devices / km2 | [300 m - 700 m] | City wide including rural areas | Stationary | 15 min | NA | NA | NA |  |

6.2.3 KPI for tracking

Table 6.2.3-1 KPIs for tracking

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Indoor tracking | 1s | 99.9% | NA | <1 kbit/s | <1k bits | 25/100 m2 | 10m indoor | 200 m2 | Static | 1 per hour | 1s | 95% | 1-3 m, 90% availability |  |
| Outdoor |  | Outdoor tracking for medium coverage | 1s | 99.9% | NA | <1 kbit/s | <1 kbits | <10 per 100 m2 | 100 m | Up to the whole PLMN | Static | 1 per hour | 1 s | 99% | several 10m |  |
|  |  | Outdoor tracking for large coverage | NA | NA | NA | NA | NA | NA | 500m | NA | Outdoor - up to [10 km/h] | NA | [10 s] | [95 %] | Cell-level Horizontal Accuracy |  |

6.2.4 KPI for actuator control

Table 6.2.4-1 KPIs for actuator control

| Deployment |  | Scenarios | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor |  | Indoor actuator control | Several seconds | 99% | NA | 1 kbit/s | <100 Bytes | less than 1.5 million/km2 | 50m indoors | <250 m2 for home, and 15,800 square meters for supermarket | stationary | 20 minutes to 2 hours | NA | NA | 3 m to 5 m indoor |  |
| Outdoor |  | Outdoor actuator control for large coverage | 1 s | 99% | N/A | NA | 128bit (DL) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA |  |
|  |  | Outdoor actuat control rs for medium coverage | Several seconds | 99% | NA | <2 kbit/s | <200 bits | <200 per 100 m2 | 200 m | City wide including rural areas | Static | NA | NA | NA | 3 m to 5 m indoor |  |

*******************End of Change*****************