3GPP TSG SA WG 1 Meeting #103 	S1-232159

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-23xxxx)

Source:	Xiaomi

pCR Title:	Pseudo-CR on Remove brackets in KPI table of use case 5.9 and 5.21

Draft Spec:	3GPP TR 22.840 V1.2.0

Agenda item:	7.2.1

Document for:	Approval

Contact:	Ruby Gong <gongyubing@xiaomi.com>

Abstract: SA1 has approved use case 5.9 and 5.21 and corresponding KPI table in the previous meeting, this pCR is proposed to remove the brackets in the KPI table of use case 5.9 and 5.21.

1. Introduction

SA1 has approved use case 5.9 and 5.21 and corresponding KPI tables in the previous meeting

2. Reason for Change

To resolve the remaining format misalignment, this pCR is proposed to remove the brackets in the KPI table of use case 5.9 and 5.21.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.2.0.

* * * First Change * * * *

5.9.6	Potential New Requirements needed to support the use case

[PR 5.9.6-001] 5G system shall optimize mobility management support for mobile Ambient-enabled IoT devices that are unable to constantly stay active.

[PR 5.9.6-002] 5G system shall be able to determine the location of Ambient IoT device, when it becomes active as triggered by the 5G network.

[PR 5.9.6-003] The 5G system shall be able to provide location services for Ambient IoT with the performances requirements reported in Table 5.9.6-1.

Table 5.9.6-1: Performance requirements for location service for Ambient  IoT

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Absolute positioning | NA | NA | NA | NA | NA | NA | 500m | NA | Outdoor - up to 10 km/h | NA | 10 s | 95 % | Cell-level horizontal accuracy (NOTE 1) |
| NOTE 1: This KPI table is mostly from the positioning service levels 1 in Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels, TS22.261 V18.7.0. NOTE 2: The Positioning service latency considers the latency of the activation of Ambient IoT device. NOTE 3: The indoor velocity is from the average human walking speed. NOTE 4: The outdoor velocity is from the ETC speed limit. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

5.21.6	Potential New Requirements needed to support the use case

[PR.5.21.6-001] The 5G system shall support to authorize a UE to perform Ambient IoT communication services with specific Ambient IoT devices.

[PR.5.21.6-002] The 5G system shall be able to support to authorize a UE to perform relative positioning operations with specific Ambient IoT devices.

[PR.5.21.6-003] The 5G system shall support to authorize a UE to obtain device identity information of an Ambient IoT device.

[PR.5.21.6-004] The 5G system shall be able to expose the identities and relative positioning results of Ambient IoT devices to a trusted third party.

[PR. 5.21.6-005] The 5G system shall be able to support suitable security mechanisms for Ambient IoT devices, including encryption and data integrity.

[PR.5.21.6-006] The 5G system shall be able to provide Ambient IoT service with the following KPIs:

Table 5.21.6-1: Ambient IoT service KPI for museum guide

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Museum guide (indoor) | 2 s | 99.9% | NA | < 1 kbit/s UL (NOTE 1) | 96 bits | <10,000 /km² | 30 m | 20,000 m² (NOTE 2) | 3 km/h | NA | NA | 90% | 3 m |
| NOTE 1: The payload includes Ambient IoT device information, e.g., Ambient IoT device ID. NOTE 2: For a relatively large-sized museum, the typical size is about 20,000 m². |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *