3GPP TSG-SA WG1 Meeting #103 	S1-232169

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-22xxxx)

Source:	Huawei, HiSilicon, ZTE?

pCR Title:	Pseudo-CR on FS_Ambient IoT, update latency KPI for clause 5.30

Draft Spec:	3GPP TR 22.840 v1.2.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang ZHANG (zhang.shuang2@huawei.com)

Abstract: This document proposes to update max allowed latency KPI to clause 5.30 “Ambient IoT device acting as a controller in smart agriculture” in TR 22.840 v.1.2.0.

1. Introduction

Clause 5.30 was captured in TR 22.840 in the last SA1 meeting, which introduces a type of Ambient IoT communication service that enables command to be sent to Ambient IoT-device based controller to turn on e.g. irrigation. The proposed max latency is “hundreds ms level”.

2. Reason for Change

To turn on irrigation system for a crop is not time-critical or time-sensitive. Different from stringent “periodic deterministic communication” as specified in TS 22.104 for industry control, a device getting a command to turn on a water tag connector is not time-critical or time-sensitive. Neither does the water tap need to be strictly turned on and off in sub-second interval. Therefore, 10s of seconds are sufficient to successfully execute the action of turning on an irrigation system.

3. Conclusions

Update the value of Max. allowed end-to-end latency in KPI to 10 s.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.2.0.

* * * First Change * * * *

### 5.30.6	Potential New Requirements needed to support the use case

[P.R.5.30.6-001] The 5G system shall provide means for a trusted third-party to trigger an ambient IoT device or group of ambient IoT devices to communicate periodically.

[P.R.5.30.6-002] The 5G system shall be able to provide ambient IoT service with following KPIs：

Table 5.x.6-1: KPI Table of Ambient IoT controller in smart agriculture

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ambient IoT controller in smart agriculture | 10 s | 99% | N/A | NA | 128bit (DL) (note1) | NA | [500]m outdoors | NA | Static | NA | NA | NA | NA |
| Note 1: this size refers to the payload size of the control information sent by the 5G network to the ambient IoT controller. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *