3GPP TSG SA WG 1 Meeting #103 	S1-232198

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-23xxxx)

Source:	Huawei, Hisilicon

pCR Title:				Pseudo-CR on update clause 5_14 “Inter-PLMN scenario - TN and multiple NTN”

Draft Spec:	3GPP TR TR22.841 V1.1.0

Agenda item:	7.9

Document for:	Approval

Contact:	Shuang ZHANG <zhang.shuang2@huawei.com>

Abstract: This pCR includes a text proposal to update clause 5.14 “Inter-PLMN scenario - TN and multiple NTN” in TR 22.841 v.1.1.0, based on associated discussion paper (S1-232204).

1. Introduction

a. Understood from antecedent discussion the missing definitions (i.e. steering, switching, splitting) will be introduced to TR 22.841, largely in alignment with the definitions captured in TS 23.501 --- these terms refer to very different mechanisms and suggest distinct enhancement of 5G System.

b. According to Clause 5.14.2 Pre-conditions:

“Jenny is a subscriber and premium user of TerrA…” and “… Jenny’s virtual reality headset is

registered on TerrA…”.

Based on this, it is clear VR device has already registered to 5G TN.

According to Clause 5.14.3 Service Flows:

“… As the app allows her to virtually walk through and interact with the home, low latency is required for a good end user experience…

…

Performance on the terrestrial network degrade… in order to better meet the low latency requirement of the Virtual Home Visit app, the headset selects the NGSO satellite RAT of SatA to start a dual 3GPP access connection.

”

Based on these descriptions, this use case proposed to use not only the 5G TN (serving network) but additionally use LEO, the combination is intended to achieve shorter latency for VR service.

2. Reason for Change

a. In places such as [PR 5.14.6-001], “splitting, steering or switching” are proposed for 5G System to support, but the

description and Service Flows have not clarified which mechanism(s) is/are intended for the particular problem being

described.

b. As LEO (33ms-45ms) and 5G TN are very different in latency performance, it does not make sense to “split then aggregate” data packets of a delay-sensitive application. As packets arriving from fast link will have to wait for packets arriving from slow link before they can be combined and (re-)ordered, the overall achievable latency for the user is dominated by the slower link (i.e. worse latency).

Another drawback is data packets (of the same data flow) split/sent over 5G terrestrial and 5G satellite access networks will be out-of-order when received.

Instead, the VR application should choose the network with lowest latency.

c. The proposal in Clause 5.14 (this use case) is not thorough, because NTN (GEO or LEO) can’t provide the required latency for VR service ([5 to 10] ms per TS 22.261). TS 22.261 captures Table 7.6.1-1 and Table  7.4.1-1 (below):

d. Based on available information in this use case, only when in a mid-call situation, “switching” from one access network to another one may apply.

3. Conclusions

In principle, no need to consider [PR 5.14.6-001] for consolidation, unless the use case is clarified: e.g. NTN can’t provide the needed latency for VR ([5 to 10] ms), per 22.261 Table 7.6.1-1 and Table 7.4.1-1, -> aggregating LEO (33ms-45ms) into TN NR will increase the overall latency.

As switching might be useful when an ongoing call loses connectivity with the serving LEO access network. Potential requirements for 5GS can be updated accordingly.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.841 v.1.1.0.

* * * First Change * * * *

[PR 5.14.6-001] Subject to operator policy and considering the required QoS, the 5G system shall be able to support mechanisms to allow a home PLMN to provide a UE with policies for the UE to enable switching of traffic (between a 5G terrestrial network and a 5G satellite access network) pertaining to the same data session.

* * * End of Changes * * * *