3GPP TSG-SA WG1 Meeting #103 	S1-232204

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-22xxxx)

Source:		Huawei, Hisilicon

Title:	Discussion paper on satellite-related aspects in FS-DualSteer

Draft Spec:	3GPP TR TR22.841 V1.1.0

Agenda item: 	7.9.1

Document for:     	Discussion

Contact:	         	Shuang ZHANG < zhang.shuang2@huawei.com>

Abstract: This document discusses the 9 use cases and associated potential requirements related to satellite captured in TR 22.841 V1.1.0, and proposes to use the revised potential requirements as the basis for consolidation. Additionally, pCR contributions (S1-232192, S1-232193, S1-232194, S1-232195, S1-232196, S1-232197, S1-232198, S1-232200, S1-232202) provide the text proposal for TR 22.841 V1.1.0.

1. Introduction

There are several different terms used in TR 22.841 (i.e. steering, switching, and splitting), which often appear without distinction (e.g. in description, potential requirements) throughout TR 22.841. As these terms have not been discussed or defined, they need to be clarified to understand whether and how each of them is relevant to the described problem for each use case, before moving to consolidating the potential requirements.

Accordingly, this document discusses the 9 use cases related to satellite and their associated potential requirements captured in TR 22.841, and proposes the way forward.

2. Discussion

The terms to be clarified:

- Steering

- Switching

- Splitting

In TR 22.841 they are not described or defined. But in specifications produced by other 3GPP WGs (e.g. TS 23.501) some definitions related to ATSSS can be found as below.

Access Traffic Steering: The procedure that selects an access network for a new data flow and transfers the traffic of this data flow over the selected access network.

Access Traffic Switching: The procedure that moves all traffic of an ongoing data flow from one access network to another access network in a way that maintains the continuity of the data flow.

Access Traffic Splitting: The procedure that splits the traffic of a data flow across multiple access networks.

The table below lists the 9 satellite-related use cases captured in TR 22.841, and by referring to the above definitions summarizes for each the observation.

| Use cases | Captured Description/Service Flow | Observations |
| --- | --- | --- |
| 5.1 Use case on dual 5G satellite access in maritime scenario | 5.1.1 Description “The ship has a UE that is served by two satellite RANs (GEO and LEO) belonging to the same PLMN that is managed by a 5G satellite operator.” 5.1.2 Pre-conditions “Based on the service agreement between the KASS managing company and the 5G satellite operator, the 5G satellite network has the following policies for the UE on KASS: - The data traffic of delay-sensitive applications is routed via LEO satellite link whenever it is available. - The data traffic of delay-tolerant applications is aggregated via both LEO and GEO satellite links.” 5.1.3 Service Flows 1. … “and the traffic of sensors data monitoring (delay-tolerant) is being aggregated via both LEO and GEO satellite links.” 2. “The LEO satellite access becomes unavailable (e.g. due to loss of line of sight between the UE and a satellite). Therefore, all the traffic that was routed via the LEO satellite access is moved to GEO satellite access, while the continuity of data sessions is maintained. The application for ship remote control (delay-sensitive) detects increased communication latency, so it adapts its operations accordingly.” | A1. Because the LEO and GEO satellite links exhibit very different latency performance (i.e. max 30 ms for LEO v.s. max 280 ms for GEO), it does not make sense to “split then aggregate” data packets of a delay-sensitive application over LEO access network and GEO access network simultaneously. As packets arriving from fast link will have to wait for packets arriving from slow link before they can be combined and (re-)ordered, the overall achievable latency for the user is dominated by the slower link (i.e. worse latency). Another impediment/pitfall is data packets (of the same data flow) split and sent over two links/access networks (e.g. LEO and GEO, TN 5G NR and NTN) will be out-of-order when received. For delay-tolerant application, the benefit of splitting is not clarified either. Splitting traffic of the application simultaneously using GEO and LEO satellite links will end up with worst latency anyways. Therefore, either of the satellite access networks that can satisfy the application needs in latency should be used. A2. As LEO satellite access becomes (regularly) unavailable (Service Flow step 2), delay-tolerant traffic should either be sent over GEO satellite access solely (the application adapts to the increased latency), or over LEO solely and switch to GEO when LEO is unavailable. A3. According to Service Flow step 2, as LEO becomes unavailable, a new communication will be established over GEO satellite access. To align with this, “switching” is the most appropriate term. |
|  | Proposals or way-forward: [PR 5.1.6-001] Subject to operator policies, the 5G System shall support a mechanism to select any suitable 5G satellite access network for transmitting the user plane traffic if two 5G satellite access networks belonging to the HPLMN are available. [PR 5.1.6-002] The 5G system shall support a mechanism to enable switching of UE’s user plane traffic (of the same data session) from one 5G satellite access network of HPLMN to another 5G satellite access network of HPLMN, if UE moves out of the coverage area of the first 5G satellite access network. NOTE: Assuming the HPLMN has two 5G satellite access networks. |  |
| 5.4 Use case on Inter-PLMN scenario - TN and NTN | 5.4.1 Description “This use case describes a ski mountain environment, where 5G NR (terrestrial) coverage is provided by a certain MNO-A (and other MNOs) in limited populated areas (around hotel/resorts and ski areas), together with 5G NTN coverage by MNO SAT.” “MNO-SAT has roaming agreement with MNO-A, including roaming connectivity in remote areas where cellular coverage is not available, plus extra services (on-demand) in joint coverage areas.” “MNO-A offers GOLD employees with a “premium” data connectivity plan, based on a (exclusive) commercial agreement with local MNO-SAT, allowing MNO-A premium users to utilize the NTN network to provide extra capacity in areas with dual coverage…” “In particular, GOLD users can get higher data rate by aggregating their traffic over both cellular (NR) link and an extra NTN connectivity link via PLMN-SAT (with data anchor and aggregation in PLMN-A’ CN). Such functionality assumes support by the UEs (of GOLD users) and both PLMNs’ CNs…” 5.4.3 Service Flows 1. “She opens the VPN app on the UE to start some large file data transfer from/to her VPN server (to update the morning delivery tasks and download the ones for the afternoon). The UE registers to PLMN-SAT and starts a dual 3GPP network connection. VPN data flows over PLMN-A and PLMN-SAT networks, with anchor in PLMN-A’s CN.” | B1. The service “File down /uploading In VPN” is a conventional best-effort service that should be satisfied by 5G NR TN capacity. When needed, within 5G NR TN, solutions are available to increase the network capacity. B2. NTN resources are expected to be scarce, and should be considered for cost-effective uses such as emergency and coverage gaps (e.g. in rural or coastal areas). In this use case, there is already 5G NR TN service available, one should not consider using scarce NTN resources for supplementary capacity gains for best-effort service. B3. TN (5G NR) and NTN have very different performance in throughout, latency, PER, etc. Splitting data traffic of a data flow over the two access networks simultaneously will have performance consequence or implications. Therefore, as TN (5G NR) is already being used, it does not make sense to additionally use NTN w.r.t. splitting. Refer to Observation A1 for more details. B4. Applicable for this use case, Worth noting is the following stage1 requirements captured in TS 22.261, e.g. - Clause 6.5 Efficient use plane: “A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks. ” - Clause 6.7 Priority, QoS, and policy control: “A 5G system with multiple access technologies shall be able to select the combination of access technologies to serve an UE on the basis of the targeted priority, pre-emption, QoS parameters and access technology availability. ” |
|  | Proposals or way-forward: [PR 5.4.6-001] Based on operator policies while satisfying the required QoS, the 5G system shall be able to support a mechanism to enable steering UE’s user plane traffic of one data session onto a suitable PLMN with a specific type of access network. NOTE: Two PLMNs (e.g., one PLMN with NR terrestrial and the other with satellite RATs) belonging to two different PLMN operators (one of which is the HPLMN). The following is assumed: - HPLMN subscription is used to access both NWs, data is anchored in the HPLMN and a proper business agreement among the two PLMN operators is in place, including negotiation of traffic routing policies, e.g. based on application type; - when multiple UE data sessions are established simultaneously (e.g. for different applications), the required mechanisms shall include the ability to steer a data session to a suitable network. |  |
| 5.5 Use case on NTN-based dual 3GPP access | 5.5.3 Service Flows “The UE establish a VoIP, a video or a data service over one 3GPP access link which appears insufficient in QoS (e.g. throughput, latency, etc.). Given that another 3GPP access link is available, it is activated and combined with the first one to support the required QoS of the service. The QoS requirements of the user plane traffic can be determined through specific policies associated to different data flows, or different traffic type within the same data flow.” Based on the QoS requirements (e.g. latency, throughput, Jitter, Error rate), traffic characteristics, radio links conditions and UE's moving speed, the traffic is steered/splitted across the access links. For example low latency requirement traffic will be best splitted/steered to the access link featuring the lowest latency characteristics…” 5.5.4 Post-conditions “Thanks to appropriate steering, splitting and switching of the user plane traffic, the dual NG-RAN access connectivity involving at least NTN can support the targeted QoS that a single access cannot support.” | C1. When the serving NTN becomes insufficient (worsened latency e.g. due to degradation) in mid-call situation, it only makes sense for UE to switch to an alternative network (either another NTN or TN) that provides the required latency by the user. However, it should not resort to splitting data traffic (of the same data flow) over two links with large latency difference, because this choice will end up the user experiencing the worst latency dominated by the slower link (This issue applies for scenarios illustrated in Fig 5.5.1 a/b NTN+ 5G NR, and Fig 5.5.2 / Fig 5.5.3 2xNTN). Refer to Observation A1 for more details. C2. For scenario depicted in Fig 5.5.2 / Fig 5.5.3 (2xNTN), switching between two NTN networks may provide benefit when UE moves out of the coverage of the currently serving NTN. C3. Understood from Service Flows, the main point is selecting (based on the required QoS) the suitable access network for transmitting data of a service. Worth noting is the following stage1 requirements captured in TS 22.261, e.g. - Clause 6.7 Priority, QoS, and policy control: “A 5G system with multiple access technologies shall be able to select the combination of access technologies to serve an UE on the basis of the targeted priority, pre-emption, QoS parameters and access technology availability. ” |
|  | Proposals or way-forward: [PR 5.5.6-001] Based on operator policy while satisfying the required QoS, the 5G system shall be able to support a mechanism to steer UE's data transmission pertaining to the same data session onto a suitable 3GPP 5G access network (when two access networks are available with at least one of them having NR satellite RAT), taking into account available information, e.g. connectivity conditions such as access network performance, congestion and UE's moving speed. This applies for the situation when a UE’s HPLMN has two access networks with at least one being 5G satellite access network, or the situation when two PLMNs (one of them is HPLMN) are concerned and at least one of them as 5G satellite access network. [PR 5.5.6-002] When two 5G access networks are used simultaneously for steering different data flows of the same data service, the 5G system shall be able to collect charging information accordingly. NOTE: In case the two 5G access networks belong to different PLMNs, single subscription and data anchoring in the HPLMN 5G CN are assumed. |  |
| 5.6 Use case on UE using Terrestrial and Satellite Access | 5.6.1 Description “Satellite access known as wide coverage can improve service availability in areas with poor terrestrial access network coverage or radio condition ... For a UE in high- speed move requests real-time services, e.g. IMS voice/video meeting, it can benefit from dual connectivity with 5G system through terrestrial access and satellite access simultaneously, and obtain the continuous and reliable service with the minimum impact of terrestrial access network unavailability.” 5.6.2 Pre-conditions “The terrestrial access network of Operator TerrA has good coverage in urban areas, suburban areas, and stopovers along the railway but limited coverage in rural areas and deserts between the stopovers. The satellite access network (e.g. LEO) of Operator SatA has covered the whole country and SatA can provide the communication service on its own.” | D1. It is a typical OOC case. For an on-going service/application, if the UE moves out of TN coverage (or if TN becomes unavailable), the UE should connect/switch to NTN, e.g. using roaming. Meanwhile, NTN has sufficient coverage and performance to provide the service (according to Pre-conditions,). Therefore, for this use case, there is no need to use TN and NTN at the same time (i.e. no splitting). D2. When outside of TN coverage, the TN access network is unavailable anyways – it can’t be used with NTN simultaneously (i.e. no splitting). D3. [PR 5.6.6-001] appears to capture steering. However, it is not sufficiently explained or rationalized in the use case to support this requirement. |
|  | Proposals or way-forward: [PR 5.6.6-001] Based on operator policy and agreement, the 5G system shall support a mechanism to steer UE’s data flow(s) of the same data session (i.e. the same service) across, or switch the user traffic of the same data session between different 3GPP access networks (i.e. 5G terrestrial and 5G satellite access networks) for UE with dual 3GPP access capability, considering service preference, traffic characteristics, radio characteristics, QoS, etc. [PR 5.6.6-002] Based on operator policy and agreement, the 5G system shall support service continuity with minimum service interruption when steering UE’s data flow(s) of the same data session (i.e. the same service) across, or when switching the user traffic between two 3GPP access networks (i.e. 5G terrestrial and 5G satellite access network) for UE with dual 3GPP access capability in use, based on network availability, service preference, etc. |  |
| 5.9 Use case on dual steering through Satellite and UAV | 5.9.2 Pre-conditions “There is satellite connectivity in the area but due to non-line of sight conditions, the connection may not be available to a subset of users… …There is coverage overlap between both the satellite access network and UAV (mounted with gNodeB/relay node).” 5.9.3 Service Flows “For example, traffic with low latency requirements should be split, steered towards the access link with lowest latency characteristics… …She may lose connectivity in between with the satellite network for a little while she is inside the site, then all the traffic is handled through the UAV. When she goes outside, satellite based connectivity is restored… … Hence, if congestion on one of the available access links increases or in the scenario of a temporary link failure, then user plane traffic is switched to the other active access link. Once both of the links are active, then depending on the QoS requirements, user plane traffic is again split and steered between the two 3GPP networks.” | E1. Scenario: A PLMN has TN 5G NR access (via gNB/IAB on UAV) and 5G satellite access network. The UE can be out of satellite coverage. E2. According to Service Flows, either TN (5G NR) or NTN can satisfy the service. Therefore, for this service using TN and NTN simultaneously is not needed (i.e. no splitting). So, normally one of the access networks (e.g. TN 5G NR) should be used when available. When connectivity is lost to the currently serving TN, new communication via satellite access should be established (e.g. switching user plane traffic). E3. Currently, in clause 5.9 evidence is missing that should explain the benefit of splitting or steering user data of the same service over TN and NTN. But, the real critical issue is: splitting data traffic of a data flow simultaneously over the two access networks with very different performance (5G NR TN vs. NTN) will have consequence and implications. Refer to Observation A1 for more details. |
|  | Proposals or way-forward: [PR 5.9.6-001] Based on operator policy, the 5G system shall be able to support a mechanism to switch data transmission to 5G satellite access network for the same data session, if the 5G NR terrestrial access network becomes unavailable while considering QoS requirements between these two 3GPP networks. This applies for HPLMN having a 5G NR terrestrial access network and a 5G satellite access network. [PR 5.9.6-002] The 5G system shall be able to collect charging information considering switching data transmission pertaining to the same user data session across two 3GPP access networks of the same HPLMN. |  |
| 5.10 Use case on NTN and TN Inter-PLMN Multi-access in a Maritime scenario | 5.10.1 Description 1. Satellite operators: MNO-G (GEO), MNO-L (LEO), and TN operator MNO-T (covers the island country and 20km from the coast). 2. “Thanks to the large coverage area of the GEO satellite of MNO-G, the UE can be assumed to be always under the GEO satellite access in the vessel navigation route… … The LEO satellite constellation of MNO-L will be deployed globally, but the coverage will be discontinuous…” …Since remote control of short-term route planning requires real-time communication and high-traffic data communication, it is only enabled when the vessel is connected to the LEO satellite access or the terrestrial 5G access… …Table 5.10-1 shows different possible applications ” 5.10.2 Pre-conditions “MNO-G has business and roaming agreements with MNO-L and MNO- T… …Company X's vessel is a subscriber of MNO-G, which is its HPLMN…” 5.10.3 Service Flows 1. “The vessel's route is basically under the GEO satellite coverage, and when the vessel is connected only to the GEO satellite, real-time data communication is not enabled.” 2. “When the vessel moves into an area where both the GEO and the LEO satellite coverage is present… … In addition to long-term route planning, Alice also remotely commands short-term route changes based on Real-time Vessel Control Data and Vessel Monitoring Data via the LEO satellite. …” 3. “When the vessel approaches the coast and moves into terrestrial 5G access coverage, the VSAT-type UE is connected to terrestrial 5G access even if it is within the LEO satellite coverage (GEO and terrestrial 5G access). The vessel’s monitoring and remote control are performed in the same way as in step 2” | F1. According to the Description, traffic of one application is steered to one access and traffic of another application is steered to another access (Table 5.10-1). F2. Based on Service Flow 1: delay-tolerant service (long-term route planning) uses GEO. It is clear the user does not consider delay-sensitive application (short-term route planning) if only GEO is available. F3. Based on Service Flow 2: only when UE is in the coverage of either LEO or TN (5G NR), will the user use delay-sensitive application (short-term route planning). F4. Based on Service Flow 3: when both LEO and TN (5G NR) are available, short-term route planning application prioritizes to use TN alone (i.e. Switching communication from LEO to TN). F5. No splitting is concerned: neither described nor proposed. F6. Whilst proposing switching, [PR 5.10.6-002] pertains to simultaneously transmitting user data onto two different access networks (one of them belongs to HPLMN, the other belong to a VPLMN). Therefore, due to contradiction to the definition/concept of “switching” in TS 23.501, [PR 5.10.6-002] is incomprehensible and should be removed. F7. Note TS 22.261 has already captured the following requirements: -Clause 6.3: Multiple access technologies When a UE is using two or more access technologies simultaneously, the 5G system shall be able to optimally distribute user traffic over select between access technologies in use, taking into account e.g., service, traffic characteristics, radio characteristics, and UE‘s moving speed. -Clause 6.5: Efficient user plane A 5G system with satellite access shall be able to select the communication link providing the UE with the connectivity that most closely fulfils the agreed QoS A 5G system with satellite access shall be capable of supporting simultaneous use of 5G satellite access network and 5G terrestrial access networks. -Clause 6.7.2 Priority, QoS, and policy control The 5G system shall be able to provide the required QoS (e.g. reliability, end-to-end latency, and bandwidth) for a service and support prioritization of resources when necessary for that service. |
|  | Proposals or way-forward: [PR 5.10.6-001] Based on operator policy while meeting the required QoS, the 5G system shall be able to support mechanisms to enable steering UE’s (with single subscription to HPLMN) different applications’ traffic onto suitable access networks (of two different PLMNs, one of which is the HPLMN), dual-radio capable UE either using two 5G satellite access networks, or using a 5G satellite access netowrk and a 5G terrestrial network, and traffic anchoring in the HPLMN’s 5G core network. Remove [PR 5.10.6-002] considering Observation F6. |  |
| 5.14 Use Case on Inter-PLMN scenario - TN and multiple NTN | 5.14.2 Pre-conditions “Jenny is a subscriber and premium user of TerrA… … Jenny’s virtual reality headset is registered on TerrA… … Jenny’s virtual reality headset can use dual- PLMN/RAT connectivity, including two options for connecting to SatA – through satellite access (GSO and/or NGSO).” 5.14.3 Service Flows 1. “…She puts on her virtual reality headset and starts the Virtual Home Visit app. As the app allows her to virtually walk through and interact with the home, low latency is required for a good end user experience.” 2. “Performance on the terrestrial network degrade… in order to better meet the low latency requirement of the Virtual Home Visit app, the headset selects the NGSO satellite RAT of SatA to start a dual 3GPP access connection” | G1. Sending data traffic of the VR application simultaneously over LEO (33ms-45ms) and over 5G TN will cause the overall latency to be determined by the worst latency among the two. Instead, the VR application should choose the network with lowest latency. Refer to Observation A1 for more details. G2. Only when in a mid-call situation, “switching” from one access network to another one may apply. G3. The proposal in Clause 5.14 (this use case) is not thorough, because NTN (GEO or LEO) can’t provide the required latency for VR service ([5 to 10] ms per TS 22.261). TS 22.261 captures Table 7.6.1-1 and Table 7.4.1-1 (below): |
|  | Proposals or way-forward: No need to consider this PR for consolidation, unless the use case is clarified: e.g. NTN can’t provide the needed latency for VR ([5 to 10] ms), per 22.261 Table 7.6.1-1 and Table 7.4.1-1, -> aggregating LEO (33ms-45ms) into TN NR will increase the overall latency. |  |
| 5.17 Use Case on Vehicle IoT devices steering via NTN and TN | 5.17.1 Description “For large area agriculture fields and farms where two 3GPP radio access network services are available IoT devices can benefit from dual steering data connection capability of 5G System via NTN and TN access networks simultaneously.” “In this use case we have a wide area agricultural automated system of applications controlling agriculture vehicles” 5.17.2 Pre-conditions “…The farm has overlapping TN and NTN radio access coverage… …Both access network providers have agreed, multi-path data traffic routing policies allowing User Data applications to use access networks based on IoT device traffic characteristics.” - Service Flows 1. “Alice has to complete various agricultural tasks on the field today.” | H1. This is a TN+NTN use case. Based on wording such as “applications” and “various agricultural tasks” in clause 5.17.2 and clause 5.17.3, the main point is to select the suitable access networks involved in transmission, and traffic of one application is steered to one access and traffic of another application is steered to another access. H2. In general, Clause 5.17 has not clarified whether or how this use case would benefit from any one of steer, switch or split. It is unclear what “simultaneously” means in Clause 5.17.1. |
|  | Proposals or way-forward: [PR 5.17.6-001] Based operator policies while satisfying the required QoS, the 5G system shall be able to support mechanisms to allow steering IoT devices’ data traffic pertaining to different applications, onto one of the two access networks i.e. 5G satellite access network, and 5G TN. [PR 5.17.6-002] Based on data usage of both access networks (i.e. 5G satellite access network, and 5G TN) for sending traffic of different applications of an IoT device, the 5G system shall be able to collect charging information for the IoT device. |  |
| 5.18 Use Case on UAV UE connecting to 3GPP TN and NTN access networks | 5.18.2 Pre-conditions “…Agreed multi-path data traffic routing policy allows to route different traffic over one or different 3GPP access connections, for example FTP file transfers (more delay tolerant) can use one data path while HTTP Video Streaming (requiring higher Tput) can use both concurrent data connection paths.” 5.18.3 Service Flows (step2) “… FTP maps data is transferred via the NTN access network. When the UAV reaches a certain identified location, the UE application begins video streaming / photo capture and start uploading data to the server, using both TN and NTN network connections…” (step4) “During this active data session, if/when the UAV encounters degradation on one of the access network paths, e.g. based on configured traffic measurement conditions, UE and 5GC will be able to steer data traffic (both FTP and Video Streaming) over the stronger data access path.” | G1. Different services (e.g. video streaming, FTP) are described to be sent over TN or NTN depending on the required QoS. Therefore, the main goal is to select the suitable access networks, so that 5G network can provide the suitable QoS for respective applications on the UE. G2. It proposes to split packets of “HTTP Video Streaming” service onto 5G TN and5G satellite access network, however, it is worth noting that the packet arrival order for streaming service is very important. Because NR TN and NTN are very different in throughput, PER, latency, splitting will result in out-of- order packets, refer to Observation A1 for more details. |
|  | Proposals or way-forward: [PR 5.18.6-001] Based on PLMN operator policies, the 5G system shall be able to support mechanisms to configure and control regarding transmission of UE data (of different services, e.g. video streaming, FTP) to two 3GPP access networks belonging to two different PLMNs, one of which is the UE’s 5GC HPLMN (e.g. for a UE using single subscription, connected via NR based TN and/or NTN). This can also include support for UE specific user data characteristics measurements (e.g., RTT, Packet loss rate) reporting to UE’s HPLMN. |  |

3. Proposals

General proposal: the identified open issues (e.g. definitions for steering, switching, and splitting) need to be resolved before consolidation of potential requirements. Based on the definitions identify on a per-use case basis which (of steering, switching, and splitting) is/are relevant and thus could be considered as input for consolidation.

Proposals: improvement of potential requirements of the 9 satellite-related use cases in TR 22.841 v.1.1.0 is introduced as the basis for consolidation. The proposed improvement is to align with the observations aforementioned with revision proposals in the main table above.

The corresponding pCRs are provided in S1-232192, S1-232193, S1-232194, S1-232195, S1-232196, S1-232197, S1-232198, S1-232200, and S1-232202.