3GPP TSG SA WG 1 Meeting #103 	S1-232216

Gothenburg, Sweden, 21 - 25 August 2023	(revision of S1-23xxxx)

Source:	Ericsson, Nokia, Deutsche Telekom AG

pCR Title:	Pseudo-CR on sensing operation chapter for overview

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:	Daniel Lönnblad Daniel.lonnblad at Ericsson.com

Abstract: This is a proposal for a chapter in overview called sensing operation. High level description of sensing operation and describing some of the definitions in this document.

1. Introduction

To introduce a reader of this TS in general overview of the sensing operation, to allow for better understanding when reading the document.

2. Reason for Change

To get a better understanding of sensing operation and some of the terms used in this document

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.137

* * * First Change * * * *

# 4	Overview

## 4.x Sensing operation

5G Wireless sensing is a technology enabler to acquire information about characteristics of the environment and/or objects within the environment, that uses radio waves to determine the distance (range), angle, or instantaneous linear velocity of objects, etc. The operation of the 5G wireless sensing service, a.k.a. sensing operation, relies on analyzing the transmissions, reflections, and scattering of wireless sensing signals.

5G wireless sensing therefore has the opportunity to enhance the 5G system from being a communication network to also a wireless sensing network where it uses 5G entities to sense objects and environment in its surrounding. This could be done either with analysing signals that are intended for communication or specific signals dedicated for sensing.

Sensing operation can be implemented in a couple of different ways, from radar like sensing where the sensing transmitter and sensing receiver are co-located in the same entity (figure 4.2-1), also called Monostatic sensing, to have the sensing receiver and sensing transmitter in different entities (figure 4.2-2), also called Bistatic sensing. A more advanced scenario with multiple sensing transmitters and receivers is also possible, called Multistatic sensing. The reflexions of the sensing signal sent from the sensing transmitter are received by the sensing receiver and calculation can be made to get a position of the sensed object. By measuring the Doppler shift of the received signal also a velocity of the sensed object can be estimated. Below is a schematic picture showing the different parts of the sensing operation and the nomenclature used in this specification.

Non-3GPP based sensing is when other means to determine objects and environment are used. This could be for instance radar camera or Wi-Fi sensing. This type of sensing is not considered in this specification but non-3GPP sensing data from this type of sensing can be used to achieve improved result or in any other way enhance the sensing service.

The sensing result can be used within the 3GPP system either for internal purposes or as a service, the sensing result can also be shared with trusted third parties. This TS describes the services either 3GPP ones or trusted third parties and not the internal 3GPP usage.

It is expected that sensing service will work independently of positioning service.

Figure 4.2-1: Illustration of sensing with co-located sensing receiver and sensing transmitter

Figure 4.2-2: Illustration of sensing with separated sensing receiver and sensing transmitter

* * * Next Change * * * *

<Proposed change in revision marks>

* * * Next Change * * * *

<Proposed change in revision marks>