3GPP TSG SA WG1 Meeting #103	S1-232220

# Goteborg, Sweden, 21-25 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.837 | CR | 0012 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on consolidated requirements on configuration and authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Sensing |  |  |  |  |  | Date: |  |  | 2023-06-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are still additional 3 consolidated requirements regarding configuration and authorization. It is proposed to utilize the following requirements as consolidated requirements. [CPR 7.1.2-2] Subject to regulation and operator policies, the 5G network shall be able to configure and/or authorize sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service. NOTE: Such configuration and authorization can be based on e.g. sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data.; Vivo’s comments: Firstly, change “network” to “system”, which is more general and not mentioning solutions. Secondly, add “discover” as an action for configuration and authorization, as e.g. BS also needs to discover neaby BSs especially based on transmitter/receiver’s capabilities. But optionally, as not all sensing modes needs the procedure of discover.. [CPR 7.1.2-3] Based on operator policies and based on location, the 5G network shall be able to configure and/or authorize sensing transmitters and sensing receivers for using licensed spectrum under network coverage for 5G wireless sensing service.; Vivo’s comments: regarding licensed spectrum, it is 5G network to configure and authorize.. [CPR 7.1.2-4] 5G network shall provide means for mobile operator to provide/revoke authorization for the operation(s) of a 5G wireless sensing service based on location, time, specific KPI level and the origin of the request.; Vivo’s comments: CPR-7.1.1-2 is general enough, and suggests to remove this reuirement.. |
| Summary of change: |  | Add following CPRs in section 7.1 Subject to regulation and operator policies, the 5G system shall be able to discover (optionally) configure and/or authorize sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service. NOTE: Such configuration and authorization can be based on e.g. sensing transmitter or sensing receiver capabilities and location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data. Based on operator policies and based on location, the 5G network shall be able to configure and/or authorize sensing transmitters and sensing receivers for using licensed spectrum under network coverage for 5G wireless sensing service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 7.1	Consolidated functional requirements

Table a – Sensing configuration and authorization Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall be able to provide mechanisms for an MNO to configure UEs supporting V2X application for 5G Wireless sensing service when not served by RAN. | PR 5.30.6-1 |  |
|  | Subject to regulation and operator policies, the 5G system shall be able to discover (optionally) configure and/or authorize sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service. NOTE: Such configuration and authorization can be based on e.g. sensing transmitter or sensing receiver capabilities and location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data. | PR 5.2.6-3 PR 5.15.6-1 PR 5.9.6-2 PR 5.20.6-2 PR 5.8.6-1 PR 5.8.6-4 PR 5.28.6-1 PR 5.1.6-1 PR 5.5.6-3 PR 5.11.6-4 P.R 5.13.6-8 P.R 5.10.6-2 PR 5.17.6-3 |  |
|  | Based on operator policies and based on location, the 5G network shall be able to configure and/or authorize sensing transmitters and sensing receivers for using licensed spectrum under network coverage for 5G wireless sensing service. | PR 5.20.6-1 PR 5.25.6-1 |  |

Table b – 5G Wireless sensing service Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall be able to provide 5G wireless sensing service in a sensing service area location using sensing transmitters and sensing receivers. | PR 5.2.6-1 PR 5.9.6-1 PR 5.11.6-1 P.R 5.14.6-1 PR 5.1.6-2 |  |
|  | Subject to regulation and operator policy, the 5G network shall be able to activate, configure, and deactivate 5G wireless sensing based on parameters such as location and network conditions (e.g. network load). | P.R 5.14.6-3 P.R 5.13.6-7 PR 5.8.6-6 |  |
|  | Subject to user consent, regulation, and operator’s policy, the 5G system shall be able to collect non-3GPP sensing data from authorized non-3GPP sensors and securely provide it to 5G network for processing. | PR.5.4.6-1 PR 5.21.6-2 |  |
|  | The 5G system shall support continuity for 5G wireless sensing service (e.g. for sensing a moving object). | PR 5.23.6 -1 PR 5.18.6.1 |  |
|  | Subject to operator’s policy, the 5G System shall be able to provide the 5G wireless sensing service in case of roaming. | PR 5.24.6-1 |  |
|  | Subject to user consent, regulation, and operator’s policy, the 5G system should support the combination of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result. | PR 5.21.6-3 PR 5.4.6-4 PR 5.27.6-2 |  |
|  | Subject to regulation and operator’s policy, 5G network shall provide prioritization among 5G wireless sensing services (e.g. prioritizing between communication and sensing services). | PR 6.2.2-1 |  |
|  | The 5G system shall be able to enable UEs without 5G coverage to use unlicensed spectrum to provide 5G wireless sensing service. | PR 5.25.6-3 PR 5.1.6-5 PR 5.25.6-2 |  |
|  | Subject to regulation, the 5G system shall enable UEs supporting V2X application to perform 5G Wireless sensing when not served by RAN using the allowed ITS spectrum and unlicensed spectrum. | PR 5.30.6-2 |  |

Table c – Network exposure Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met. NOTE: These conditions could be e.g. the target object distance from the restricted area border or entering restricted area. | PR 5.13.6-6 |  |
|  | Subject to operator’s policy, the 5G network shall be able to provide secure means to enable trusted third-party to request discovering a sensing group in the proximity of the UE that is requesting a service from application server. | PR 5.19.6-2 |  |
|  | The 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g. refresh rate, period of time, sensing KPIs, geographical location) and to receive the respective corresponding sensing results. | PR 5.11.6-3 PR 5.13.6-4 PR 5.12.6-3 PR 5.12.6-5 PR 5.12.6-4 PR 5.5.6-1 PR 5.5.6-2 |  |
|  | Subject to operator’s policy, the 5G system shall be able to provide secure means for a trusted third-party application to receive sensing results with contextual information. | PR 5.8.6-5 |  |

Table d – Security Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall provide a mechanism to protect identifiable information that can be derived from the 3GPP sensing data from eavesdropping. | PR 5.16.6-1 |  |
|  | The 5G system shall limit the exposure of the sensing results only to third party authorized to receive that sensing results. | PR 6.1.2-1 |  |
|  | The 5G system shall support encryption, integrity protection, privacy of the 3GPP sensing data, non-3GPP sensing data and sensing results, to protect the data inside the 5G system. | PR 5.27.6-4 PR 5.27.6-3 PR 5.23.6 -2 PR 6.1.2-2 |  |
|  | The 5G system shall support appropriate sensing KPIs of 5G wireless sensing for both situations where consent can be obtained from the sensing targets, and where it cannot. | PR 6.1.2-3 |  |

Table e – Charging Consolidated Requirements

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
|  | The 5G system shall be able to support charging for the 5G wireless sensing service (e.g. considering sensing KPIs, duration). | PR 5.20.6-4 |  |

* * * End of Change * * * *